A6-EC

Servo Drive

User Guide

Data code: C2312010XXX Version:  A00

Power range: 0.1 kW to 2.9 kW

Legal Information Statement:

z The product described in this documentation may be operated only by personnel qualified for the specific task in accordance with the relevant documentation.

z All operations on the product must follow respective descriptions provided in the documentation, in particular, its warning notices and safety instructions.

z    Damage caused by improper use is not covered by warranty.

z    The company will disclaim any legal liability for any personal injury or property damage caused by

improper usage.

Contents

Chapter 1 Product Information	09

  - Features	09

  - Model	10

  - Nameplate	10

  - Components	11

  - Rated Data	12

  - Technical Specifications	13

  - Communication Specifications	16

Chapter 2 Mechanical Installation	17

Installation Environment	17

  - Installation clearance	18

  - Installation Orientation	20

  - Installation Dimensions	21

  - Installation Guide	23

Chapter 3 Electrical Installation	24

System Topology	24

  - System Wiring	28

3.3    Ports	30

3.4     Power Supply Connection	35

3.4.1     Main circuit wiring	36

    - Cable specifications and recommendations	38

    - Grounding	39

  - Motor Connection	43

  - Encoder Connection (CN2)	44

  - Control Signal Connection (CN1)	46

    - I/O signal	46

    - Digital input/output (DI/DO) signals	47

    - Wiring of the brake	50

  - Communication Signal Connection (CN3 and CN4)	52

  - Communication Terminal Connection (CN6)	54

  - Braking Resistor Connection	54

Chapter 4 Function Overview	56

Basic Functions of the Servo Drive	56

    - Function Overview	56

    - Profile Position (PP) Mode	56

    - Profile Velocity (PV) Mode	62

    - Profile Torque (PT) Mode	65

    - Homing Mode (HM)	69

    - Cyclic Synchronous Position (CSP) Mode	106

    - Cyclic Synchronous Velocity (CSV) Mode	109

    - Cyclic Synchronous Torque (CST) Mode	112

    - Touch Probe Function	115

Chapter 5 Absolute System	120

Software Position Limit	118

  - Absolute System Setting	120

  - Absolute Position Linear Mode	121

  - Absolute Position Rotation Mode	122

  - Absolute Position Single-turn Mode	125

  - Precautions for Use of the Absolute system Battery Box	125

Chapter 6 System Commissioning	127

Commissioning Tool	127

    - Buttons	127

    - Display	128

  - Commissioning Process	131

  - Commissioning Procedure	132

    - Power on the servo drive	132

    - Jogging	132

    - Set parameters	133

    - Servo running	137

Chapter 7 Gain Tuning	140

6.3.5     Stop the servo drive	138

02

  - Overview	140

  - Inertia Auto-tuning	140

7.3     Basic Gain Tuning	142

7.4     Pseudo derivative feedback and feedforward control	145

7.5    Gain Switchover	146

  - Speed Feedforward	148

  - Torque Feedforward	148

  - Position Reference Filter	149

  - Model Tracking Control	149

  - Speed Feedback Filter	150

  - Speed Observer	151

  - Disturbance Observer	151

  - Friction Compensation	152

  - Vibration Suppression	153

Chapter 8 Communication Description	156

Overview	156

    - EtherCAT Overview	156

    - EtherCAT communication technical specifications	158

    - EtherCAT communication specification	159

  - Communication Transmission Modes	159

    - EtherCAT Communication Structure	159

    - Communication State Machine	160

8.2.3     DC	162

  - Communication Data Frame Structure	163

    - Process Data	163

    - Mailbox Data	168

Chapter 9 DI/DO Function	169

Chapter 10 Troubleshooting	171

  - Fault Alarms	171

    - Fault display and category	171

    - Troubleshooting and reset	172

    - List of faults and alarms	173

03

|  | 10.2 Solutions | 179 |
| --- | --- | --- |
|  | Chapter 11 Parameter List | 200 |
|  | 11.1 Parameter Group Description | 200 |
|  | 11.2 Parameter List | 200 |
|  | 11.2.1 Common Parameters in Group 2000h | 200 |
|  | 11.2.2 Common Parameters in Group 6000h | 226 |
|  | 11.3 Description of Parameters | 231 |
|  | 11.3.1 Group C00 | 231 |
|  | 11.3.2 Group C01 | 232 |
|  | 11.3.3 Group C03 | 238 |
|  | 11.3.4 Group C05 | 239 |
|  | 11.3.5 Group C06 | 240 |
|  | 11.3.6 Group C0A | 240 |
|  | 11.3.7 Group C13 | 240 |
|  | 11.3.8 Group R21 | 241 |
|  | 11.3.9 Group F30 | 242 |
|  | 11.3.10 Group F31 | 242 |
|  | 11.3.11 Group U40 | 243 |
|  | 11.3.12 Group 6000 | 245 |
|  | Chapter 12 Motor and Options | 254 |
|  | 12.1 Model | 254 |
|  | 12.2 Nameplate | 255 |
|  | 12.3 Components | 255 |
|  | 12.4 Terminal Definition | 255 |
|  | 12.5 General Specifications | 257 |
|  | 12.5.1 Mechanical Characteristics | 257 |
|  | 12.5.2 Overload Characteristics | 258 |
|  | 12.5.3 Load moment of inertia | 259 |
|  | 12.6 Selection Precautions | 260 |
|  | 12.7 Technical Specifications | 261 |
|  | 12.7.1 Model of 3000 rpm | 261 |
|  | 12.8 Motor torque-speed characteristics | 265 |
|  | Chapter 13 Peripheries | 266 |
| 04 |  |  |

13.1   List of Peripheries	266

13.2  Fuse	266

  - Electromagnetic Contactor	267

  - Circuit Breaker	268

  - Absolute Encoder Battery	269

Chapter 14 Maintenance	270

14.1  Daily Maintenance	270

  - Periodic Maintenance	271

  - Part Replacement	271

Chapter 15 Troubleshooting for Common EMC Issues	274

14.3.1   Plain Key Replacement	271

15.1  RCD Malfunction	274

15.2  Harmonic Suppression	275

  - Control Circuit Interference	275

    - Common I/O Signal Interference	275

Chapter 16 Certification and Standard	277

EtherCAT Communication Interference	276

16.1  CE Certification	277

05

Safety Information and Precautions

To avoid personal injury or damage to the equipment, matters to be followed are stated as follows:

y   Read and follow the Safety Information and Precautions before use.

y   Use this product according to the designated environment requirements.

y   Follow all safety information and precautions described in the product identification and manual.

The degree of injury and damage caused by improper use of this product is distinguished and described as follows:

| DANGER | This mark indicates that failure to comply with the notice will result in severe personal injury or even death. |
| --- | --- |
| WARNING | This mark indicates that failure to comply with the notice may result in severe personal injury or even death. |
| CAUTION | This mark indicates that failure to comply with the notice may result in minor or moderate personal injury or damage to the equipment. |
| NOTICE | If rated precautions are not taken, it may cause undesirable result or state. |

Matters to be followed are described using the following graphic marks:

| This graphic mark indicates contents that must be performed. | This graphic mark indicates contents that must not be performed. |
| --- | --- |

DANGER

y   Install this product on non-combustible materials such as metal.

y   Set up the product in a clean place where it does not contact water or oil.

y   Installation and wiring must be performed by qualified electricians.

y Installation personnel must be familiar with product installation requirements and relevant technical materials.

y The moving, installation, wiring, and inspection of this product can be performed only after you cut off the power supply, wait at least 10 minutes, and determine that there is no risk of electric shock.

y Follow the proper electrostatic discharge (ESD) procedures and wear an anti-static wrist strap to perform wiring.

y The cables should be properly connected. The energized part must be properly insulated using an insulator.

▏ Safety Information and Precautions

y   Do not place any combustible material around this product.

y   Do not place this product around heating elements such as heaters and large wire-wound

resistors.

y Do not use this product in a corrosive and inflammable gas environment or in a place close to combustible materials.

y   Do not use this product in a place with strong vibration or impact.

y   Do not use this product after the cables are immersed in oil or water.

y   Do not perform wiring at power-on.

y Do not damage the cables or apply any excessive external force, weight, or pinch to them.

y   Do not connect this product directly to the commercial power supply.

y   Do not perform installation and wiring in a place with strong electric or magnetic field.

y   Do not perform wiring and equipment operations with wet hands.

y   Do not reach your hands into this product.

WARNING

y Specialized loading and unloading equipment must be used to handle the product. y When handling the equipment with bare hands, hold the equipment casing firmly with care to prevent parts from falling. y Handle the equipment with care during transportation and mind your steps. y When this product is installed in a terminal device, the terminal device must be equipped with protection. The protection class must comply with relevant IEC standards and local regulations. y Cables used for wiring must meet cross sectional area and shielding requirements. The shield of the cable must be reliably grounded at one end.

y Do not install the equipment if you find damage, rust, or signs of use on the equipment or accessories upon unpacking. y Do not install the equipment if you find water seepage or missing or damaged components upon unpacking. y Do not install the equipment if you find the packing list does not conform to the equipment you received. y When the product is lifted by a crane, personnel cannot stand or stay under the product. y Do not modify this product. y Do not fiddle with the bolts used to fix equipment components or the bolts marked in red. y Do not connect the input power supply to the output end of the equipment.

Safety Information and Precautions ▕

CAUTION

y Check whether the equipment or accessories show the evidence of damage, rust, impact, or dampness. y Check whether the package contents are consistent with the packing list. y After wiring is completed, ensure that there are no screws fallen or cables exposed in the equipment. y Make sure that the temperature around the equipment is within the range of temperature and humidity. y Dispose of the equipment as industrial waste during discarding.

y Do not stand on the equipment or place a weight on it. y Do not let the equipment fall or invert it during the handling or setup. y Do not place any barriers around the product and peripheral equipment to hinder ventilation. y Do not let the equipment suffer from any strong impact.

Safety Signs

Danger

y Conduct protective grounding to prevent electric shock. Read through the guide and follow the safety instructions before use.

High Voltage

y Do not touch terminals with power-on or within 10 minutes after disconnecting the power supply to prevent the risk of electric shock.

Hot

y Do not touch the drive during operation and within a short time after shutdown. Failure to comply may cause burns.

Environmental Protection

Reuse

y Some components of the product can be reused due to high metal content. Dismantle the product into individual components to improve the metal recycling efficiency. Electrical and electronic components contain metal materials that can also be recycled through a specific separation process.

Disposal

y Discard components that cannot be degraded and recycled as industrial wastes according to local regulations.

Chapter 1	Product Information

Efficient, Reliable, and Reassuring

The A6-EC series is a fast, reliable, and accurate positioning platform servo drive from STEPPERONLINE covering the most common power range of 0.1 kW to 2.9 kW for general automation. It supports both single-phase and three-phase power supply systems. It features a space-saving design and excellent servo control performance.

Working together with the A6M servo drive, the A6-EC series servo drive can optimize the performance and the ease of use. Compatible with mainstream PLCs from brands such as Beckhoff and Omron, the A6-EC series servo

drive provides efficient and convenient motion control solutions for various

industries.

1.1   Features

Excellent performance y High response: 2 kHz speed loop bandwidth y High accuracy: 17-bit absolute encoder y High speed: 250 μs synchronization period y High synchronization: 300 nodes at 120 m distance, 15 ns synchronization error, ±20 ns synchronization jitter

Sophisticated and novel design y Integrated communication ports, enhancing wiring efficiency y Compact design, making the product be able to be installed in narrow space y Easy to connect and use, with serial commissioning cable to improve commissioning efficiency y Support to EtherCAT bus servo parameter copy for quick and easy access

Safe and reliable y Built-in dynamic brake function y Optional high-protection model to cope with harsh application environments

1.2   Model

200

400

750

800

1000

...

200W

400W

750W

850W

1000W

...

A6

series servo drive

A6 - 400EC

| EC | EtherCAT bus |
| --- | --- |

1.3   Nameplate

Nameplate

Model

Rated  input Rated output Manufacturing SN

MODEL:  A6-400EC

INPUT: 1PH/3PH AC200-240V 4.0A 50/60Hz OUTPUT:3PH AC0-240V 2.8A 0-500Hz 400W S/N: CA000447224000001

www.omc-stepperonline.com

1.4   Components

Figure 1-1 Components of the A6-400EC servo drive (SIZE A)

| No. | Name | No. | Name |
| --- | --- | --- | --- |
| ① | Commissioning and communication port (CN6) | ⑦ | QR code on machine |
| ② | Display and operation area | ⑧ | EtherCAT communication network port (CN3 and CN4) |
| ③ | Control signal port (CN1) | ⑨ | Encoder signal port (CN2) |
| ④ | Power input | ⑩ | Motor power output |
| ⑤ | Braking resistor port | ⑪ | System ground |
| ⑥ | Charging indicator | - | - |

NOTICE

y The above figure describes the component layout of the SIZE A drive. Component layout of other models may be different. For positions of ports of other models, see section 3.3 Ports.

1.5    Rated Data

Single-phase 220 V servo drive

| Item | SIZE-A |  | SIZE-B |
| --- | --- | --- | --- |
| Power | 0.2kW | 0.4 kW | 0.75 kW |
| Drive model A6-EC | A6-200EC | A6-400EC | A6-750EC |
| Continuous output current (Arms) | 1.6 | 2.8 | 5.5 |
| Maximum output current (Arms) | 5.8 | 10.1 | 16.9 |
| Main circuit power supply | Single-phase 200–240 V AC, –10% to +10%, 50/60 Hz |  |  |
| Control circuit power supply | Powered up by the bus, sharing one power supply and rectification part with the main circuit |  |  |
| Braking capability | External braking resistor |  | Built-in braking resistor |

Single-phase/Three-phase 220 V servo drive

| Item | SIZE-C | SIZE-D |
| --- | --- | --- |
| Power | 1.0 kW | 1.5 kW |
| Drive model A6-EC | A6-1000EC | A6-1500EC |
| Continuous output current (Arms) | 7.6 | 11.6 |
| Maximum output current (Arms) | 23 | 32 |
| Main circuit power supply | Single-phase/three-phase 200–240 V AC, –10% to +10%, 50/60 Hz |  |
| Control circuit power supply | Single-phase 200–240 V AC, –10% to +10%, 50/60 Hz |  |
| Braking capability | Built-in braking resistor |  |

NOTICE

y The main circuits of A6-1000EC and A6-1500EC drives can be connected to a single-phase or a three-phase power supply, depending on which one is available on site.

Three-phase 380 V servo drive

| Item | SIZE-D |  |
| --- | --- | --- |
| Power | 2.0 kW | 3.0 kW |
| Drive model A6-EC | A6-2000EC | A6-3000EC |
| Continuous output current (Arms) | 8.4 | 11.9 |
| Maximum output current (Arms) | 20 | 29.8 |
| Main circuit power supply | Three-phase 380–440 V AC, –10% to +10%, 50/60 Hz |  |
| Control circuit power supply | Single-phase 380–440 V AC, –10% to +10%, 50/60 Hz |  |
| Braking capability | Built-in braking resistor |  |

1.6    Technical Specifications

Basic specifications

| Item | Specifications |
| --- | --- |
| Control mode | IGBT PWM control, sine wave current drive mode 220 V, 380 V: Single-phase or three-phase full-bridge rectification |
| Encoder feedback | 17-bit multi-turn absolute encoder, which can be used as a single-turn absolute encoder in absence of the battery |
| Operating temperature | 0°C to +55°C (over 45°C: derate 10% for every additional 5°C) |
| Storage temperature | –40°C to +70°C |
| Altitude | Up to 2000 m. For altitude above 1000 m, derate 1% for every additional 100 m |
| IP rating | IP20 (except for the power terminal IP00) |

Speed/torque control mode

| Item |  | Specifications |
| --- | --- | --- |
| Performance | Speed control range | 1:6000 (The lower limit is the threshold within which the servo drive keeps running with the rated torque load.) |
|  | Speed loop bandwidth | 2 kHz |
|  | Torque control accuracy | ±1% |
|  | Ramp time | 0 s to 100 s (This parameter can be set for acceleration and deceleration separately.) |
| Input signal | Speed reference input | Source of network-type references: EtherCAT communication |
|  | Torque reference input |  |

Position control mode

| Item |  | Specifications |
| --- | --- | --- |
| Performance | Positioning time | 1 ms to 10 ms |
| Input signal | Position reference | Source of network-type references: EtherCAT communication |
| Digital input (DI) signal | Supporting signal allocation change | P-OT (positive limit switch) N-OT (negative limit switch) HomeSwitch (home switch) TouchProbe1 (probe 1) TouchProbe2 (probe 2) |
| Digital output (DO) signal | Supporting signal allocation change | 3 DOs Load capacity: 50 mA Voltage range: 5 V to 30 V |
|  |  | S-RDY (servo ready) ALM (fault output) BK (brake output) |

Built-in functions

| Item | Specifications |
| --- | --- |
| Overtravel (OT) prevention | The servo drive stops immediately when P-OT or N-OT becomes active |

| Item |  | Specifications |
| --- | --- | --- |
| Protection functions |  | Protections against overcurrent, overvoltage, undervoltage, overload, main circuit detection error, heatsink overheat, overspeed, encoder error, CPU error, and parameter error |
| LED display |  | Main power supply CHARGE indicator, 5-digit LED display |
| Vibration suppression |  | Five notches (including two adaptive notches), 50 Hz to 8000 Hz |
| Communication function | Connection protocol | Serial port 232 and USB |
|  | Communication protocol | EtherCAT |
|  | Multi-station communication | Up to 255 slave stations |
|  | Axis address setting | 0 to 255 set through the software |
|  | Function | Status display, user parameter setting, monitored value display, alarm tracing display, jogging, and speed/torque reference signal observation |
| Other |  | Gain tuning, alarm record, and I/O setting |

1.7    Communication Specifications

| Item |  | Specifications |
| --- | --- | --- |
| Basic performance of EtherCAT slaves | Communication protocol | EtherCAT protocol |
|  | Service supported | CoE (PDO and SDO) |
|  | Synchronization mode | Distributed clock (DC) |
|  | Physical layer | 100BASE-TX |
|  | Baud Rate | 100 Mbps (100BASE-TX) |
|  | Duplex mode | Full duplex |
|  | Topology | Ring and linear |
|  | Transmission medium | Shielded Ethernet cable of Cat5e or above |
|  | Transmission distance | Less than 100 m between two nodes (with proper environment and cables) |
|  | Number of slaves | Up to 65535 supported by protocol, not exceeding 100 in actual use |
|  | EtherCAT frame length | 44 bytes to 1,498 bytes |
|  | Process data | Up to 1486 bytes per Ethernet frame |
|  | Synchronization jitter of two slaves | < 1 μs |
|  | Update time | About 30 μs for 1000 on-off inputs and outputs About 100 μs in case of 100 servo axes The value varies with the interface type. |
|  | Communication bit error ratio | 10-10 Ethernet standard |
| EtherCAT configuration units | Number of FMMU units | 8 |
|  | Number of storage synchronization management units | 8 |
|  | Process data RAM | 8 KB |
|  | Distributed clock | 64 bits |
|  | EEPROM capacity | 32 kbit The initialization data is written by the EtherCAT master. |

Mechanical Installation ▕

Chapter 2	Mechanical Installation

CAUTION

y   To facilitate heat dissipation upward, fix the drive longitudinally on the mounting surface.

y When installing the drive in a control cabinet, take into consideration the temperature change of the cooling air. Rapid temperature drop of the cooling air is not allowed.

y   For installation of multiple servo drives inside the cabinet, install them side by side.

y   For dual-row installation, install an air guide plate.

y   Use the flame-retardant mounting bracket if necessary.

y Ground the grounding terminal properly. Failure to comply may cause an electric shock or malfunction due to interference.

y Route the servo drive cables downwards to prevent liquid from flowing into the servo drive along the cables.

2.1   Installation Environment

| Item | Requirements |
| --- | --- |
| Place | Indoors |
| Grid | Overvoltage category (OVC): III |
| Altitude | Below 1000 m, up to 2000 m. For altitude above 1000 m, derate 1% for every additional 100 m. |
| Temperature | Storage: –40°C to +70°C Operation: 0°C to +55°C (For temperature above 45°C, derate 10% for every additional 5°C.), with temperature change less than 0.5°C/min |
| Humidity | Less than 95% RH, non-condensing |
| Vibration | Less than 4.9 m/s2 |
| Heat dissipation | Install and fix the device to the surface of an incombustible object and leave sufficient surrounding space for heat dissipation |
| Protection | y IP rating: IP20 (except for the power terminal IP00) y Avoid places with direct sunlight exposure, moisture, and water drop y Avoid places with corrosive, combustible, or explosive gas y Avoid places with oil and dust y Avoid places with strong electromagnetic interference y Avoid places with constant vibration or physical shock |

2.2   Installation clearance

The A6-EC drive can only operate in an enclosed housing or control cabinet and must be fitted with a protective device and protective cover. Three clearances are allowed based on the drive power rating and heat dissipation requirements.

CAUTION

y To ensure the cooling effect and uniform temperature distribution in the control cabinet, reserve sufficient space and install a fan. y The drive heats up as the motor runs, and running the drive in an enclosed control cabinet may cause abnormal temperature rise inside the cabinet. Install a cooling device for the drive to meet the ambient temperature requirements.

Clearance for side-by-side installation:

Applicable for all models.

Air outlet	Air outlet	Air outlet

≥ 20mm

≥ 50mm

≥ 20mm

≥ 10mm

≥ 50mm

Air inlet	Air inlet	Air inlet	Air inlet

Clearance for compact installation:

Applicable for models of SIZE A and SIZE B (0.2 kW to 0.75 kW).

≥ 50mm

≥ 1mm

≥ 50mm

Air outlet	Air outlet	Air outlet

≥ 20mm

≥ 20mm

Air inlet	Air inlet	Air inlet	Air inlet

CAUTION

y The distance between adjacent servo drives must be equal to or greater than 1 mm. Take the installation tolerance into consideration. y When adopting compact installation, derate the rated load rate to 75%.

Zero-clearance installation:

Applicable for models of SIZE C, SIZE D (1.0KW to 3.0kW), no derating.

Air outlet	Air outlet	Air outlet

≥ 50mm

A6

A6

A6

A6

≥ 20mm

≥ 20mm

≥ 50mm

Air inlet	Air inlet	Air inlet	Air inlet

2.3   Installation Orientation

The A6-EC drive can only be installed vertically, and improper installation orientation may cause over-

temperature.

CAUTION

y The A6-EC series servo drive has a vertical structure and thus must be installed vertically. Improper installation orientation may cause over-temperature and then damage to the drive.

2.4    Installation Dimensions

SIZE A (rated power: 0.2kW to 0.4 kW)

Front view

40

Side view

150

Ø5

Rear view

28

Top view

170

161

Ø5

Figure 2-1    Installation dimensions for SIZE A (unit: mm)

SIZE B (rated power: 0.75 kW)

Front view

50

Side view

174

Ø5

Rear view

37

Top view

170

161

Ø5

Figure 2-2    Installation dimensions for SIZE B (unit: mm)

SIZE C (rated power: 1.0kW)

Front view

Side view

Rear view

Top view

55

174

Ø5

44

170

160

Ø5

Figure 2-3    Installation dimensions for SIZE C (unit: mm)

Front view

Side view

Rear view

Top view

80

182

Ø5

170

0

Ø5

SIZE D (rated power: 1.5 kW to 3.0 kW)

| 71 |  |
| --- | --- |
|  | 16 |

Figure 2-4    Installation dimensions for SIZE D (unit: mm)

Table 2-1    Dimensions of the outer packing box

| SIZE | Drive Model | Outer Length | Outer Height | Outer Width | Weight |
| --- | --- | --- | --- | --- | --- |
| A | A6-200EC / A6-400EC | 215 mm | 85 mm | 195 mm | 0.78 kg |
| B | A6-750EC | 220 mm | 95 mm | 215 mm | 1.04 kg |
| C | A6-1000EC | 220 mm | 95 mm | 215 mm | 1.20 kg |
| D | A6-1500EC / A6-2000EC / A6-3000EC | 280 mm | 120 mm | 200 mm | 1.70 kg |

2.5    Installation Guide

The A6-EC series servo drive must be installed on a base through a backplate. For the mounting holes, see the dimensions diagrams of each model.

NOTICE

Fixing with upper and lower screws y SIZE-A/B/C: M4 screws, 1 each on top and bottom. Torque: 1.3 N·m to 1.6 N·m y SIZE-D: M4 screws, 2 on top and 1 on bottom. Torque: 1.3 N·m to 1.6 N·m

Figure 2-5    Backplate mounting

y For the tightening torque of the mounting screws, take into account the strength of the screws used and the material of the mounting position, and ensure that there is no looseness or damage.

CAUTION

▏ Electrical Installation

Chapter 3	Electrical Installation

3.1    System Topology

SIZE A

Power supply

Circuit breaker

Filter

Communication cable (Serial cable or USB cable)

EtherCAT cable

A6

Parallel connection

PC

Electromagnetic contactor

I/O cable

PLC

Braking resistor

System ground

Encoder cable

Motor power cable

SIZE B

Power supply

Circuit breaker

Filter

Electromagnetic contactor

Communication cable (Serial cable or USB cable)

EtherCAT cable

A6

Parallel connection

PC

I/O cable

PLC

Braking resistor

System ground

Encoder cable

Motor power cable

NOTICE

y Remove the jumper bar between terminals P and D before connecting an external braking resistor.

SIZE C

Power supply

Circuit breaker	PC

Filter

A6

Communication cable (Serial cable or USB cable)

Electromagnetic contactor

Braking resistor

Parallel connection

EtherCAT cable I/O cable

PC

PLC

System ground

Encoder cable

Motor power cable

NOTICE

y Remove the jumper bar between terminals P and D before connecting an external braking resistor.

SIZE D

Power supply

Circuit breaker

A6

Filter

PC

Communication cable (Serial cable or USB cable)

Electromagnetic contactor

Parallel connection

EtherCAT cable

PC

PLC

Braking resistor	I/O cable

System ground

Encoder cable

Motor power cable

NOTICE

z Remove the jumper bar between terminals P and D before connecting an external braking resistor.

3.2    System Wiring

WARNING

y Only electrical engineering specialists can perform wiring. y Connect an electromagnetic contactor between the input power supply and the main circuit of the drive, to form a structure that can cut off the power supply on the power side of the drive. If no electromagnetic contactor is connected, continuous large current upon drive faults may cause a fire. y Ensure that the input voltage of the drive is within the allowable range. Failure to comply may result in product faults. y Connect the drive protective earth (PE) terminal to that of the control cabinet. Failure to comply may result in an electric shock. y Insulate the connection part of power supply terminals during wiring of the power supply and main circuit. Failure to comply may result in an electric shock. y Ground the entire system. Failure to comply may result in malfunction. y After power-off, wait at least 10 minutes before further wiring operations because residual voltage exists after power-off. Failure to comply may result in an electric shock.

y Never power the servo drive with the IT grid. Use the TN or TT grid instead. Failure to comply may result in an electrical shock. y Do not connect the output terminals U, V, and W of the drive to a three-phase power supply. Failure to comply may result in physical injury or a fire. y Do not connect the motor terminals U, V, and W to a mains power supply. Failure to comply may result in physical injury or a fire. y Do not power on the device before wiring is completed. Failure to comply may result in an electrical shock.

CAUTION

y   Protect external wiring, branches, and short circuits according to local regulations.

y When using peripheral devices, read the user guide for each component and use it properly after fully confirming the precautions.

y   Route the device properly. Improper wiring may cause damage to the drive and motor.

y Connect the drive to the motor directly, without connecting any electromagnetic contactor between them. Failure to comply may result in faults.

y Separate the main circuit cables from the I/O signal cables and encoder cables by at least 30 cm. Failure to comply may result in drive malfunction.

y Use twisted pairs or multi-conductor shielded twisted pairs as I/O signal cables or encoder cables. Failure to comply may result in drive malfunction.

y The maximum wiring lengths of I/O signal cables and encoder cables are 3 m and 10 m, respectively.

y   Use a power filter to reduce electromagnetic interference on electronic devices around the

drive.

y  When wiring, do not allow conductive materials such as wire shavings to fall inside the drive.

y Never place cables under heavy objects or drag cables vigorously. Failure to comply may result in an electric shock due to cable damage.

Host controller

| TD+ | 9 |  |  | TD+ | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TD- | 10 |  |  | TD- | 2 |  |  |
| RD+ | 11 |  |  | RD+ | 3 |  |  |
| RD- | 14 |  |  | RD- | 6 |  |  |
| Next slave |  |  |  |  |  | CN3/CN4 |  |
| TD+ | 1 |  |  | TD+ | 9 |  |  |
| TD- | 2 |  | TD- |  | 10 |  |  |
| RD+ | 3 |  | RD+ |  | 11 |  |  |
| RD- | 6 |  | RD- |  | 14 |  |  |
| Cat5e shielded |  |  |  |  |  |  |  |

twisted-pair Ethernet cable

+24 V power supply

Internal +24 V power supply

+24V

15

A6-EC

1       S-RDY+(DO1+)

Voltage range: 20 V to 28 V Maximum current: 200 mA

COM+       13

P-OT(DI1)       10

4.7kΩ

6       S-RDY-(DO1-)

Positive

limit switch

3       ALM+(DO2+)

Negative limit switch

N-OT(DI2)       9

4.7kΩ

2       ALM-(DO2-)

State output

5       BK+(DO3+)

Home switch

Probe 1

Probe 2

HomeSwitch(DI3)       8

TouchProbe2(DI4)       7

TouchProbe1(DI5)       11

4.7kΩ

4.7kΩ

4.7kΩ

4       BK-(DO3-)

The DO power supply needs to be prepared by users.

Voltage range: 5 V to 24 V

Maximum allowable voltage of the DO port: 30 VDC

Maximum allowable current: 50 mA

COM-       14

PE shield connected to connector housing

Figure 3-1    System wiring

CN6

Commissioning and communication port

CN3

Communication network port IN

CN4

Communication network port OUT

CN1

User control port

CN2

Encoder port

SIZE B

A6

CN6

Commissioning and communication port

CN3

Communication network port IN

CN4

Communication network port OUT

CN1

User control port

CN2

Encoder port

SIZE A

A6

3.3   Ports

| L1/L2 | Main power input |  |
| --- | --- | --- |
| P C | External braking resistor |  |
| N | Bus negative electrode |  |
| U/V/W | Motor power output |  |
| PE | Motor grounding terminal |  |

| L1/L2 | Main power input |  |
| --- | --- | --- |
| P D C | External braking resistor |  |
| N | Bus negative electrode |  |
| U/V/W | Motor power output |  |
| PE | Motor grounding terminal |  |

CN4

Communication network port OUT

CN3

Communication network port IN

CN6

Commissioning and communication port

CN1

User control port

CN2

Encoder port

SIZE C

A6

CN3

Communication network port IN

CN4

Communication network port OUT

CN6

Commissioning and communication port

CN1

User control port

CN2

Encoder port

SIZE D

A6

| L1C/L2C | Control power input |  |
| --- | --- | --- |
| R/S/T | Main power input |  |
| P D C | External braking resistor |  |
| N | Bus negative electrode |  |

| U/V/W | Motor power output |  |
| --- | --- | --- |
| PE | Motor grounding terminal |  |

| L1C/L2C | Control power input |  |
| --- | --- | --- |
| R/S/T | Main power input |  |
| P D C | External braking resistor |  |
| N | Bus negative electrode |  |

| U/V/W | Motor power output |  |
| --- | --- | --- |
| PE | Motor grounding terminal |  |

Table 3-1    Drive terminals

| Terminal | Pin | Description |
| --- | --- | --- |
| Main circuit terminals of SIZE A | L1 and L2: Power input terminals | Connected to the input power supply as per the rated voltage class on the nameplate. |
|  | P and N : Servo bus terminals | Used when multiple servo drives share one DC bus. |
|  | P and C: External braking resistor connection terminals | If an external braking resistor is needed, connect it between terminals P and C. |
|  | U, V, and W: Servo motor connection terminals | Connected to U, V, and W phases of the servo motor. |
|  | PE: Motor grounding terminal | Connected to the grounding terminal of the motor for grounding purpose. |
| Main circuit terminals of SIZE B | L1 and L2: Power input terminals | Connected to the input power supply as per the rated voltage class on the nameplate. |
|  | P and N : Servo bus terminals | Used when multiple servo drives share one DC bus. |
|  | P , D, and C: External braking resistor connection terminals | If an external braking resistor is needed, connect it between terminals P and C. Note: Remove the jumper between terminals P and D before installing an external braking resistor. Otherwise, the braking transistor will be damaged due to overcurrent. |
|  | U, V, and W: Servo motor connection terminals | Connected to U, V, and W phases of the servo motor. |
|  | PE: Motor grounding terminal | Connected to the grounding terminal of the motor for grounding purpose. |

| Terminal | Pin |  | Description |
| --- | --- | --- | --- |
| Main circuit terminals of SIZE C | L1C and L2C: Control circuit power input terminals |  | Connected to the control circuit power supply as per the rated voltage class on the nameplate. |
|  | R, S, and T: Main circuit power input terminals |  | Connected to the main circuit power supply as per the rated voltage class on the nameplate. |
|  | P and N : Servo bus terminals |  | Used when multiple servo drives share one DC bus. |
|  | P , D, and C: External braking resistor connection terminals |  | If an external braking resistor is needed, connect it between terminals P and C. Note: Remove the jumper between terminals P and D before installing an external braking resistor. Otherwise, the braking transistor will be damaged due to overcurrent. |
|  | U, V, and W: Servo motor connection terminals |  | Connected to U, V, and W phases of the servo motor. |
|  | PE: Motor grounding terminal |  | Connected to the grounding terminal of the motor for grounding purpose. |
| 15 5 10 14 4 9 13 3 8 12 2 7 11 1 6 CN1 user control terminal | 10 | DI1 | Positive limit switch |
|  | 9 | DI2 | Negative limit switch |
|  | 8 | DI3 | Home switch |
|  | 7 | DI4 | Probe 2 |
|  | 11 | DI5 | Probe 1 |
|  | 15 | + 24 V | Internal 24 V power supply Voltage range: 20 V to 28 V Max. output current: 200 mA |
|  | 14 | COM– |  |
|  | 13 | COM+ | Common terminal of DI terminals |
|  | 1 | DO1+ | Servo ready |
|  | 6 | DO1– |  |
|  | 3 | DO2+ | Fault |
|  | 2 | DO2– |  |
|  | 5 | DO3+ | Brake |
|  | 4 | DO3– |  |

9

10

11

12/13

14

15/16

1

2

3

4/5

6

7/8

| Terminal | Pin |  | Description |
| --- | --- | --- | --- |
| 5 3 1 6 4 2 CN2 encoder terminal | 1 | + 5 V | 5 V power supply |
|  | 2 | 0 V | - |
|  | 3 | Reserved | - |
|  | 4 | Reserved | - |
|  | 5 | PS+ | Encoder signal |
|  | 6 | PS– |  |
|  | Enclosure | PE | Shield |
| CN3 CN4 EtherCAT communication terminals | 1 | TD+ | Data transmit+ |
|  | 2 | TD– | Data transmit– |
|  | 3 | RD+ | Data receive+ |
|  | 4/5 | - | - |
|  | 6 | RD– | Data receive– |
|  | 7/8 | - | - |
|  | 9 | TD+ | Data transmit+ |
|  | 10 | TD– | Data transmit– |
|  | 11 | RD+ | Data receive+ |
|  | 12/13 | - | - |
|  | 14 | RD– | Data receive– |
|  | 15/16 | - | - |
| CN6 commissioning and communication terminal | Type-c |  | 1: Type-c to serial, serial to USB 2: Type-c→USB |

3.4    Power Supply Connection

CAUTION

y Do not connect the input power cables to the output terminals U, V and W. Failure to comply may cause damage the servo drive. y Do not touch power terminals within 10 minutes after power-off because high voltage exists inside the drive after power off. y Do not bundle power cables and signal cables together or route them through the same duct. Power cables and signal cables must be separated by a distance of at least 30 cm to prevent interference. y Do not turn on and off the power supply frequently (once a minute). Failure to comply may cause drive faults. y Do not power on the servo drive when any screw of the terminal block or any cable becomes loose. Failure to comply may cause a fire.

y When cables are bundled in a metal duct, take the current reduction ratio into consideration because of the inadequate cooling condition of the duct. y Select cables based on the ambient temperature of the controller: When the ambient temperature is high, use heat-resistant cables. Teflon cables are recommended. Take heat preservation measures in low-temperature environments because the surface of regular cables may be easily hardened and cracked under low temperature. y Use cables whose bending radius is over 10 times its outer diameter, to prevent the internal conductor from breaking due to long-term bending. y Use cables that can resist a voltage of 600 V AC or above and have a rated temperature of 75°C or above for the main circuit. y Use a grounding cable with the same cross-sectional area as the main circuit cable. If the cross-sectional area of the main circuit cable is less than 1.6 mm2, use a grounding cable with a cross-sectional area of 2.0 mm2. y Ground the drive reliably. Failure to comply may result in malfunction or even damage of the drive. y Use shielded cables to comply with the EMC requirements.

    - Main circuit wiring

Power supply

Check the power supply specifications

as per the drive nameplate.

Do not use the electromagnetic contactor

to run or stop the motor.

Circuit breaker

L1

U

White

L2

V

Black

W

Filter

Red

Yellow/green

Electromagnetic contactor

CN2

PG

STOP button

P

CN1

ALM-

com

C

1D

24V

Motor

N

ALM+

RUN button

Alarm signal output

Surge protection device

Do not connect the

grounding cable to other cable connectors or make them touch each other.

Alarm output relay

Alarm indicator

Grounding cable

Figure 3-2    Wiring of the single-phase 220 V main circuit

NOTICE

Models with a single-phase 220 V power supply y Model SIZE-A: A6-200EC, A6-400EC y Model SIZE-B: A6-750EC y Model SIZE-C: A6-1000EC (The main circuit can be connected to a single-phase or a three-phase 220 V power supply, depending on which one is available on site.) y Model SIZE-D: A6-1500EC (The main circuit can be connected to a single-phase or a three-phase 220 V power supply, depending on which one is available on site.)

Power supply	Check the power supply specifications

as per the drive nameplate.

Circuit breaker

L1C

L2C

Do not use the electromagnetic contactor

Filter	to run or stop the motor.

U

White

R S

T

V

Black

W

Red

Yellow/green

Electromagnetic contactor

CN2

PG

P

STOP button

D

CN1

ALM-

com

C

1D

24V

Motor

N

ALM+

RUN button

Alarm signal output

Surge protection device

Do not connect the

grounding cable to other cable connectors or make them touch each other.

Alarm output relay

Alarm indicator

Grounding cable

Figure 3-3    Wiring of the three-phase 220 V or 380V main circuit

NOTICE

Models with a three-phase 220 V power supply y Model SIZE-C: A6-1000EC (The main circuit can be connected to a single-phase or a three-phase 220 V power supply, depending on which one is available on site.) y Model SIZE-D: A6-1500EC (The main circuit can be connected to a single-phase or a three-phase 220 V power supply, depending on which one is available on site.) Models with a three-phase 380 V power supply y Model SIZE-D: A6-2000EC, A6-3000EC

    - Cable specifications and recommendations

Table 3-2    Drive input and output current and recommended cables

| Drive Model |  | Rated Input Current | Rated Output Current | Max. Output Current | Input Cable Specifications |
| --- | --- | --- | --- | --- | --- |
| Single-phase 220 V |  |  |  |  |  |
| SIZE A | A6-200EC | 2.3 A | 1.6 A | 5.8 A | 0.75 mm2 |
| SIZE A | A6-400EC | 4 A | 2.8 A | 10.1 A | 0.75 mm2 |
| SIZE B | A6-750EC | 7.9 A | 5.5 A | 16.9 A | 0.75 mm2 |
| SIZE C | A6-1000EC | 9.6 A | 7.6 A | 23 A | 1 mm2 |
| SIZE D | A6-1500EC | 8 A | 11.6 A | 32 A | 0.75 mm2 |
| Three-phase 220 V |  |  |  |  |  |
| SIZE C | A6-1000EC | 5.1 A | 7.6 A | 23 A | 0.75 mm2 |
| Three-phase 380 V |  |  |  |  |  |
| SIZE D | A6-2000EC | 5.6 A | 8.4 A | 20 A | 0.75 mm2 |
| SIZE D | A6-3000EC | 8 A | 11.9 A | 29.8 A | 0.75 mm2 |

Table 3-3    Drive cable specifications and recommendations

| Cable Type | Cable Size | Outer Diameter (OD) |
| --- | --- | --- |
| Power cable | 4 × 12 AWG | 12.2 ± 0.4 mm |
|  | 4 × 14 AWG | 10.5 ± 0.3 mm |
|  | 4 × 16 AWG | 9.5 ± 0.4 mm |
|  | 4 × 18 AWG | 7.8 ± 0.2 mm |
|  | 4 × 20 AWG | 6.5 ± 0.2 mm |
| Power shielded cable | 4 × 12 AWG | 12.9 ± 0.4 mm |
|  | 4 × 14 AWG | 11.2 ± 0.4 mm |
|  | 4 × 16 AWG | 10.1 ± 0.4 mm |
|  | 4 × 18AWG | 8.3 ± 0.2 mm |
|  | 4 × 20 AWG | 6.5 ± 0.2 mm |
| Power cable + brake cable | 4 × 20 AWG + 2 × 24 AWG | 6.5 ± 0.2 mm |
| Brake cable | 2 × 18 AWG | 5.8 ± 0.2 mm |
|  | 2 × 20 AWG | 5.0 ± 0.2 mm |

    - Grounding

CAUTION

y   To prevent electric shock, ground the grounding terminal properly.

y Use grounding cables that meet technical standards for electrical devices and use protective grounding conductors that meet technical specifications. and shorten the grounding cable as much as possible.

y For use of multiple servo drives, ground them all. Improper grounding of the device may cause malfunction of the servo drive and the device.

y Do not use one grounding cable for multiple devices. Improper grounding of the device may result in servo drive or device faults caused by electrical interference.

y For drives equipped with selective grounding screws for VDR and insulation resistor, remove the selective grounding screw for VDR before voltage resistance test. Failure to comply may cause the servo drive to fail the test.

y Install the servo drive on a conductive metal mounting surface. Ensure that the whole conductive bottom of the device is attached properly to the mounting surface.

y Fix the grounding screw with the recommended torque. Avoid loosening or over- tightening of the protective grounding conductor.

Grounding one servo drive alone

PE

- Connect the PE cable on the input power supply side to the input PE terminal of the drive.

- Connect the output PE terminal of the drive to the motor output cable shield.

2

1

Grounding multiple servo drives

Control cabinet

1

Connect the main circuit input PE terminal of the servo drive to the grounding copper busbar of the control cabinet through a protective grounding conductor.

- Connect the PE cable on the input power supply side to the grounding copper busbar of the control cabinet.

- Connect the grounding copper busbar of the control cabinet to the metal housing through a protective grounding conductor.

4

4

1

Connect the motor output cable shield to the output PE terminal of the servo drive.

Grounding copper busbar

2

3

Grounding the control cabinet system

To suppress interference in the control cabinet, isolate the interference source from devices that may be interfered with. Divide the control cabinet into multiple EMC compartments or use multiple control cabinets based on the intensity of interference sources.

CAUTION

System installation principles: y Place the control unit and the drive unit in two separate control cabinets. y For installation involving multiple control cabinets, use a grounding cable with a cross-sectional area of at least 16 mm2 to connect the control cabinets. This is to ensure equipotentiality between the cabinets. y If only one control cabinet is used, place different devices in different compartments of the control cabinet based on signal intensity. y Apply equipotential bonding to devices in different compartments inside the control cabinet. y Shield all communication and signal cables drawn from the control cabinet. y Place the power input filter in a position near the input interface of the control cabinet. y Apply spray coating to each grounding point in the control cabinet.

Controller

Hub

Equipotential bonding

Controller

Drive

Fil

Drive cabinet

Control cabinet

Control area

Equipotential bonding

Drive area

Drive

ter

Filter    Reactor

Reactor

Filter

PE

Achieve all-round connection with the shield

PE

Grid

Grid

Motor cable

Motor cable

M

M

I/O signal cable and communication cable

Equipotential bonding lead and PE

Power cable

Motor cable

Filter

Recommended grounding cable lugs for main circuit

Table 3-4    Recommended grounding cable lugs for power circuit

| Drive Model |  | Rated Output Current | Lug Model of Power Cable | Lug Model of Brake Cable | Lug Model of PE Cable |
| --- | --- | --- | --- | --- | --- |
| SIZE A | A6-200EC | 1.6A | E1008 | E0508 | TVR2-4 |
|  | A6-400EC | 2.8 A | E1008 | E0508 | TVR2-4 |
| SIZE B | A6-750EC | 5.5 A | E1008 | E0508 | TVR2-4 |
| SIZE C | A6-1000EC | 7.6 A | E1508 | E1008 | TVR2-4 |
| SIZE D | A6-1500EC | 11.6 A | E1508 | E1008 | TVR2-4 |
|  | A6-2000EC | 8.4 A | E1508 | E1008 | TVR2-4 |
|  | A6-3000EC | 11.9 A | E1508 | E1008 | TVR2-4 |

B

L

ØD	ØC

Table 3-5    Lug models and dimensions

| Lug model | L | B | ØC | ØD | Color |
| --- | --- | --- | --- | --- | --- |
| E0508 | 14 mm | 8 mm | 1.0 mm | 2.6 mm | Orange |
| E1008 | 14 mm | 8 mm | 1.4 mm | 3.0 mm | Yellow |
| E1508 | 14 mm | 8 mm | 1.7 mm | 3.5 mm | Red |

Table 3-6    Dimensions and appearance of TVR2-4 cable lugs of the grounding cable

| Lug model |  | D | d2 | B | Appearance |
| --- | --- | --- | --- | --- | --- |
| TVR | 2-4 | 4.5mm | 4.3mm | 8.5mm | Ød2 B ØD |

3.5    Motor Connection

Terminal-type motor

| Pin No. | Usage |
| --- | --- |
| 1 | Phase V |
| 2 | Phase U |
| 3 | Phase W |
| 4 | Grounding cable |
| A | Brake (polarity insensitive) |
| B | Brake (polarity insensitive) |

Power input connector

1

- A

- B

4

Power cable without brake

U V W

PE

Power cable with brake

U V W

BK+

BK-

PE

Figure 3-4    Power cable for terminal-type motor

Aviation plug-type motor

| Pin No. | Usage |
| --- | --- |
| A | Phase U |
| B | Phase V |
| C | Phase W |
| D | Grounding cable |

C

  - D

Power input connector

| Pin No. | Usage |
| --- | --- |
| A | Phase U |
| B | Phase V |
| C | Phase W |
| D | Grounding cable |
| 1 | Brake (polarity insensitive) |
| 2 | Brake (polarity insensitive) |

1

- D

2

- C

3.6    Encoder Connection (CN2)

CAUTION

Precautions for wiring of encoder signal cables: y Ground the shield on the drive side and the motor side. Otherwise, the drive will report false alarms. y Do not connect cables to "reserved" terminals. y When determining the length of the encoder cable, take into full account the voltage drop caused by cable resistance and signal attenuation caused by distributed capacitance. Use shielded twisted pairs above 26 AWG (as per UL2464 standard) and keep the length within 10 m.

Terminal-type motor

| Pin No. | Usage |
| --- | --- |
| 1 | DATA+ |
| 2 | DATA- |
| 3 | BAT+ |
| 4 | BAT- |
| 5 | +5V |
| 6 | 0V |
| 7 | FG |

Encoder connector

5	1

6

3

2

7	4

Single-turn encoder cable

Multiple-turn encoder cable

Figure 3-5    Absolute encoder signal cables

CAUTION

Battery box precautions: y Install the battery in the correct direction. Do not pinch the connector cable when closing the battery box cover. y Do not disassemble the battery because the internal electrolyte may spread out and cause physical injury. y Do not short circuit the battery. Failure to comply may deteriorate the battery power and even incur the risk of explosion due to violent overheating. y Before discarding the battery, insulate the battery with tape and then dispose of it according to local regulations.

Aviation plug-type motor

| Pin No. | Usage |
| --- | --- |
| 1 | DATA+ |
| 2 | DATA- |
| 4 | +5V |
| 5 | BAT- |
| 6 | BAT+ |
| 9 | 0V |
| 10 | Enclosure |

Encoder connector

1  2    3

4  5    6    7

8  9    10

3.7    Control Signal Connection (CN1)

    - I/O signal

Use shielded signal cables to protect I/O signal circuits against strong interference noise at the periphery.

y   Use a separate shielded cable for each type of analog signal.

y   Shielded twisted pairs are recommended as digital signal cables.

Figure 3-6    Shielded twisted pairs

CAUTION

y To avoid electromagnetic interference, keep a distance of at least 30 cm between I/O signal cables and power cables (input RST cables, output UVW cables, DC bus, and braking cables).

    - Digital input/output (DI/DO) signals

DI circuits

NOTICE

y The circuits for DI1 to DI5 are the same. The following takes the DI1 circuit as an example.

When the host controller adopts relay output:

y   For use of the internal 24 V power supply of the servo drive.

Servo drive

24V

+24 V power supply     15

COM+     13

DI1 (CMD1)

10

4.7kΩ

COM-     14

Relay

10

4.7kΩ

COM+

10

4.7kΩ

COM-

y   For use of an external power supply.

Servo drive

Servo drive

External

+24 V power supply

External

+24 V power supply

24V

15

+24 V

power supply

24V

Two power

supplies mess up.

15

COM+    13

13

DI1 (CMD1)

DI1 (CMD1)

Relay

Relay

14

14

External 0 V

External 0 V

When the host controller adopts open collector output:

24V

15

13

10

4.7kΩ

14

COM+

24V

15

13

10

4.7kΩ

14

y   For use of the internal 24 V power supply of the servo drive.

Servo drive

Servo drive

+24 V power supply

+24 V power supply

PNP

COM+

DI1 (CMD1)

DI1 (CMD1)

NPN

COM-

COM-

24V

15

13

10

4.7kΩ

14

24V

15

13

10

4.7kΩ

14

y   For use of an external power supply.

Servo drive

Servo drive

External

+24 V power supply

External

+24 V power supply

PNP

COM+

COM+

DI1 (CMD1)

DI1 (CMD1)

NPN

External 0 V

External 0 V

CAUTION

y PNP and NPN inputs cannot be used together in the same circuit.

DO circuits

NOTICE

y The circuits of DO1 to DO3 are the same. The following takes the DO1 circuit as an example.

When the host controller adopts relay input:

1

6

Servo drive

External 5 V to 24 V

power supply

Relay

DO1+

DO1-

External 0 V

CAUTION

y When the host controller adopts relay input, a flywheel diode must be installed. Otherwise, the DO terminals may be damaged.

When the host controller adopts optocoupler input:

NOTICE

y The maximum allowable voltage and maximum current capacity of the optocoupler output circuit inside the servo drive are 30 VDC and DC50 mA, respectively.

Servo drive

External

5 V to 24 V power supply

Servo drive

Optocoupler

External

5 V to 24 V power supply

Current limiting resistor not used

Optocoupler

1     DO1+

1     DO1+

6     DO1-

6     DO1-

External 0 V

External 0 V

    - Wiring of the brake

Some servo motors have a brake inside. The motor brake is used to prevent the movement of the servo motor shaft and keep the motor locked in the position when the servo motor may move unexpectedly due to external forces or its own weight during non-running conditions.

CAUTION

y The motor brake can only be used on a stopped motor and is only used to keep the load stationary. Do not use it to brake a moving load.

NOTICE

y The brake coil has no polarity. y Switch off the S-ON signal after the servo motor stops. y When the motor with a built-in brake runs, the brake may generate a click sound, which does not affect its function. y When brake coils are energized (the brake is released), flux leakage may occur on the shaft end. Pay special attention when using magnetic sensors near the motor.

R

Power supply

Circuit breaker

Filter

S

U V

W

M

T

Check the power supply

specifications as per the drive nameplate.

Electromagnetic contactor

CN2

PG  Encoder

Control brake relay BK-RY

CN1 DO3+ (BK+)

DO3- (BK-)

+24V

BK  Brake

+24 V (brake power supply)

The DC power supply for the brake must be prepared by users.

Figure 3-7    Wiring of the brake

Precautions during wiring:

When determining the length of the motor brake cable, take into full account the voltage drop caused by cable resistance. The input voltage must be at least 21.6 V to enable the brake to work properly.

Table 3-7    Brake specifications

| Motor Model | Holding Torque | Rated Power | Power Supply Voltage |
| --- | --- | --- | --- |
| A6M40-100L2B1-M17 | 0.32N.m | 6.9 W | 24 V DC |
| A6M60-400H2B1-M17 | 1.27 N·m | 7.3 W | 24 V DC |
| A6M80-750H2B1-M17 | 3.2 N·m | 8.5 W | 24 V DC |
| A6M80-1000H2B1-M17 | 3.2 N·m | 8.5 W | 24 V DC |
| A6M100-1500H2B1-M17 | 9 N·m | 22 W | 24 V DC |
| A6M100-2000H3B1-M17 | 8 N·m | 17.6 W | 24 V DC |
| A6M100-2500H3B1-M17 | 9 N·m | 22 W | 24 V DC |
| A6M130-850H2B1-M17 | 12 N·m | 23 W | 24 V DC |
| A6M130-1300H2B1-M17 | 16 N·m | 24 W | 24 V DC |
| A6M130-1800H3B1-M17 | 12 N·m | 23 W | 24 V DC |
| A6M180-2900H3B1-M17 | 50 N·m | 23 W | 24 V DC |

3.8    Communication Signal Connection (CN3 and CN4)

Communication signals are connected by EtherCAT network cables. Connect CN3 (IN) to the communication port of the master and CN4 (OUT) to a slave.

PLC

3(

3(

3(

3(

Figure 3-8    Communication networking topology

CAUTION

y To enhance the system's anti-interference capability, the EtherCAT communication cable must be an Ethernet Category 5 (100BASE-TX) network cable or high-strength shielded network cable of no more than 100 m. y When multiple EtherCAT servo drives are installed, connect the network cables in the sequence of left-in and right-out.

EtherCAT communication supports multiple connection methods:

y   Linear connection

Master

IN	OUT

IN	OUT

IN	OUT

y   Redundant ring connection

Master

IN	OUT

IN	OUT

IN	OUT

Figure 3-9    EtherCAT communication connection

CAUTION

y When a redundant ring network is used, the EtherCAT Enhanced Link Check function must be enabled, which will take effect upon next power-on of the servo drive.

3.9    Communication Terminal Connection (CN6)

You can connect the drive to the PC through the CN6 terminal by using a serial cable (two-part wiring: Type-c to serial, and serial to USB) or a USB cable.

Top view

Type-c-to serial cable

(optional)

Serial-to-USB cable

(prepared by users)

DB9

USB cable

(optional or prepared by users)

Figure 3-10    Communication terminal connection

| DB9 Female Connector (Hole Type) | Pin No. | Signal | Description |
| --- | --- | --- | --- |
| 6 7 8 9 1 2 3 4 5 | 2 | RXD | PC receive end |
|  | 3 | TXD | PC transmit end |
|  | 5 | GND | Ground |
|  | Housing | PE | Shield |

3.10 Braking Resistor Connection

CAUTION

y Do not connect the external braking resistor directly to positive and negative electrodes of the bus. Failure to comply may result in drive damage or even a fire. y Do not select a resistor with resistance lower than the allowable minimum value. Failure to comply may result in an alarm or damage to the servo drive. y Do not contact the external braking resistor during use. Failure to comply may result in burns caused by the hot braking resistor.

y Set parameters of the braking resistor properly before using the servo drive. y Install the external braking resistor on incombustible objects (such as metal).

Electrical Installation ▕

Figure 3-11    Connection of the external braking resistor

NOTICE

y The above connection example is for model SIZE A. Remove the jumper bar between terminals P and D before connecting an external braking resistor for other models.

Table 3-8    Specifications of the braking resistor

| Drive Model |  | Resistance | Resistor Power | Minimum Allowable Resistance of External Resistor | Maximum Braking Energy Absorbed by Capacitor | Braking Resistor Type |
| --- | --- | --- | --- | --- | --- | --- |
| SIZE A | A6-200EC | - | - | 45 Ω | 9.3 J | External braking resistor |
|  | A6-400EC | - | - | 45 Ω | 26.29 J | External braking resistor |
| SIZE B | A6-750EC | 50 Ω | 50 W | 40 Ω | 22.41 J | Built-in braking resistor |
| SIZE C | A6-1000EC | 25 Ω | 80 W | 20 Ω | 26.70 J | Built-in braking resistor |
| SIZE D | A6-1500EC | 25 Ω | 80 W | 15 Ω | 26.70 J | Built-in braking resistor |
| SIZE D | A6-2000EC | 50 Ω | 80 W | 45 Ω | 50.41 J | Built-in braking resistor |
| SIZE D | A6-3000EC | 50 Ω | 80 W | 40 Ω | 50.41 J | Built-in braking resistor |

Chapter 4	Function Overview

4.1    Basic Functions of the Servo Drive

    - Function Overview

The servo system consists of three critical parts: the servo drive, servo motor, and encoder. The servo drive processes input signals and feedback signals to precisely control the position, speed, and torque of the servo motor.

Position loop control

Speed calculation

Drive

Reference

input     +

−

Position feedback

+

−

Speed feedback

Speed loop control

+

−

Current feedback

Motor

Encoder

Current loop control

Figure 4-1    Structure of a servo system

| Para. No. | Name | Description | Default |
| --- | --- | --- | --- |
| C00.00 | Servo mode | 0: Position mode 1: Speed mode 2: Torque mode 10: EtherCAT mode | 10 |

    - Profile Position (PP) Mode

Position control is the most important and common control mode of the servo system. In PP mode, the drive can find the absolute position and relative position of the motor. You can set the target position, start speed, stop speed, acceleration rate, and deceleration rate on the host controller. When the PP mode is enabled, the object dictionary servo mode (6060h) must be set to 1.

Speed feedforward gain  Speed feedforward filter

Target position 607Ah

Profile velocity 6081h

Profile acceleration rate 6084h Profile deceleration rate 6083h Reference polarity 607Eh

2001.15h

(C01.14)

2001.16h

(C01.15)

+

PP profile

generator

Electronic gear

6091.01h

6091.02h

+

Position loop gain   +

2001.01h (C01.00)

−

Position feedback 6063h

Torque feedforward filter  Torque feedforward gain

2001.19h	2001.18h

(C01.18)	(C01.17)

+

+

Speed regulator

2001.02h (C01.01)

2001.03h (C01.02)

+

−

Speed feedback 2040.02h (U40.01)

Max. speed limit

of the motor

Speed reference 2040.01h (U40.00)

Speed calculation

Torque filter 2001.04h (C01.03)

Torque limit     Torque regulator

Motor  Encoder

Figure 4-2    Control flow in PP mode

Basic configurations recommended in PP mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 607Ah Target Position | 6064h Position actual value | Mandatory |
| 6081h Profile velocity | - | Mandatory |
| 6083h Profile acceleration | - | Optional |
| 6084h Profile deceleration | - | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in PP mode:

The following table lists the meanings of each bit in the control word (6040h) in PP mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 | Position reference update | When this bit changes from 0 to 1, the next group of position reference parameters are loaded, including the target position or position increment, start speed, operating speed, acceleration rate, and deceleration rate. |
| 5 | Immediate update | 0: Execute a new reference after the current position reference is executed. 1: Stop executing the current position reference and execute a new one. |
| 6 | Position reference type | 0: Absolute position reference 1: Relative position reference |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 | Halt | 0: Inactive 1: Active The system stops executing references when this bit is set to 1. |

When bit 5 in the control word (6040h) is set to 0, the system waits for the current position reference to be executed before executing the new reference to update the position data in the motion, as shown in the following figure.

Speed

Control word

bit4

Target position

Current position

Status word

bit12

Status word

bit10

When bit 5 in the control word (6040h) is set to 1, the system stops executing the current position reference and immediately executes the new reference to update the position data in the motion, as shown in the following figure.

Speed

Control word

bit4

Target position

Current position

Status word

bit12

Status word

bit10

Status word definition in PP mode:

The following table lists the meanings of each bit in the status word (6041h) in PP mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Position reach | Not supported. This bit remains 1 all the time. |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Receiving status of new position reference | 0: Position reference update allowed 1: Position reference update not allowed |
| 13 | Position deviation error | 0: The position deviation value is within the preset range (6065h) 1: The position deviation value is beyond the preset range (6065h) |
| 14 | Manufacturer-specific | Not supported |
| 15 | Homing completion | 0: Homing not completed 1: Homing completed |

Parameters related to PP mode:

Dictionary objects related to PP mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6064h | 00 | Position feedback | RO | I32 | - | - |
| 6065h | 00 | Excessive position deviation threshold | RW | U32 | 0 to (232-1) | 3145728 |
| 6066h | 00 | Following error time out | RW | U16 | 0 to 65535 | 0 |
| 6067h | 00 | Position reach threshold | RW | U32 | 0 to (232-1) | 734 |
| 6068h | 00 | Position reach time window | RW | U16 | 0 to 65535 | 0 |
| 607Ah | 00 | Target position | RW | I32 | -231 to (231-1) | 0 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 607Fh | 00 | Max. speed | RW | U32 | 0 to (232-1) | 104857600 |
| 6081h | 00 | Profile operating speed | RW | U32 | 0 to (232-1) | 1747627 |
| 6083h | 00 | Profile acceleration rate | RW | U32 | 0 to (232-1) | 1747626667 |
| 6084h | 00 | Profile deceleration rate | RW | U32 | 0 to (232-1) | 1747626667 |

PP mode example:

Start and operation processes in PP mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 1 |
| 607Ah | Position reference | Set by the user |
| 6081h | Set speed in PP mode | The rotation speed is 600 rpm for writing data of 1310720 bits at the default gear ratio of 1:1. |

| Address | Name | Value |
| --- | --- | --- |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Set absolute position (not immediate update) | 6→7→15→31 |
|  | Set absolute position (immediate update) | 6→7→47→63 |
|  | Set relative position (not immediate update) | 6→7→79→95 |
|  | Set relative position (immediate update) | 6→7→111→127 |
| 6083h | Profile acceleration rate | Default: 13107200 |
| 6084h | Profile deceleration rate | Default: 1310720 |

    - Profile Velocity (PV) Mode

In PV mode, the host controller sends the target velocity, acceleration rate, and deceleration rate to the servo drive. The servo drive generates the speed reference profiles and executes speed control and torque control. When the PV mode is enabled, the object dictionary control mode (6060h) must be set to 3.

Torque feedforward Torque feedforward gain	filter

2001.18h	2001.19h

(C01.17)	(C01.18)

+

Target velocity 60FFh

607Eh

Acceleration rate and

deceleration rate      Max. speed limit

+

Speed regulator

2001.02h (C01.01)

2001.03h (C01.02)

+

6083h

6084h

607Fh

−

Torque filter 2001.04h (C01.03)

Torque limit

Torque regulator

Actual torque feedback 6077h

Motor

Encoder

Figure 4-3    Control flow in PV mode

Basic configurations recommended in PV mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 60FFh Target Velocity | - | Mandatory |
| - | 6064h Position actual value | Optional |
| - | 606Ch Velocity actual value | Optional |
| 6083h Profile acceleration | - | Optional |
| 6084h Profile deceleration | - | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in PV mode:

The following table lists the meanings of each bit in the control word (6040h) in PV mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 to 6 | Reserved for PV mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 | Halt | 0: Inactive 1: Active (The system halts when this bit is set to 1 and executes references when this bit is set to 0.) |
| 9 | Reserved for PV mode | Not supported |
| 10 | Reserved for PV mode | Not supported |

Status word definition in PV mode:

The following table lists the meanings of each bit in the status word (6041h) in PV mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Speed reach | 0: Speed not reached 1: Speed reached |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Zero speed status | 0: Speed not equal to 0 1: Speed equal to 0 |
| 13 to 15 | Reserved for PV mode | Not supported |

Parameters related to PV mode:

Dictionary objects related to PV mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 606Ch | 00 | Actual speed | RO | I32 | - | - |
| 606Dh | 00 | Speed reach threshold | RW | U16 | 0 to 65535 | 10 |
| 606Eh | 00 | Speed reach time window | RW | U16 | 0 to 65535 | 0 |
| 606Fh | 00 | Zero speed threshold | RW | U16 | 0 to 65535 | 10 |
| 6070h | 00 | Zero speed time window | RW | U16 | 0 to 65535 | 0 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 607Fh | 00 | Max. speed | RW | U32 | 0 to (232-1) | 104857600 |
| 6083h | 00 | Profile acceleration rate | RW | U32 | 0 to (232-1) | 1747626667 |
| 6084h | 00 | Profile deceleration rate | RW | U32 | 0 to (232-1) | 1747626667 |
| 60FFh | 00 | Target velocity | RW | I32 | -231 to (231-1) | 0 |

PV mode example:

Start and operation processes in PV mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 3 |
| 60FFh | Set speed in PV mode | The rotation speed is 600 rpm for writing data of 1310720 bits at the default gear ratio of 1:1. |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Motor moment | The motor rotates at the set speed reference after being enabled. |
| 60830h | Profile acceleration rate | Default: 13107200 |
| 6084h | Profile deceleration rate | Default: 1310720 |

    - Profile Torque (PT) Mode

In PT mode, the host controller sends the target torque and torque reference change rate (torque slope) to the servo drive. The servo drive generates the torque reference profiles and executes torque control. When the PT mode is enabled, the object dictionary control mode (6060h) must be set to 4.

2001.02h (C01.01)

2001.03h (C01.02)

Speed regulator   Max. speed limit

607Fh

Selection upon speed limit

Actual torque feedback

6077h

Target torque 6071h

Torque slope 6087h

607Eh

Torque filter 2001.04h (C01.03)

Torque limit     Torque regulator

Motor  Encoder

Figure 4-4    Control flow in PT mode

Basic configurations recommended in PT mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 6071h Target Torque | - | Mandatory |
| 6087h Torque slope | - | Optional |
| - | 6064h Position actual value | Optional |
| - | 606Ch Velocity actual value | Optional |
| - | 6077h Torque actualvalue | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in PT mode:

The following table lists the meanings of each bit in the control word (6040h) in PT mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 to 6 | Reserved for PT mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |

| Bit | Name | Description |
| --- | --- | --- |
| 8 | Halt | 0: Inactive 1: Active (The system halts when this bit is set to 1 and executes references when this bit is set to 0.) |
| 9 to 10 | Reserved for PT mode | Not supported |
| 11 to 15 | Manufacturer-specific | Not supported |

Status word definition in PT mode:

The following table lists the meanings of each bit in the status word (6041h) in PT mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Torque reached | 0: Torque not reached 1: Torque reached |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |

Parameters related to PT mode:

Dictionary objects related to PT mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6071h | 00 | Target torque | RW | I16 | -4000 to 4000 | 0 |
| 6072h | 00 | Max. torque | RW | U16 | 0 to 4000 | 3000 |
| 6074h | 00 | Torque reference | RO | I16 | - | - |
| 6077h | 00 | Actual torque | RO | I16 | - | - |
| 6087h | 00 | Torque slope | RW | U32 | 0 to (232-1) | 232-1 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 607Fh | 00 | Max. speed | RW | U32 | 0 to (232-1) | 104857600 |
| 60E0h | 00 | Positive torque limit | RW | U16 | 0 to 4000 | 3000 |
| 60E1h | 00 | Negative torque limit | RW | U16 | 0 to 4000 | 3000 |
| 2003h | 4a | Reference value for torque reach | RW | U16 | 0 to 4000 | 0 |
| 2003h | 4b | Torque output value when DO signal for torque reach turned on | RW | U16 | 0 to 4000 | 20 |
| 2003h | 4c | Torque output value when DO signal for torque reach turned off | RW | U16 | 0 to 4000 | 10 |

PT mode example:

Start and operation processes in PT mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 4 |
| 6071h | PT reference | User-defined |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Motor moment | Reference after enabled |
| 6087h | Torque slope | User-defined (acceleration/deceleration ramp in torque mode) |

NOTICE

Torque limit: y To protect mechanical devices, you can limit the torque references of the drive in each position, speed, and torque control mode by setting the maximum torque 6072h, positive torque limit 60E0h, and negative torque limit 60E1h. However, the torque cannot exceed the maximum torque allowed by the drive.

    - Homing Mode (HM)

The CiA402 protocol defines 33 homing modes according to the home switch (HSW) signal, limit switch signal, and encoder Z signal. When the mode is enabled, the object dictionary control mode (6060h) must be set to 6.

Speed feedforward Speed feedforward

Homing method 6098h

Homing speed 6099h Homing acceleration 609Ah

Homing timeout interval 2010.09h (C10.08)

gain 2001.15h (C01.14)

filter 2001.16h (C01.15)

+

Homing profile

generator

Electronic gear

6091.01h

6091.02h

+

−

Position loop gain  +

2001.01h

(C01.00)

Position feedback 6063h

Torque feedforward Torque feedforward

filter 2001.19h (C01.18)

gain 2001.18h (C01.17)

+

+

Speed regulator

2001.02h (C01.01)

2001.03h (C01.02)

+

−

Speed feedback 2040.02h (U40.01)

Max. speed limit

of the motor

Speed reference 2040.01h (U40.00)

Speed calculation

Torque filter

2001.04h	Torque limit    Torque regulator

(C01.03)

Motor Encoder

Figure 4-5    Control flow in homing mode

Basic configurations recommended in homing mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 6098h Homing method | - | Optional |
| 6099.01h Speed during search for switch | - | Optional |
| 6099.02h Speed during search for zero | - | Optional |
| 609Ah Homing acceleration | - | Optional |
| - | 6064h Position actual value | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in homing mode:

The following table lists the meanings of each bit in the control word (6040h) in homing mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 | Homing enable | 0: Inactive 1: Active (indicating that the homing process is initiated and homing must be active throughout the homing process. If the value is switched to inactive, the homing process is stopped.) |
| 5 to 6 | Reserved for homing mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 | Halt | 0: Inactive 1: Active (indicating that the homing process is decelerated and stopped) |
| 9 | Reserved for homing mode | Not supported |
| 10 | Reserved for homing mode | Not supported |
| 11 to 15 | Manufacturer-specific | Not supported |

Status word definition in homing mode:

The following table lists the meanings of each bit in the status word (6041h) in homing mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Position reach | 1: Home located or homing interrupted |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Homing completion output | 0: Homing not completed 1: Homing completed |
| 13 | Homing error | 0: Homing error not occurred 1: Homing error occurred |
| 14 | Manufacturer-specific | Not supported |
| 15 | Homing completion | 0: Homing not completed 1: Homing completed |

Parameters related to homing mode:

Dictionary objects related to homing mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6064h | 00 | Position feedback | RO | I32 | - | - |
| 6065h | 00 | Excessive position deviation threshold | RW | U32 | 0 to (232-1) | 3145728 |
| 6066h | 00 | Following error time out | RW | U16 | 0 to 65535(ms) | 0 |
| 607Ch | 00 | Home offset | RW | I32 | -231 to (231-1) | 0 |
| 607Fh | 00 | Max. speed | RW | U32 | 0 to (232-1) | 104857600 |
| 6098h | 00 | Homing method | RW | I8 | 1 to 35 | 1 |
| 6099h | 01 | Speed during search for switch | RW | U32 | 0 to (232-1) | 1747627 |
|  | 02 | Speed during search for zero | RW | U32 | 10 to (232-1) | 174763 |
| 609Ah | 00 | Acceleration rate | RW | U32 | 0 to (232-1) | 1747626667 |
| 60E6h | 00 | Homing position calculation method | RW | U8 | 0 to 1 | 0 |
| 60C5h | 00 | Max. profile acceleration rate | RW | U32 | 0 to (232-1) | 231-1 |
| 2010h | 09 | Homing time limit | RW | U32 | 0 to (232-1) | 60000 |

Homing mode example:

Start and operation processes in homing mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 6 |
| 6098h | Homing mode | -2 to 35 |
| 6040h (control word) | Alarm clearance | Random number → 128 (rising edge active) |
|  | Homing | 6 → 7 → 15 → 31 (homing enabled when bit 4 reaches the rising edge) |
| 6099-01h | Speed during search for switch in homing mode | Default: 13981013 |
| 6099-02h | Speed during search for zero in homing mode | Default: 1398101 |
| 609Ah | Homing acceleration rate | Default: 131072 |

Homing mode introduction:

Table 4-1    Mode overview table

| Mode setting | Description |
| --- | --- |
| -2 | The mode is used to search for the mechanical extreme position and Z pulse in the forward direction. |
| -1 | The mode is used to search for the mechanical extreme position and Z pulse in the reverse direction. |
| 0 | - |
| 1 | The motor starts operation in the reverse direction, switches to low-speed operation when the negative limit (NL) status changes from OFF to ON during reverse operation, and then retreats to search for the nearest Z pulse position as the home. |
| 2 | The motor starts operation in the forward direction, switches to low-speed operation when the positive limit (PL) status changes from OFF to ON during forward operation, and then retreats to search for the nearest Z pulse position as the home. |
| 3 | The motor operates in the forward direction when HSW is inactive and reverse direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from ON to OFF during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |
| 4 | The motor operates in the forward direction when HSW is inactive and reverse direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from OFF to ON during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 5 | The motor operates in the reverse direction when HSW is inactive and forward direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from ON to OFF during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 6 | The motor operates in the reverse direction when HSW is inactive and forward direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from OFF to ON during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |
| 7 | The motor operates in the forward direction when HSW is inactive and reverse direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from ON to OFF during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |

| Mode setting | Description |
| --- | --- |
| 8 | The motor operates in the forward direction when HSW is inactive and reverse direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from OFF to ON during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 9 | The motor operates in the forward direction no matter whether HSW is active or inactive. The motor switches to low-speed operation when the HSW status changes from OFF to ON during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |
| 10 | The motor operates in the forward direction no matter whether HSW is active or inactive. The motor switches to low-speed operation when the HSW status changes from ON to OFF during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 11 | The motor operates in the reverse direction when HSW is inactive and forward direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from ON to OFF during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 12 | The motor operates in the reverse direction when HSW is inactive and forward direction when HSW is active. The motor switches to low-speed operation when the HSW status changes from OFF to ON during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |
| 13 | The motor operates in the reverse direction no matter whether HSW is active or inactive. The motor switches to low-speed operation when the HSW status changes from OFF to ON during forward operation, and then continues the forward operation to search for the nearest Z pulse position as the home. |
| 14 | The motor operates in the reverse direction no matter whether HSW is active or inactive. The motor switches to low-speed operation when the HSW status changes from ON to OFF during reverse operation, and then continues the reverse operation to search for the nearest Z pulse position as the home. |
| 15 | Reserved |
| 16 | Reserved |
| 17 | Similar to mode 1. During reverse operation, the position where the NL status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 18 | Similar to mode 2. During forward operation, the position where the PL status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 19 | Similar to mode 3. During reverse operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |

| Mode setting | Description |
| --- | --- |
| 20 | Similar to mode 4. During forward operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 21 | Similar to mode 5. During forward operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |
| 22 | Similar to mode 6. During reverse operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 23 | Similar to mode 7. During reverse operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |
| 24 | Similar to mode 8. During forward operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 25 | Similar to mode 9. During reverse operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 26 | Similar to mode 10. During forward operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |
| 27 | Similar to mode 11. During forward operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |
| 28 | Similar to mode 12. During reverse operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 29 | Similar to mode 13. During forward operation, the position where the HSW status changes from OFF to ON is used as the home, without searching for the Z pulse. |
| 30 | Similar to mode 14. During reverse operation, the position where the HSW status changes from ON to OFF is used as the home, without searching for the Z pulse. |
| 31 | Reserved |
| 32 | Reserved |
| 33 | The motor starts to operate in the reverse direction, and searches for the nearest Z pulse position as the home. |
| 34 | The motor starts to operate in the forward direction, and searches for the nearest Z pulse position as the home. |
| 35 | The current position is used as the home. |

Home mode:

NL

HSW

PL

Start position

H

Direction

change

﹣L

High speed

Direction change

Low speed

Home	(reverse direction)

H

Operating

direction

Active state

PL signal

Inactive state

HSW signal

Active state

Position to switch to low speed

Inactive state

Z pulse train

Z pulse position

Figure 4-6    Definition of the home mode

Mode-2: The mode is used to search for the mechanical extreme position and Z pulse in the forward direction.

y The motor starts to operate in the forward direction at a high speed. After the motor runs into the mechanical extreme position, if the torque reaches the torque limit value, the speed is near zero, and the state is maintained for a certain period of time, the axis reaches the mechanical extreme position. The motor switches to operate in the reverse direction at a low speed, and searches for the nearest Z pulse position as the home.

Mechanical extreme position

H

Motion profile

H

﹣L

Z pulse train

Mechanical limit

Figure 4-7    Home mode-2 profile and signal status

Mode-1: The mode is used to search for the mechanical extreme position and Z pulse in the reverse direction.

y The motor starts to operate in the reverse direction at a high speed. After the motor runs into the mechanical extreme position, if the torque reaches the torque limit value, the speed is near zero, and the state is maintained for a certain period of time, the axis reaches the mechanical extreme position. The motor switches to operate in the forward direction at a low speed, and searches for the nearest Z pulse position as the home.

Mechanical extreme position

﹣H

Motion profile

L

H

Z pulse train

Mechanical limit

Figure 4-8    Home mode-1 profile and signal status

Mode 1: Search for the NL and Z pulse.

y If the NL is inactive upon startup, the motor operates in the reverse direction at a high speed. When the NL status changes from OFF to ON, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the NL status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y   If the NL is active upon startup, the motor operates in the forward direction at a low speed. When the NL status changes from ON to OFF during forward operation, the motor continues the forward operation to search for the nearest Z pulse position as the home.

NL

﹣H

Motion profile 1

L

H

Motion profile 2

L

H

Z pulse train

NL signal

Figure 4-9    Home mode 1 profile and signal status

Mode 2: Search for the PL and Z pulse.

y If the PL is inactive upon startup, the motor operates in the forward direction at a high speed. When the PL status changes from OFF to ON, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the PL status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the PL is active upon startup, the motor operates in the reverse direction at a low speed. When the PL status changes from ON to OFF during reverse operation, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

PL

H

Motion profile 1

H

﹣L

Motion profile 2

H

﹣L

Z pulse train

PL signal

Figure 4-10    Home mode 2 profile and signal status

Mode 3: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position and Z pulse.

y If the HSW is inactive upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

H

Motion profile 1

H

﹣L

Motion profile 2

H

﹣L

Z pulse train

HSW signal

Figure 4-11    Home mode 3 profile and signal status

Mode 4: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position and Z pulse.

y If the HSW is inactive upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

H

Motion profile 1

﹣L

L

H

﹣L

Motion profile 2

L

H

Z pulse train

HSW signal

Figure 4-12    Home mode 4 profile and signal status

Mode 5: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position and Z pulse.

y If the HSW is inactive upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

﹣H

Motion profile 1

L

H

Motion profile 2

L

H

Z pulse train

HSW signal

Figure 4-13    Home mode 5 profile and signal status

Mode 6: When the HSW status changes from OFF to ON during reverse operation, the motor searches for the position and Z pulse.

y If the HSW is inactive upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

﹣H

Motion profile 1

L

H

﹣L

L

Motion profile 2

H

﹣L

Z pulse train

HSW signal

Figure 4-14    Home mode 6 profile and signal status

Mode 7: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates and continues to operate in the reverse direction at

a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in

the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

﹣L

﹣H

Motion profile 1

L

H

﹣L

Motion profile 2

H

H

﹣L

Motion profile 3

H

﹣L

Z pulse train

HSW signal

PL signal

Figure 4-15    Home mode 7 profile and signal status

Mode 8: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and

then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates, and continues to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, and then

switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣L

﹣H

L

H

H

Motion profile 2

﹣L

L

H

﹣L

Motion profile 3

L

H

Z pulse train

HSW signal

PL signal

Figure 4-16    Home mode 8 profile and signal status

Mode 9: When the HSW status changes from OFF to ON during reverse operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop,

and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates, and continues to operate in the forward direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣H

L

H

﹣L

H

L

Motion profile 2

H

﹣L

L

Motion profile 3

H

﹣L

Z pulse train

HSW signal

PL signal

Figure 4-17    Home mode 9 profile and signal status

Mode 10: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates, and continues to operate in the forward direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣H

L

H

H

L

Motion profile 2

﹣L

L

H

Motion profile 3

L

H

Z pulse train

HSW signal

PL signal

Figure 4-18    Home mode 10 profile and signal status

Mode 11: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position and Z pulse and automatically turns to the forward direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates and continues to operate in the forward direction at

a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to

the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣H

Motion profile 1

L

﹣H

H

H

L

Motion profile 2

﹣L

L

H

Motion profile 3

L

H

Z pulse train

HSW signal

NL signal

Figure 4-19    Home mode 11 profile and signal status

Mode 12: When the HSW status changes from OFF to ON during reverse operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor

continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates, and continues to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣H

Motion profile 1

L

H

﹣L

﹣H

H

L

Motion profile 2

H

﹣L

L

Motion profile 3

H

﹣L

Z pulse train

HSW signal

NL signal

Figure 4-20    Home mode 12 profile and signal status

Mode 13: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates, and continues to operate in the reverse direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop,

and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor continues the forward operation to search for the nearest Z pulse position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

Motion profile 1

﹣L

﹣H

L

H

﹣H

H

Motion profile 2

﹣L

L

H

﹣L

Motion profile 3

L

H

Z pulse train

HSW signal

NL signal

Figure 4-21    Home mode 13 profile and signal status

Mode 14: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position and Z pulse and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates and continues to operate in the reverse direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor continues the reverse operation to search for the nearest Z pulse position as the home.

y   If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the

HSW status changes from ON to OFF during reverse operation at a low speed, the motor searches for the nearest Z pulse position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣L

﹣H

Motion profile 1

L

H

﹣L

﹣H

Motion profile 2

H

H

﹣L

Motion profile 3

H

﹣L

Z pulse train

HSW signal

NL signal

Figure 4-22    Home mode 14 profile and signal status

Mode 15 and mode 16: Reserved Mode 17: Search for the NL.

y   If the NL is inactive upon startup, the motor operates in the reverse direction at a high speed. When the

NL status changes from OFF to ON, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the NL status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the NL is active upon startup, the motor operates in the forward direction at a low speed. When the NL status changes from ON to OFF during forward operation, the motor decelerates to stop and uses the stop position as the home.

NL

﹣H

Motion profile 1

L

H

Motion profile 2

L

H

NL signal

Figure 4-23    Home mode 17 profile and signal status

Mode 18: Search for the PL.

y If the PL is inactive upon startup, the motor operates in the forward direction at a high speed. When the PL status changes from OFF to ON, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the PL status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the PL is active upon startup, the motor operates in the reverse direction at a low speed. When the PL status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

PL

H

Motion profile 1

H

﹣L

﹣L

Motion profile 2

H

PL signal

Figure 4-24    Home mode 18 profile and signal status

Mode 19: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position.

y If the HSW is inactive upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor uses the stop position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

H

Motion profile 1

﹣L

H

Motion profile 2

H

﹣L

HSW signal

Figure 4-25    Home mode 19 profile and signal status

Mode 20: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position.

y If the HSW is inactive upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

H

Motion profile 1

﹣L

L

H

﹣L

Motion profile 2

L

H

HSW signal

Figure 4-26    Home mode 20 profile and signal status

Mode 21: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position.

y If the HSW is inactive upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor uses the stop position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

﹣H

Motion profile 1

L

H

Motion profile 2

L

H

HSW signal

Figure 4-27    Home mode 21 profile and signal status

Mode 22: Search for the PL.

y  If the HSW is inactive upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y In this mode, no matter whether the NL or PL is in ON state, the homing process is stopped and an alarm is reported.

HSW

﹣H

Motion profile 1

L

H

﹣L

L

Motion profile 2

H

﹣L

HSW signal

Figure 4-28    Home mode 22 profile and signal status

Mode 23: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates and continues to operate in the reverse direction at

a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y  If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates

in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor uses the stop position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣L

﹣H

L

H

﹣L

H

Motion profile 2

H

﹣L

Motion profile 3

H

﹣L

HSW signal

PL signal

Figure 4-29    Home mode 23 profile and signal status

Mode 24: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates, and continues to operate in the reverse direction at

a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣L

﹣H

L

H

H

Motion profile 2

﹣L

L

H

﹣L

Motion profile 3

L

H

HSW signal

PL signal

Figure 4-30    Home mode 24 profile and signal status

Mode 25: When the HSW status changes from OFF to ON during reverse operation, the motor searches for the position and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop,

and then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is

inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates, and continues to operate in the forward direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣H

L

H

﹣L

H

L

Motion profile 2

H

﹣L

L

Motion profile 3

H

﹣L

HSW signal

PL signal

Figure 4-31    Home mode 25 profile and signal status

Mode 26: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position and automatically turns to another direction upon the PL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the PL is in ON state, the motor decelerates to stop, and

then operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates and continues to operate in the forward direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor uses the stop position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

HSW

PL

H

Motion profile 1

﹣H

L

H

L

H

Motion profile 2

﹣L

L

H

Motion profile 3

L

H

HSW signal

PL signal

Figure 4-32    Home mode 26 profile and signal status

Mode 27: When the HSW status changes from ON to OFF during forward operation, the motor searches for the position and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates and continues to operate in the forward direction at

a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor uses the stop position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣H

Motion profile 1

﹣H

L

H

H

L

Motion profile 2

﹣L

L

H

Motion profile 3

L

H

HSW signal

NL signal

Figure 4-33    Home mode 27 profile and signal status

Mode 28: When the HSW status changes from OFF to ON during reverse operation, the motor searches for the position and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates, and continues to operate in the forward direction at a low speed. When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to

ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as

the home.

y If the HSW is active upon startup, the motor operates in the forward direction at a low speed. When the HSW status changes from ON to OFF during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from OFF to ON during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣H

Motion profile 1

L

﹣L

﹣H

H

Motion profile 2

H

L

H

﹣L

L

Motion profile 3

H

﹣L

HSW signal

NL signal

Figure 4-34    Home mode 28 profile and signal status

Mode 29: When the HSW status changes from OFF to ON during forward operation, the motor searches for the position and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates, and continues to operate in the reverse direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop,

and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, returns to the position where the HSW is inactive at a low speed, decelerates to stop again, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor decelerates to stop, and then switches to operate in the forward direction at a low speed. When the HSW status changes from OFF to ON during forward operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣L

﹣H

Motion profile 1

L

H

﹣H

H

Motion profile 2

﹣L

L

H

﹣L

Motion profile 3

L

H

HSW signal

NL signal

Figure 4-35    Home mode 29 profile and signal status

Mode 30: When the HSW status changes from ON to OFF during reverse operation, the motor searches for the position and automatically turns to another direction upon the NL.

y If the HSW is inactive and located in the forward direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the HSW status changes from OFF to ON during reverse operation, the motor decelerates and continues to operate in the reverse direction at a low speed.

When the HSW status changes from ON to OFF, the motor decelerates to stop, returns to the position where the HSW is active at a low speed, decelerates to stop again (if the active HSW range is narrow, it may enter the inactive HSW position range on the other side), and switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is inactive and located in the reverse direction of the HSW upon startup, the motor operates in the reverse direction at a high speed. When the NL is in ON state, the motor decelerates to stop, and then operates in the forward direction at a high speed. When the HSW status changes from OFF to ON during forward operation, the motor decelerates to stop, and then switches to operate in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation at a low speed, the motor decelerates to stop and uses the stop position as the home.

y If the HSW is active upon startup, the motor operates in the reverse direction at a low speed. When the HSW status changes from ON to OFF during reverse operation, the motor uses the stop position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

HSW

﹣L

﹣H

Motion profile 1

L

H

﹣L

﹣H

H

Motion profile 2

H

﹣L

Motion profile 3

H

﹣L

HSW signal

NL signal

Figure 4-36    Home mode 30 profile and signal status

Mode 31 and mode 32: Reserved

Mode 33: The motor searches for the nearest Z pulse during reverse operation.

y The motor starts to operate in the reverse direction at a low speed, and searches for the nearest Z pulse position as the home. If the motor encounters the ON state of NL before the Z pulse during reverse operation, the motor decelerates to stop, and then operates in the forward direction to search for the nearest Z pulse position as the home.

y   In this mode, the motor automatically turns to another direction when it encounters the ON state of the NL for the first time during reverse operation. When the motor encounters the ON state of the PL or encounters the ON state of the NL again, the homing process is stopped and an alarm is reported.

NL

Motion profile 1

H

﹣L

﹣L

Motion profile 2

L

H

Z pulse train

NL signal

Figure 4-37    Home mode 33 profile and signal status

Mode 34: The motor searches for the nearest Z pulse during forward operation.

y The motor starts to operate in the forward direction at a low speed, and searches for the nearest Z pulse position as the home. If the motor encounters the ON state of PL before the Z pulse during forward operation, the motor decelerates to stop, and then operates in the reverse direction to search for the nearest Z pulse position as the home.

y In this mode, the motor automatically turns to another direction when it encounters the ON state of the PL for the first time during forward operation. When the motor encounters the ON state of the NL or encounters the ON state of the PL again, the homing process is stopped and an alarm is reported.

PL

L

Motion profile 1

H

L

Motion profile 2

H

﹣L

Z pulse train

PL signal

Figure 4-38    Home mode 34 profile and signal status

Mode 35: The current position is used as the home.

y Homing mode 35: The present position is taken as the mechanical home. After homing is triggered (control word 6040h: 0x0F → 0x1F):

60E6 = 0 (absolute homing):

6064 (position feedback) is set to 607C (home offset) after homing is done. 60E6 = 1 (relative homing):

6064 (position feedback) is the sum of the original value plus 607C (home offset) after homing is done.

    - Cyclic Synchronous Position (CSP) Mode

In CSP mode, the host controller generates the start speed, stop speed, acceleration rate, and deceleration rate for reaching the target position, and sends the absolute target position to the servo drive cyclically. The servo drive operates by following the target position. When the CSP mode is enabled, the object dictionary control mode (6060h) must be set to 8.

Torque offset 60B2h

Torque feedforward  Torque feedforward

filter

2001.19h

(C01.18)

gain 2001.18h (C01.17)

Speed deviation 60B1h  Speed feedforward

gain 2001.15h (C01.14)

Speed feedforward filter

2001.16h (C01.15)

Position offset 60B0h

Target position 607Ah  + +

−

Gear ratio

6091.01h

6091.02h

+

+

−

Speed limit

Speed regulator       + +

2001.02h (C01.01)

2001.03h (C01.02)

Torque filter 2001.04h (C01.03)

Torque limit

Torque regulator

Speed calculation

Actual torque feedback 6077h

Motor

Encoder

Figure 4-39    Control in CSP mode

Basic configurations recommended in CSP mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 607Ah Target position | 6064h Position actual value | Mandatory |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in CSP mode:

The following table lists the meanings of each bit in the control word (6040h) in CSP mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |

| Bit | Name | Description |
| --- | --- | --- |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 to 6 | Reserved for CSP mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 to 10 | Reserved for CSP mode | Not supported |
| 11 to 15 | Manufacturer-specific | Not supported |

Status word definition in CSP mode:

The following table lists the meanings of each bit in the status word (6041h) in CSP mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Position reach | Not supported. This bit remains 1 all the time. |

| Bit | Name | Description |
| --- | --- | --- |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Target position following | This bit remains 1 all the time. |
| 13 | Followed position error alarm | 0: No alarm generated 1: Alarm generated |
| 14 to 15 | Manufacturer-specific | Not supported |

Parameters related to CSP mode:

Dictionary objects related to CSP mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6064h | 00 | Position feedback | RO | I32 | - | - |
| 6065h | 00 | Excessive position deviation threshold | RW | I32 | 0 to (232-1) | 3145728 |
| 6066h | 00 | Following error time out | RW | U16 | 0 to 65535 | 0 |
| 606Ch | 00 | Actual speed | RO | I32 | - | - |
| 6077h | 00 | Actual torque | RO | I16 | - | - |
| 607Ah | 00 | Target position | RW | I32 | -231 to (231-1) | 0 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 60B0h | 00 | Position offset | RW | I32 | -231 to (231-1) | 0 |
| 60B1h | 00 | Speed deviation | RW | I32 | -231 to (231-1) | 0 |
| 60B2h | 00 | Torque offset | RW | I16 | -4000 to 4000 | 0 |
| 60F4h | 00 | Position deviation | RO | I32 | - | - |

CSP mode example:

Start and operation processes in CSP mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 8 |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 or MC_Power |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Axis fault reset | Host controller reference or PLC reference MC_Reset |
| 607Ah | Position reference | Host controller reference (including the acceleration rate and deceleration rate) |
|  | Analog speed control | Host controller reference, PLC reference MC_MoveVelocity |
|  | Relative position reference | Host controller reference, PLC reference MC_MoveRelative |
|  | Incremental position reference | Host controller reference, PLC reference MC_MoveAdditive |
|  | Absolute position reference | Host controller reference, PLC reference MC_MoveAbsolute |
|  | Axis deceleration to stop | Host controller reference, PLC reference MC_Stop |
|  | Synchronization cycle time | Host controller reference (DC-SYn-chro) |

    - Cyclic Synchronous Velocity (CSV) Mode

In CSV mode, the host controller generates the acceleration rate and deceleration rate for reaching the target torque, and sends the target torque to the servo drive cyclically. The servo drive operates by

following the target torque. When the CSV mode is enabled, the object dictionary control mode (6060h) must be set to 9.

Torque offset 60B2h

Torque feedforward  Torque feedforward

filter

2001.19h

(C01.18)

gain 2001.18h (C01.17)

Speed deviation 60B1h

Target velocity 60FFh + +	Speed regulator       + +

2001.02h (C01.01)

−

Torque filter

2001.04h	Torque limit	Torque regulator

(C01.03)

2001.03h (C01.02)

Speed calculation

Actual torque feedback 6077h

Motor

Encoder

Figure 4-40    Control in CSV mode

Basic configurations recommended in CSV mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 60FFh Target Velocity | - | Mandatory |
| - | 6064h Position actual value | Optional |
| - | 606Ch Velocity actual value | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in CSV mode:

The following table lists the meanings of each bit in the control word (6040h) in CSV mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 to 6 | Reserved for CSV mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 to 10 | Reserved for CSV mode | Not supported |
| 11 to 15 | Manufacturer-specific | Not supported |

Status word definition in CSV mode:

The following table lists the meanings of each bit in the status word (6041h) in CSV mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |

| Bit | Name | Description |
| --- | --- | --- |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Reserved for CSV mode | Not supported |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Target position following | This bit remains 1 all the time. |
| 13 | Reserved for CSV mode | Not supported |
| 14 to 15 | Manufacturer-specific | Not supported |

Parameters related to CSV mode:

Dictionary objects related to CSV mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6064h | 00 | Position feedback | RO | I32 | - | - |
| 606Ch | 00 | Actual speed | RO | I32 | - | - |
| 6077h | 00 | Actual torque | RO | I16 | - | 0 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 60B1h | 00 | Speed deviation | RW | I32 | -231 to (231-1) | 0 |

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 60B2h | 00 | Torque offset | RW | I16 | -4000 to 4000 | 0 |
| 60FFh | 00 | Target velocity | RW | I32 | -231 to (231-1) | 0 |

CSV mode example:

Start and operation processes in CSV mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 9 |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 or MC_Power |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Axis fault reset | Host controller reference or PLC reference MC_Reset |
| 60FFh | Speed reference | Host controller reference, PLC reference MC_ SyncMoveVelocity |
|  | Axis deceleration to stop | Host controller reference, PLC reference MC_Stop |
|  | Synchronization cycle time | Host controller reference (DC-SYn-chro) |

    - Cyclic Synchronous Torque (CST) Mode

In CST mode, the host controller generates the torque slope change rate for reaching the target torque, and sends the target torque to the servo drive cyclically. The servo drive operates by following the target torque. When the CST mode is enabled, the object dictionary control mode (6060h) must be set to 10.

Torque offset 60B2h

Target torque 6071h  + +

Speed loop as the source   	  of the torque reference

upon speed limit

Torque filter 2001.04h (C01.03)

Torque limit	Torque regulator

Actual torque feedback 6077h

Motor

Speed regulator

2001.02h (C01.01)

2001.03h (C01.02)

Max. speed limit    	  of the motor

Speed calculation

Encoder

Figure 4-41    Control in CST mode

Basic configurations recommended in CST mode:

| RPDO | TPDO | Remarks |
| --- | --- | --- |
| 6040h Control word | 6041h Status word | Mandatory |
| 6071h Target Torque | - | Mandatory |
| - | 6064h Position actual value | Optional |
| - | 606Ch Velocity actual value | Optional |
| - | 6077h Torque actual value | Optional |
| 6060h Modes of operation | 6061h Modes of operation display | Optional |

Control word settings in CST mode:

The following table lists the meanings of each bit in the control word (6040h) in CST mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Switch on | This bit must be set to 1 when the servo drive is enabled. |
| 1 | Enable voltage | This bit must be set to 1 when the servo drive is enabled. |
| 2 | Quick stop | This bit must be set to 1 when the servo drive is enabled. If this bit is set to 0, the servo drive stops quickly. |
| 3 | Operation enable | This bit must be set to 1 when the servo drive is enabled. |
| 4 to 6 | Reserved for CST mode | Not supported |
| 7 | Fault reset | The system performs fault reset once when this bit changes from 0 to 1. If system needs fault reset for multiple times, this bit must change from 0 to 1 for multiple times. When this bit is set to 1, other control references are inactive. |
| 8 to 10 | Reserved for CST mode | Not supported |
| 11 to 15 | Manufacturer-specific | Not supported |

Status word definition in CST mode:

The following table lists the meanings of each bit in the status word (6041h) in CST mode.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Ready to switch on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 1 | Switched on | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |

| Bit | Name | Description |
| --- | --- | --- |
| 2 | Operation enabled | 0: Inactive 1: Active (indicating that the servo drive has been enabled) |
| 3 | Servo drive fault | 0: No fault 1: Fault occurred |
| 4 | Voltage enabled | 0: Inactive 1: Active (indicating that the servo drive can be enabled) |
| 5 | Quick stop | 0: Active 1: Inactive |
| 6 | Switch on disabled | 0: Inactive 1: Active (indicating that the servo drive cannot be enabled) |
| 7 | Alarm | 0: No alarm 1: Alarm generated |
| 9 | Remote control | 0: Inactive 1: Active (indicating that the control word has taken effect) |
| 10 | Target torque | This bit remains 1 all the time. |
| 11 | Internal software position limit status | 0: Software position limit not reached 1: Software position limit reached |
| 12 | Target torque following | This bit remains 1 all the time. |
| 13 | Reserved for CST mode | Not supported |
| 14 to 15 | Manufacturer-specific | Not supported |

Parameters related to CST mode:

Dictionary objects related to CST mode:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6040h | 00 | Control word | RW | U16 | 0 to 65535 | 0 |
| 6041h | 00 | Status word | RO | U16 | - | - |
| 6060h | 00 | Operation mode | RW | I8 | 0 to 10 | 0 |
| 6061h | 00 | Mode display | RO | I8 | - | - |
| 6071h | 00 | Target torque | RW | I16 | -4000 to 4000 | 0 |
| 6072h | 00 | Max. torque | RW | U16 | 0 to 4000 | 3000 |
| 6074h | 00 | Torque reference | RO | I16 | - | 0 |

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 6077h | 00 | Actual torque | RO | I16 | - | 0 |
| 607Eh | 00 | Reference polarity | RW | U8 | 0 to 255 | 0 |
| 607Fh | 00 | Max. speed | RW | U32 | 0 to (232-1) | 104857600 |
| 60B2h | 00 | Torque offset | RW | I16 | -4000 to 4000 | 0 |
| 60E0h | 00 | Positive torque limit | RW | U16 | 0 to 4000 | 3000 |
| 60E1h | 00 | Negative torque limit | RW | U16 | 0 to 4000 | 3000 |

CST mode example:

Start and operation processes in CST mode:

| Address | Name | Value |
| --- | --- | --- |
| 6060h | Control mode | 10 |
| 6071h 607Fh | Torque/Speed reference | Torque reference/speed limit |
| 6040h (control word) | Enable | Random number → 6 → 7 → 15 or MC_Power |
|  | Alarm clearance | Random number → 128 (rising edge active, if the alarm can be cleared) |
|  | Axis fault reset | Host controller reference or PLC reference MC_Reset |
|  | Synchronization cycle time | Host controller reference (DC-SYn-chro) |

    - Touch Probe Function

The touch probe function is used to latch the position. It can latch the position value (reference unit) when a DI signal or Z signal of the motor changes. The A6-EC series servo drive offers two touch probes to save position values corresponding to the rising edge and falling edge of each touch probe signal, which means a total of four position values can be latched simultaneously.

NOTICE

y No specific DI logic is required when a DI is used to trigger the touch probe signal. y You can set the filter window for the touch probe signal in 2010.2Bh (C10.2A) when a DI is used to trigger the touch probe signal. y The touch probe function can be set only for the DI4 and DI5 high speed DIs.

Related parameters:

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 2004h | 0D | DI4 function | RW | U16 | 0 to 65535 | 30 |
| 2004h | 11 | DI5 function | RW | U16 | 0 to 65535 | 31 |
| 60B8h | 0 | Touch Probe Function | RW | U16 | 0 to 65535 | 0 |
| Bit description for 60B8h: Bit Name Description 0: Disabled 0 Touch probe 1 function selection 1: Enabled 0: Single trigger mode (Latches the position at the first - Touch probe 1 trigger mode trigger event.) 1: Continuous trigger 0: DI signal - Touch probe 1 trigger signal selection 1: Z signal - Reserved 0: Switch off latching at positive edge - Touch probe 1 positive edge 1: Enable latching at positive edge 0: Switch off latching at negative edge - Touch probe 1 negative edge 1: Enable latching at negative edge - to 7 Reserved 0: Disabled - Touch probe 2 function selection 1: Enabled 0: Single trigger mode (Latches the position at the first - Touch probe 2 trigger mode trigger event.) 1: Continuous trigger 0: DI signal - Touch probe 2 trigger signal selection 1: Z signal - Reserved 0: Switch off latching at positive edge - Touch probe 2 positive edge 1: Enable latching at positive edge 0: Switch off latching at negative edge - Touch probe 2 negative edge 1: Enable latching at negative edge - to 15 Reserved |  |  |  |  |  |  |

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Touch probe 1 function selection | 0: Disabled 1: Enabled |
| 1 | Touch probe 1 positive edge value | 0: No positive edge value latched 1: Positive edge value latched |
| 2 | Touch probe 1 negative edge value | 0: No positive edge value latched 1: Positive edge value latched |
| 3 to 7 | Reserved |  |
| 8 | Touch probe 2 function selection | 0: Disabled 1: Enabled |
| 9 | Touch probe 2 positive edge value | 0: No positive edge value latched 1: Positive edge value latched |
| 10 | Touch probe 2 negative edge value | 0: No positive edge value latched 1: Positive edge value latched |
| 11 to 15 | Reserved |  |

| Index | Sub- index | Name | Access | Data Type | Value Range | Default |
| --- | --- | --- | --- | --- | --- | --- |
| 60B9h | 0 | Touch probe status | RO | U16 | - | 0 |
| Bit description for 60B9h: |  |  |  |  |  |  |
| 60BAh | 0 | Touch probe 1 positive edge | RO | I32 | - | 0 |
| 60BBh | 0 | Touch probe 1 negative edge | RO | I32 | - | 0 |
| 60BCh | 0 | Touch probe 2 positive edge | RO | I32 | - | 0 |
| 60BDh | 0 | Touch probe 2 negative edge | RO | I32 | - | 0 |
| 60D5h | 0 | Touch probe 1 positive edge counter | RO | U16 | - | 0 |
| 60D6h | 0 | Touch probe 1 negative edge counter | RO | U16 | - | 0 |
| 60D7h | 0 | Touch probe 2 positive edge counter | RO | U16 | - | 0 |
| 60D8h | 0 | Touch probe 2 negative edge counter | RO | U16 | - | 0 |

4.1.10 Software Position Limit

Description:

In conventional drives, the position limit is defined by external sensor signals connected to the DI of the

servo drive.

FunIN.6

FunIN.7

DI

Servo drive

DI

Figure 4-42    Installation of the limit switch

Table 4-2    Comparison between the hardware position limit and software position limit

| Hardware Position Limit | Software Position Limit |
| --- | --- |
| Restricted to linear motion and single-turn rotational motion. | Applicable to both linear motion and rotational motion. |
| Requires an external mechanical limit switch. | Removes the need for hardware wiring, preventing malfunction due to poor cable contact. |
| Suffered from the risk of mechanical slip. | Prevents malfunction due to mechanical slip through internal position comparison. |
| Unable to sense or detect an overtravel fault after power-off. |  |

The software position limit works by comparing the limit value with the internal feedback value. If the latter exceeds the former, a warning will be reported and the servo drive stops. This function applies to both the absolute position mode and the incremental position mode. In incremental position mode, set 2006.08h (C06.07) to 2, which means the servo drive performs homing to find the mechanical home after power-on, and then enables the software position limit.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006.08h | C06.07 | Mechanical limit position | 0: Disabled 1: Enabled 2: Enabled after homing | 0 to 2 | 0 | - | U16 | During operation | Immediately |

| Index | Name | Access | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 607D.01h | Min. software position limit | RW | RPDO | I32 | Reference unit | -231 to (231-1) | -231 | During operation | Immediately |
| 607D.02h | Max. software position limit | RW | RPDO | I32 | Reference unit | -231 to (231-1) | 231-1 | During operation | Immediately |

CAUTION

y Ensure the value of 607D.01 is lower than or equal to 607D.02. If 607D.01 is set to a value higher than 607D.02, the servo drive prompts Er841 (software position limit setting error). y In absolute rotation mode or single-turn mode, ensure that 607D.01 and 607D.02 are within the mechanical position limit. Otherwise, the servo drive reports Er841. y Ensure that the value of 607C (home offset) is within the software position limit. Otherwise, the servo drive reports Er843 (home offset outside the software position limit).

Chapter 5	Absolute System

NOTICE

y The absolute encoder must carry a battery box.

5.1   Absolute System Setting

Overview

The absolute encoder, which carries a resolution of 131072 (217) PPR, detects the motor position within one revolution and counts the number of revolutions, with 16-bit multi-turn data saved.

The absolute encoder system works in the position control, speed control, and torque control modes. When the servo drive is powered off, the encoder battery serves as the power supply to enable the encoder to back up data. The servo drive therefore can calculate the absolute mechanical position through the encoder after power-on, removing the need for homing.

When A6-EC series servo drive matches the absolute encoder, set 2000.08h (C00.07) to (absolute system selection) based on actual conditions. Er208 (encoder battery fault) will be reported when the battery is connected for the first time. In this case, set 2031.11h (F31.10) to reset the encoder fault, and then perform homing.

NOTICE

y When the value of 2000.02h (C00.01) (rotation direction) or 2031.11h (F31.10) (absolute encoder reset selection), or the electronic gear ratio is changed, the mechanical position will change abruptly, requiring a homing operation. y After homing is done, the deviation between the mechanical absolute position and that saved in the encoder will be calculated automatically and saved in the EEPROM of the servo drive.

Absolute system setting

Set 2000.08h (C00.07) to select the absolute position mode.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08h | C00.07 | Absolute mode | 0: Incremental position mode 1: Absolute position linear mode 2: Absolute position linear infinite mode 3: Absolute position single-turn mode 4: Absolute position rotation mode 5: Absolute mechanical single-turn mode (operating direction selectable) | 0 to 5 | 0 | - | U16 | At stop | Upon re- power-on |

Encoder feedback data

The feedback data of an absolute encoder includes the number of revolutions and the motor position within one revolution. In incremental position mode, the number of revolutions will not be counted.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Dh | U40.1C | Encoder single-turn data | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |
| 1Fh | U40.1E | Encoder multi-turn position data | - | 0 to 65535 | 0 | Rev | U16 | - | Immediately |
| 21h | U40.20 | Encoder multi-turn data (low 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |
| 23h | U40.22 | Encoder multi-turn data (high 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |

5.2    Absolute Position Linear Mode

This mode applies to scenarios where the axis travel range is fixed without multi-turn data overflow.

Limit

Limit

The value range of 2000.08h=1 (C00.07) (encoder multi-turn data) in absolute position linear infinite mode is 0 to 65535. If the number of forward or reverse revolutions is greater than 65535, ErA01 (encoder multi- turn counting overflow) will occur. In this case, set 2031.11h (F31.10) to 4 to reset the multi-turn data, and then perform homing again. In special occasions, you can set 2000.08h (C00.07) to 2 in absolute position linear infinite mode to prevent the encoder overflow alarm.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17h | U40.16 | Absolute position feedback (reference unit) | - | -231 to (231-1) | 0 | Unit in application | I32 | - | Immediately |
| 25h | U40.24 | Absolute position feedback (encoder unit) (low 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |
| 27h | U40.26 | Absolute position feedback (encoder unit) (high 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |

| Index | Sub- index | Name | Access | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6063h | 0 | Position feedback | RO | TPDO | I32 | Encoder unit | - | - | - | - |
| 6064h | 0 | Position feedback | RO | TPDO | I32 | Reference unit | - | - | - | - |

5.3   Absolute Position Rotation Mode

This mode applies in cases where the load travel range is unlimited and the number of unidirectional revolutions is less than 32767, as shown in the following figure.

Figure 5-1    Rotating load

The single-turn position range of the rotating load is 0 to (RM - 1) (RM: Encoder pulses per load revolution). When the gear ratio is 1:1, the variation law of the target position and the single-turn position of the

rotating load during forward operation is as follows.

Target position

0

Rotation

Single﹣turn position

of the rotating load

0

Rotation

The variation law of the target position and the single-turn position of the rotating load during reverse operation is as follows.

Target position

0

Rotation

Single﹣turn position

of the rotating load

0

Rotation

When the motor works in the absolute position rotation mode while the servo drive works in HM mode, the home offset range is 0 to (RM  - 1).

NOTICE

y The servo drive calculates the upper limit of the mechanical absolute position using 2010.1Bh (C10.1A) and 2010.1Dh (C10.1C) first. If 2010.1Bh (C10.1A) and 2010.1Dh (C10.1C) are both set to 0, the servo drive employs the electronic gear ratio 2010.19h (C10.18) and 2010.1Ah (C10.19) for calculation.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19h | C10.18 | Numerator of electronic gear ratio in rotation mode | - | 1 to 65535 | 1 | - | U16 | At stop | Immediately |
| 1Ah | C10.19 | Denominator of electronic gear ratio in rotation mode | - | 1 to 65535 | 1 | - | U16 | At stop | Immediately |
| 1Bh | C10.1A | Upper limit of mechanical absolute position in rotation mode (low 32 bits) | - | 0 to (232-1) | 0 | P | U32 | At stop | Immediately |
| 1Dh | C10.1C | Upper limit of mechanical absolute position in rotation mode (high 32 bits) | - | 0 to (232-1) | 0 | P | U32 | At stop | Immediately |
| 29h | U40.28 | Position feedback in rotation mode (reference unit) (low 32 bits) | - | -231 to (231-1) | 0 | Unit in application | I32 | - | Immediately |
| 2Bh | U40.2A | Position feedback in rotation mode (encoder unit) (low 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |
| 2Dh | U40.2C | Position feedback in rotation mode (encoder unit) (high 32 bits) | - | -231 to (231-1) | 0 | P | I32 | - | Immediately |

| Index | Sub- index | Name | Access | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6063h | 0 | Position feedback | RO | TPDO | I32 | Encoder unit | - | - | - | - |
| 6064h | 0 | Position feedback | RO | TPDO | I32 | Reference unit | - | - | - | - |

5.4    Absolute Position Single-turn Mode

In this mode, the absolute encoder needs no battery and does not record the number of motor revolutions. The input range for the target position of the EtherCAT host controller is -231 to (231 - 1), with no need for modulus operation on the encoder resolution. The initial position for feedback upon each power-on is the single-turn absolute position. For a 17-bit encoder, the value range of 6064 is 0 to (217 - 1) for each power- on. If the value of C00.07 remains 3 (absolute position single-turn mode), after homing (homing mode of the servo drive, that is, 6060 = 6), the servo drive saves the absolute position of the encoder after homing and the value of 607C (home offset). Upon power-up again, the value of 6064 (current position feedback) will be calculated based on the coordinate system after the last homing operation, eliminating the need for re-homing.

If 60E6h (absolute position single-turn mode) is inactive, after successful homing, the value of 6064 (position feedback) is equal to that of 607C (home offset).

Related parameters:

| Index | Sub- index | Name | Access | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6063h | 0 | Position feedback | RO | TPDO | I32 | Encoder unit | - | - | - | - |
| 6064h | 0 | Position feedback | RO | TPDO | I32 | Reference unit | - | - | - | - |

5.5   Precautions for Use of the Absolute system Battery Box

Er208 (encoder battery fault) will be reported when the battery is connected for the first time. In this case, set 2031.11h (F31.10) to reset the encoder fault, power on again, and then perform the absolute position device operation.

When the battery voltage detected is lower than 3.0 V, ALF90 (encoder battery warning) will be reported. In this case, replace the battery based on the following steps:

① Power on the servo drive and make it stay in the non-operational state.

② Replace the battery.

③ After ALF90 (encoder battery warning) is automatically cleared, if no other warning/fault occurs, you can continue operating the servo drive.

If you replace the battery after power-off, Er208 (encoder battery fault) will be reported, with the multi-turn data changed abruptly. In this case, set 2031.11h (F31.10) to 4 to reset the fault, and then perform homing again.

Ensure the motor speed does not exceed 6000 RPM after the servo drive is powered off. This is to enable the encoder to save the position data accurately.

Keep the battery in environments within the required ambient temperature and ensure the battery is in reliable contact with sufficient power reserved. Failure to comply may result in encoder data loss.

NOTICE

y The absolute position saved by the encoder changes abruptly after multi-turn data reset. In this case, perform mechanical homing.

Related parameters:

| Index | Parameter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11h | F31.10 | Encoder data reset | - | 0 to 31 | 0 | - | U16 | At stop | Immediately |

System Commissioning ▕

Chapter 6	System Commissioning

6.1    Commissioning Tool

The A6-EC series servo drive can be commissioned by the operating panel which consists of an LED display area and five buttons.

A6

Display and operation area

    - Buttons

| Button |  | Description |
| --- | --- | --- |
|  | MODE | Press: y Switch/return to menus of different levels Press and hold: y Switch the group number quickly in level-2 menus |
|  | UP | Press: y Switch the state display quickly in level-1 menus y Increase the value |
|  | DOWN | Press: y Switch the state display quickly in level-1 menus y Decrease the value |
|  | SHIFT | Press: y Move the cursor to the left Press and hold: y Enter the JOG mode quickly in level-1 menus y Page up or down when the content is displayed on multiple pages |
|  | SET | Press: y Switch to the lower-level menu y Execute commands such as storing parameter setpoints |

    - Display

When the servo drive is running, the servo drive status, parameters, faults, and monitored values are displayed in the LED display area.

Status display: Display current servo drive status, such as servo ready or servo running.

Parameter display: Display parameters and their setpoints.

Fault display: Display faults and alarms that occurred on the servo drive.

Monitored value display: Display values of running parameters of the servo drive.

NOTICE

y After power-on, "Init" is displayed in the LED display area and then the system enters the level-1 menu status display mode. y In the status display mode, select the parameter to be monitored. When the motor rotates, the display area automatically switches to monitored value display. After the motor stops, the display area automatically returns to status display. y In the parameter display mode, after you select the parameter to be monitored, the system switches to monitored value display. y Once a fault occurs, the LED display area switches to fault display automatically, with all the LEDs blinking. to stop the LEDs from blinking, and then to switch to parameter display.

Display menu

y   Level-1 menu: Status display

①

②   ③   ④	⑤

① Display of the model with brake

② Status of the network port

③ Communication status

④ Running mode

⑤ Servo status

Press  and  to   switch   among   different   display   modes. Press               to enter a level-2 menu.

y   Level-2 menu: Display the parameter group number in hexadecimal

| Parameter group number | C: Function parameter R: System parameter F: Operation parameter U: Monitoring parameter |
| --- | --- |

Press and  to switch the group number. Press     to enter a level-3 menu.

Press 	 to return to the level-1 menu.

y   Level-3 menu: Offset within the parameter group in hexadecimal

Offset within the parameter group

Press  to   enter   a   level-4   menu. Press             to return to a level-2 menu.

y   Level-4 menu: Parameter setpoints in decimal

Press 	 to increase or decrease the value.

Status display

Press  to confirm the setting. Then, the system .

Press 	 to return to a level-3 menu.

| LED Display | Meaning | Description |  |
| --- | --- | --- | --- |
| ① ② ③ ④ ⑤ | ① Display of the model with brake | On: Model with brake Off: Model without brake |  |
|  | ② Status of the network port | No display: No network port is connected. " ▔ " displayed: The OUT network port is connected. " ▁ " displayed: The IN network port is connected. |  |
|  | ③ Communication status | 0: No meaning 1: Initializing 2: Pre-operation 4: Operating safely 8: Running |  |
|  | ④ Running mode | 1: Profile position mode 3: Profile velocity mode 4: Profile torque mode 6: Homing mode 8: Cyclic synchronous position mode 9: Cyclic synchronous velocity mode A: Cyclic synchronous torque mode |  |
|  | ⑤ Servo status | nr: Servo not ready rd: Servo ready rn: Servo running |  |

Parameter display

| Category | LED Display |
| --- | --- |
| Signed number with 4 digits and below: Display on one page (5 digits) When the rightmost "●" is turned on, the high bit " ▁ " indicate a negative sign. |  |
| Unsigned number with 5 digits and below: Display on one page (5 digits) |  |
| Signed number with more than 4 digits: Such numbers are displayed from low to high bits in multiple pages (5 digits per page). For a negative value, when the rightmost "●" is turned on, the high bit " ▁ " indicate a negative sign. Such numbers are displayed in the format of "number of current page + values on current page". To switch to the next page, press and for more than 2 seconds. Page 1 Page 2 Page 3 Four low bits Four middle bits Four high bits Indicate the low-bit page Indicate the middle-bit page Indicate the high-bit page |  |
| Unsigned number with more than 5 digits: Such numbers are displayed from low to high bits in multiple pages (5 digits per page). Such numbers are displayed in the format of "number of current page + values on current page". To switch to the next page, press and for more than 2 seconds. Page 1 Page 2 Page 3 Four low bits Four middle bits Four high bits Indicate the low-bit page Indicate the middle-bit page Indicate the high-bit page |  |

Fault display

y The panel displays the current or history faults and alarm codes. For analysis and troubleshooting of faults and alarms, see the section "Troubleshooting".

y When a single fault or an alarm occurs, the panel displays the fault or alarm code. When multiple faults or alarms occur, the panel displays the fault code of the highest level.

6.2    Commissioning Process

Start

Checklist before running

y   Mechanical and wiring inspection and safety inspection

Power on the servo drive

y Connect the power supply of the main circuit and control circuit

Jogging

y   Jog through the panel

Set parameters

y   Set common parameters

y   Set relevant parameters in each control mode

Servo running

y   Run the servo drive at a low speed for the first time

y   Set relevant parameters to achieve the required effect

y   Commission the servo drive

Stop the servo drive

y    Turn off the S-ON signal

y   Stop the servo drive upon a fault

y   Stop the servo drive if the limit switch is reached

y   Perform emergency stop

Finish

6.3    Commissioning Procedure

CAUTION

y Confirm that the site is equipped with conditions suitable for the safe operation of the servo drive (no interference, no foreign objects, and no dangerous goods).

y   Confirm that the power input terminals are connected correctly and firmly.

y  Confirm that the wiring (U/V/W) between the servo drive and the motor is correct and firm.

y Confirm that the control signal cables of the servo drive are connected correctly. External signal cables for brake and overtravel protection are connected properly.

y Confirm that the servo motor and its axes are installed and mechanically connected reliably.

y   Confirm that the servo drive and motor have been grounded reliably.

    - Power on the servo drive

NOTICE

y Single-phase input: The power terminals are L1 and L2. y Three-phase input: The power terminals are R, S, and T for the main circuit or L1C and L2C for the control circuit.

After the input power supply is connected:

y   The LED panel  ,  which  means  that  the  servo  drive  is  ready  to  run  and  is  waiting for the S-ON signal from the host controller.

y   If the LED panel keeps  or   other   faults,   troubleshoot   the   problem   according   to the section "Troubleshooting".

    - Jogging

Use the jogging function for trial run of the servo motor and servo drive to check whether the servo motor rotates properly without abnormal vibration or noise generated during rotation.

During the jog, use F30.03 to set the acceleration/deceleration time constant for the velocity/position reference.

CAUTION

y To use the jogging function, deactivate the S-ON signal. Otherwise, this function cannot be used.

Procedure for setting the jogging function:

Start

Power on the servo drive

Set the parameter F30-00 on the panel

Press and  in the level-1 menu

Press        to confirm the setting

Set the jog speed

Press 	 to increase or decrease the value

Confirm the motor speed

Press  to confirm the setting

JOG-S is displayed

Press

Finish

Enable forward/reverse motor rotation

Press

CCW	CW

NOTICE

y Press to increase or decrease the motor speed for the jog running. If the system exits jog running, the motor speed restores to the initial value. y Press to make the servo motor rotate in the forward or reverse direction. After the button is released, the servo motor stops running immediately.

    - Set parameters

Select the rotation direction

Set C00.01 (2000.02h, rotation direction) to change the direction of motor rotation without modifying the input command polarity.

Set the brake

The brake is used to prevent the servo motor shaft from moving and lock the motor position when the servo drive is not running. This is to keep the mechanical load from moving due to gravity or external force.

NOTICE

y The built-in brake is a special non-energized mechanism designed for position-lock in the stop state. Do not use the built-in brake for any other purposes, such as braking. y The brake coil has no polarity. y Switch off the S-ON signal after the servo motor stops. y When the motor with a built-in brake runs, the brake may generate a click sound, which does not affect its function. y When brake coils are energized (the brake is released), flux leakage may occur on the shaft end. Pay special attention when using magnetic sensors near the motor.

For the servo motor with a brake, assign function 3 (brake output) to a DO terminal (DO3 by default) of the servo drive and set valid logic for the DO terminal.

The operating time sequences of the brake are different between normal state and fault state of the servo

drive.

The brake time sequence in the normal state changes with the motor states: static and rotating.

y   Static: The motor speed is lower than 30 rpm.

y   Rotating: The motor speed is equal to or greater than 30 rpm.

Brake time sequence in the motor static state:

y If the servo enabling (S-ON) signal changes from ON to OFF, and the present motor speed is lower than 30 rpm, the servo drive acts according to the brake time sequence in the motor static state.

CAUTION

y After the brake (BK) output signal changes from OFF to ON, do not input a position/speed/torque reference within the time defined by C05.13. Otherwise, reference loss or running error may occur.

NOTICE

y When the motor is used to drive a vertical axis, the mechanical motion part may move slightly due to the gravity or external force. In the motor static state, if the S-ON signal becomes OFF, the brake (BK) output signal becomes OFF immediately. However, within the time defined by C05.10, the motor is still energized to prevent the mechanical motion part from moving due to the gravity or external force.

ON

ON

S-ON

OFF

OFF

ON

Motor energized

OFF

OFF

C05-10

ON

Brake output (BK)

OFF

OFF

ON (brake released)

Brake contactor

OFF (brake applied)

OFF

Position/speed/torque reference

C05-13

Stop at zero speed

OFF

OFF

Motor speed

Figure 6-1    Brake time sequence in the motor static state

Brake time sequence in the motor rotating state:

y If the S-ON signal changes from ON to OFF, and the present motor speed is equal to or greater than 30 rpm, the servo drive acts according to the brake time sequence in the motor rotating state.

CAUTION

y If the S-ON signal changes from OFF to ON, do not input a position/speed/torque reference within the time defined by C05.13. Otherwise, reference loss or running error may occur.

NOTICE

y If the S-ON signal becomes OFF during motor rotating, the servo motor enters the "Stop according to ramp in 6085h" state, but the brake (BK) output signal becomes OFF only after one of the following conditions is met: - The motor has decelerated to the value defined by C05.11, but the time defined by C05.12 is not reached. - The time defined by C05.12 has been reached, but the motor speed is still higher than the value defined by C05.11. y After the brake (BK) output signal changes from ON to OFF, the motor remains energized within the time defined in C05.10 to prevent the mechanical motion part from moving due to the gravity or external force.

ON

ON

S-ON

OFF

OFF

ON

Motor energized

OFF

OFF

C05-10

ON

Brake output (BK)

OFF

OFF

ON (brake released)

Brake contactor

OFF (brake applied)

OFF

Position/speed/torque reference

C05-13

Stop at zero speed

OFF

OFF

Motor speed

Figure 6-2    Brake time sequence in the motor rotating state

Set the brake

When the torque and speed directions of the motor are opposite, the energy is transferred from the motor to the drive, increasing the bus voltage. When the voltage is increased to the braking point, the energy can be consumed only by the braking resistor. In this case, the braking energy must be consumed according to the braking requirements. Otherwise, the servo drive may be damaged.

CAUTION

y Set the power (C00.11) and resistance (C00.12) for the external braking resistor properly. Otherwise, the braking function may be affected. y When an external braking resistor is used, check whether its resistance is above the minimum allowable resistance. y In a natural environment, the temperature of the resistor will rise above 120°C (under continuous braking) when the average power of the braking resistor is below its rated power. For safety consideration, use forced cooling to cool down the braking resistor or use a braking resistor with a thermal switch. Consult the manufacturer about load characteristics of the braking resistor.

Set the heat dissipation coefficient (C00.13) based on the heat dissipation condition of the external braking

resistor.

    - Servo running

Switch on the S-ON signal. When the servo drive is ready to run, the LED panel .             If there is no command input at this moment, the servo motor does not rotate and stays locked. After a command is input, the servo motor starts rotating.

NOTICE

Operation of the servo drive y During initial operation, set a proper command to make the motor run at low speed and check whether the motor rotates properly. y Check whether the motor rotates in the correct direction. If the direction of rotation is opposite to the expected direction, check the reference signal and reference direction signal. y If the motor rotates in the correct direction, you can view the actual motor speed in U40.01 (2040.02h) and the average load factor in U40.07 (2040.08h) through the drive panel. y After checking preceding conditions, adjust related parameters to make the motor operate as desired. y Adjust the servo drive parameters according to the section "Gain Tuning".

    - Stop the servo drive

Five type of stop modes are available for the servo drive: coast to stop, stop at zero speed, ramp to stop, stop at emergency-stop torque, and dynamic braking (DB) stop, along with three kinds of stop status:

de-energized, position lock, and DB. After brake output is enabled, the servo drive selects a stop mode.

Table 6-1    Comparison of the stop modes

| Stop Mode | Description | Feature |
| --- | --- | --- |
| Coast to stop | The servo motor is de-energized and coasts to 0 RPM. The deceleration time is affected by the mechanical inertia and mechanical friction. | This mode features smooth and slow deceleration with small mechanical shock. |
| Stop at zero speed | The servo motor decelerates to 0 RPM immediately and stops. | This mode features quick deceleration with obvious mechanical shock. |
| Ramp to stop | The motor decelerates to 0 RPM smoothly upon position/speed/torque reference input. | This mode features smooth and controllable deceleration with small mechanical shock. |
| Stop at emergency-stop torque | The servo drive outputs a reverse braking torque to stop the motor. | This mode features quick deceleration with obvious mechanical shock. |
| Dynamic braking | The servo motor is in the dynamic braking status. | This mode features quick deceleration with obvious mechanical shock. |

Table 6-2    Comparison of the stop statuses

| Stop Status | Description |
| --- | --- |
| De-energized | The motor is de-energized and the motor shaft can rotate freely after the motor stops rotating. |
| Position lock | The motor shaft is locked and cannot rotate freely after the motor stops rotating. |
| Dynamic braking | The motor is de-energized and the motor shaft cannot rotate freely after the motor stops rotating. |

Table 6-3    Comparison of the stop modes

| Stop Mode | Stop Mode (Model Without Brake) | Stop Mode (Model with Brake) | Description |
| --- | --- | --- | --- |
| Stop at S-ON OFF | 605Ch | Ramp to quick stop as defined by 6085h, keeping dynamic braking status | Deactivate the S-ON signal through communication to make the servo drive stop according to the stop mode at S-ON OFF. |

| Stop Mode | Stop Mode (Model Without Brake) | Stop Mode (Model with Brake) | Description |
| --- | --- | --- | --- |
| Stop at fault | No.1 fault: C05.03 | No. 1 fault: Dynamic braking stop, keeping dynamic braking status | The stop mode varies with the fault type. For fault classification, see sections related to faults. |
|  | No.2 fault: 605Eh | Ramp to quick stop as defined by 6085h, keeping dynamic braking status | - |
| Stop at overtravel | C05.02 | Ramp to quick stop as defined by 6085h, keeping position lock status | When a mechanical motion part moves beyond the range of safe movement, the limit switch outputs a level change to force the servo motor to stop. |
| Emergency stop | 605Ah | 605Ah < 4: Ramp to quick stop as defined by 6085h, keeping dynamic braking status | Use the DI function 4 for emergency stop (Note: The stop mode is specified by 605Ah. The final stop state is S-ON OFF.) |
| Quick stop | 605Ah | 605Ah < 4: Ramp to quick stop as defined by 6085h, keeping dynamic braking status | Quick stop applies when bit 2 (Quick stop) of the control word 6040h is set to 0 during operation of the servo drive. |
| Halt | 605Dh |  | The halt function applies when bit 8 of the control word 6040h is set to 1 (Halt) during operation of the servo drive. The halt mode is defined by object dictionary 605Dh. |

Chapter 7	Gain Tuning

7.1   Overview

The servo drive must drive the motor as quick and accurate as possible to follow the commands from the host controller or internal setting. A proper gain tuning is required to make the motor actions more closely follow the commands and to maximize the performance of the servo system.

Gain: Low

Gain: High

Gain: High + Feedforward

Position loop gainѪ40.0 rad/s	Position loop gainѪ80.0 rad/s	Position loop gainѪ80.0 rad/s Speed loop gainѪ25.0 Hz	Speed loop gainѪ50.0Hz	Speed loop gainѪ50.0 Hz

Speed loop integral time constantѪ50.0 ms	Speed loop integral time constantѪ25.0 ms	Speed loop integral time constantѪ25.0 ms Speed feedforwardѪ0	Speed feedforwardѪ0	Speed feedforwardѪ50.0%

Inertia ratioѪ30	Inertia ratioѪ30	Inertia ratioѪ30

CAUTION

y Before gain tuning, perform a trial run through jogging to ensure the motor operates properly.

7.2    Inertia Auto-tuning

The load inertia ratio is the ratio of the total moment of inertia of motor load to the moment of inertia of the motor. The load inertia ratio is a critical parameter of the servo system. A correct load inertia ratio facilitates commissioning.

You can set the C00.06 (load inertia ratio) manually based on the weight and composition of different mechanical parts, but the operation is very tedious. It is increasingly difficult to get the correct solution for the complex mechanical composition. However, it can also be automatically auto-tuned by F30.10 (inertia auto-tuning function) of the servo drive.

During auto-tuning, the drive will drive the servo motor to run in the forward or reverse direction multiple times, so as to obtain the load inertia ratio.

NOTICE

Inertia auto-tuning may fail in the following conditions: y The load mechanical system is poor, with low stiffness and vibration during localization. y The motor operation range is too small, less than 0.5 turns. y The load torque changes dramatically. y The motor acceleration rate is less than 3000 rpm/s. y The actual maximum speed of the motor is less than 150 rpm.

Inertia auto-tuning process:

Start

Power on the servo drive

Set the parameter F30-10 on the panel

Press        to confirm the

Press

to increase

or decrease the value

Set the number of auto-tuning turns

setting

Confirm the number of auto-tuning turns

Press        to confirm the setting

Display the inertia value

Press

Perform auto-tuning on auto/manual gains

Press

End

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C00.06 | Load inertia ratio | 0 to 12000 | 100 | % | - | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C07.00 | Offline inertia auto-tuning mode setting | 0 to 785 | 769 | - | Related modes during the auto-tuning are set. | At stop | Immediately |
| C07.01 | Offline inertia auto-tuning speed reference | 50 to 8000 | 500 | rpm | The speed reference for auto-tuning is set. | At stop | Immediately |
| C07.02 | Acceleration/ Deceleration time for offline inertia auto- tuning | 0 to 65535 | 100 | ms | The acceleration/ deceleration time for auto-tuning is set. | At stop | Immediately |
| C07.03 | Offline inertia auto- tuning target torque | 1 to 1500 | 150 | 0.1% | A larger target torque leads to shorter auto- tuning acceleration/ deceleration time. | At stop | Immediately |
| C07.04 | Offline inertia auto-tuning revolutions | 10 to 65535 | 200 | 0.01r | The number of auto- tuning revolutions should be within the mechanical motion range. | At stop | Immediately |

7.3    Basic Gain Tuning

The servo system consists of three feedback loops, which are the position loop, speed loop, and current loop. The basic control diagram is shown in the following figure.

Position loop control

Speed calculation

Drive

Reference input

+

−

Position feedback

+

−

Speed feedback

Speed loop control

+

−

Current feedback

Motor

Encoder

Current loop control

CAUTION

y The responsiveness of the inner loop must be higher than that of the outer loop. Otherwise, the responsiveness may be poor or vibrations may occur.

The default current loop gain of the servo drive ensures sufficient responsiveness, removing the need for further tuning. You only need to adjust the position loop gain, speed loop gain, and other auxiliary gains. Therefore, to ensure system stability during gain tuning in position control mode, the position loop gain must be increased together with the speed loop gain, and the responsiveness of the former must be lower than the latter.

The drive provides three types of gain auto-tuning modes: 0: Manual tuning

1: Standard tuning by stiffness level 2: Positioning mode

When the automatic gain tuning does not achieve the expected effect, you can manually fine-tune the gain. The effect can be optimized by more detailed tuning.

The following table lists the basic gain parameter tuning methods.

| No. | Parameter | Name | Description |
| --- | --- | --- | --- |
| 1 | C01.00 | Position loop gain | Function: Determines the responsiveness of the position loop of the servo unit. Increasing the position loop gain improves the responsiveness and shortens the positioning time. In general, the position loop gain cannot exceed the range of the certain vibration count of the mechanical system. Increase the value of C01﹣00 Position reference Increase the value of C01﹣01 Actual speed Tuning method: To ensure system stability, the gain frequency of the speed loop must be 3 to 5 times that of the position loop. |
| 2 | C01.01 | Speed loop gain | Function: Determines the speed loop responsiveness. Too low responsiveness of the speed loop may be a delay factor of the outer position loop, so overshoot or a variable speed reference occurs. Therefore, within the non-vibration range of the mechanical system, increasing the setpoint stabilizes the speed and improves the responsiveness of the servo system. Speed reference Increase the value of C01﹣01 Actual speed Tuning method: In the vibration range without noise, increasing the parameter value will shorten the positioning time and lead to higher speed stability and following performance. |

| No. | Parameter | Name | Description |
| --- | --- | --- | --- |
| 3 | C01.02 | Speed loop integral time constant | Function: To respond to even minor inputs, the speed loop contains an integral element. Since this integral element acts as a delay element for the servo system, when the time parameter is set too large, it will cause overshoot, or extend the positioning time, making the response worse. Speed reference Lower C01﹣02 Actual speed Tuning method: Reducing the setpoint can enhance the integral effect and shorten the positioning time, but setting the value too low can easily cause mechanical vibration. If the value is set too high, the speed loop deviation will never return to zero. |
| 4 | C01.03 | Torque reference filter cutoff frequency | Function: This parameter applies a low-pass filter to the torque reference, where the setpoint is the cutoff frequency of the low- pass filter. The smaller the setpoint, the better the filtering effect. Setting the value too low can cause excessive delay in the speed loop, thereby reducing the speed loop bandwidth. When mechanical vibrations occur, adjusting the following torque reference filter time parameters may potentially eliminate the vibrations. Speed reference Lower C01﹣03 Actual speed Tuning method: Ensure that the cutoff frequency of the torque reference low-pass filter is more than four times the highest follow- up frequency of the speed loop. |

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C00.04 | Auto-tuning mode | 0 to 2 | 1 | - | 0: Manual mode 1: Standard mode 2: Positioning mode | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C00.05 | Stiffness level | 1 to 31 | 12 | - | Increasing the stiffness level improves the responsiveness, but a too high level can cause oscillation. | During operation | Immediately |
| C01.00 | 1st position loop gain | 0 to 20000 | 400 | 0.1rad/ s | Increasing the setpoint improves the responsiveness and shortens the positioning time. | During operation | Immediately |
| C01.01 | 1st speed loop gain | 1 to 20000 | 250 | 0.1Hz | Increasing the setpoint improves the speed follow-up responsiveness of the servo system. | During operation | Immediately |
| C01.02 | 1st speed loop integral time parameter | 1 to 51200 | 3184 | 0.01ms | Reducing the setpoint can enhance the integral effect and shorten the positioning time. | During operation | Immediately |
| C01.03 | 1st torque reference filter cutoff frequency | 5 to 16000 | 200 | Hz | Reducing the setpoint improves the filtering effect but increases the delay. | During operation | Immediately |

7.4   Pseudo derivative feedback and feedforward control

In position and speed modes, the pseudo derivative feedback and feedforward control can be used for the speed loop. When C01.1B is 100%, the speed loop uses proportional-integral (PI) control; when C01.1B is 0%, the speed loop switches to pure integral-proportional (IP) control.

In PI control mode, the speed response is faster, but the overshoot increases. In IP control mode, the speed response decreases correspondingly, but the follow-up is better and the overshoot decreases.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.1B | PDFF control coefficient | 0 to 1000 | 1000 | 0.1% | Decreasing the value can reduce the speed overshoot. | During operation | Immediately |

7.5    Gain Switchover

In position and speed modes, gain switchover can improve the system responsiveness and reference follow- up and reduce the positioning time.

High gain parameters correspond to the second group of loop gains, while low gain parameters correspond to the first group of loop gains. When the switchover conditions are met, the loop gain will switch between the first and second groups of gains.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.38 | Gain switchover mode | 0 to 8 | 0 | - | Set the gain switchover mode. | During operation | Immediately |
| C01.39 | Gain switchover time | 10 to 10000 | 50 | 0.1ms | Set the gain switchover time. | During operation | Immediately |
| C01.3A | Gain switchover threshold | 0 to 65535 | 10 | - | Set the gain switchover threshold. | During operation | Immediately |
| C01.3B | Gain switchover loop width | 0 to 65535 | 10 | - | Set the gain switchover loop width. | During operation | Immediately |
| C01.00 | 1st position loop gain | 0 to 20000 | 400 | 0.1rad/ s | Increasing the setpoint improves the responsiveness and shortens the positioning time. | During operation | Immediately |
| C01.01 | 1st speed loop gain | 1 to 20000 | 250 | 0.1Hz | Increasing the setpoint improves the speed follow-up responsiveness of the servo system. | During operation | Immediately |
| C01.02 | 1st speed loop integral time parameter | 1 to 51200 | 3184 | 0.01ms | Reducing the setpoint can enhance the integral effect and shorten the positioning time. | During operation | Immediately |
| C01.03 | 1st torque reference filter cutoff frequency | 5 to 16000 | 200 | Hz | Reducing the setpoint improves the filtering effect but increases the delay. | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.08 | 2nd position loop gain | 0 to 20000 | 560 | 0.1rad/ s | Increasing the setpoint improves the responsiveness and shortens the positioning time. | During operation | Immediately |
| C01.09 | 2nd speed loop gain | 1 to 20000 | 350 | 0.1Hz | Increasing the setpoint improves the speed follow-up responsiveness of the servo system. | During operation | Immediately |
| C01.0A | 2nd speed loop integral time parameter | 1 to 51200 | 2274 | 0.01ms | Reducing the setpoint can enhance the integral effect and shorten the positioning time. | During operation | Immediately |
| C01.0B | 2nd torque reference filter cutoff frequency | 5 to 16000 | 280 | Hz | Reducing the setpoint improves the filtering effect but increases the delay. | During operation | Immediately |

Mode description:

| C01.38 | Switchover Mode | Switchover Time | Switchover Threshold | Switchover Loop Width | Threshold and Loop Width Unit |
| --- | --- | --- | --- | --- | --- |
| 0 | Fixed to the 1st gain set | Inactive | Inactive | Inactive | - |
| 1 | DI switchover | Active | Active | Active | - |
| 2 | DI P-PI switchover | Active | Active | Active | - |
| 3 | Torque reference | Active | Active | Active | 0.1% |
| 4 | Speed reference | Active | Active | Active | rpm |
| 5 | Speed feedback | Active | Active | Active | rpm |
| 6 | Speed reference change rate | Active | Active | Active | rpm/ms |
| 7 | Position deviation | Active | Active | Active | p |
| 8 | Position reference | Active | Active | Active | p |

7.6    Speed Feedforward

Speed feedforward can be applied to position control mode to improve the speed reference responsiveness, shorten the positioning time, and reduce the position deviation at fixed speed.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.13 | Speed feedforward source | 0 to 5 | 0 | - | 0: No feedforward 1: Internal reference 2: Model tracking 5: Communication | At stop | Immediately |
| C01.14 | Speed feedforward percentage | 0 to 2000 | 0 | 0.1% | Increasing the speed feedforward improves the responsiveness. | During operation | Immediately |
| C01.15 | Speed feedforward filter cutoff frequency | 5 to 16000 | 318 | Hz | Decreasing the cutoff frequency improves the feedforward smoothness but increases the response delay. | During operation | Immediately |

7.7    Torque Feedforward

Torque feedforward can be applied only to the position and speed modes.

In position control mode, torque feedforward can improve torque reference responsiveness and reduce the speed deviation during operation at a constant speed. In speed control mode, torque feedforward can improve torque reference responsiveness and reduce the speed deviation during acceleration and deceleration.

A too high setpoint of torque feedforward may cause overshoot.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.16 | Torque feedforward source | 0 to 5 | 0 | - | 0: No feedforward 1: Internal reference 2: Model tracking 5: Communication | At stop | Immediately |
| C01.17 | Torque feedforward percentage | 0 to 2000 | 0 | 0.1% | Increasing the torque feedforward improves the responsiveness. | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.18 | Torque feedforward filter cutoff frequency | 5 to 16000 | 318 | Hz | Decreasing the cutoff frequency improves the feedforward smoothness but increases the response delay. | During operation | Immediately |

7.8   Position Reference Filter

The position reference filter filters the position references (encoder unit) divided or multiplied by the electronic gear ratio to smoothen motor operation and reduce the shock on the machine.

The position reference filter includes the low-pass and overlapping average filters.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.20 | Position reference overlapping average filter time constant A | 0 to 1280 | 0 | 0.1ms | Increasing the setpoint improves the reference smoothness but increases the delay. | At stop | Immediately |
| C01.21 | Position reference overlapping average filter time constant B | 0 to 1280 | 0 | 0.1ms | Increasing the setpoint improves the reference smoothness but increases the delay. | At stop | Immediately |
| C01.22 | Position reference low-pass filter time constant A | 0 to 65535 | 0 | 0.1ms | Increasing the setpoint improves the reference smoothness but increases the delay. | At stop | Immediately |
| C01.23 | Position reference low-pass filter time constant B | 0 to 65535 | 0 | 0.1ms | Increasing the setpoint improves the reference smoothness but increases the delay. | At stop | Immediately |

7.9   Model Tracking Control

Model tracking control can improve the responsiveness and shorten the positioning time. This function is only available in the position control mode.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C02.00 | Model tracking control | 0 to 1 | 0 | - | 0: Disabled 1: Enabled | At stop | Immediately |
| C02.01 | Model tracking control gain | 10 to 20000 | 500 | 0.1rad/ s | Increasing the setpoint improves the position tracking. | During operation | Immediately |
| C02.02 | Model tracking inertia correction coefficient | 10 to 8000 | 1000 | 0.1% | When the inertia ratio setpoint is not accurate, this value can be used for correction. | During operation | Immediately |

7.10 Speed Feedback Filter

When the encoder bit number is low or the noise contribution is large, the speed feedback fluctuation or burr calculated by the drive is large. You can set the speed feedback low-pass filter or overlapping average filter to reduce the speed feedback fluctuation. However, a too high setpoint will increase the delay in the servo system, which could potentially cause system oscillation.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.10 | Speed feedback filter | 0 to 4 | 0 | - | 0: Internal setting 1: Low-pass filter 2: Overlapping average filter 3: Speed observer 4: No filter | At stop | Immediately |
| C01.11 | Cutoff frequency of speed feedback low- pass filter | 10 to 16000 | 8000 | Hz | Set the cutoff frequency of the low-pass filter. | During operation | Immediately |
| C01.12 | Speed feedback overlapping average filter time constant | 0 to 6 | 0 | - | 0: No filter 1: Two times filter 2: Four times filter 3: Eight times filter 4: Sixteen times filter 5: Thirty-second times filter 6: Sixty-fourth times filter | During operation | Immediately |

7.11 Speed Observer

The speed observer can filter high-frequency signals for speed feedback, reduce the impact of encoder position feedback noise on the servo system, and improves the stiffness level of the servo system to some extent.

To enable the speed observer function, set C01.10 to 3.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.10 | Speed feedback filter | 0 to 4 | 0 | - | 0: Internal setting 1: Low-pass filter 2: Overlapping average filter 3: Speed observer 4: No filter | At stop | Immediately |
| C02.30 | Speed observer gain | 0 to 40000 | 0 | 0.1Hz | Increasing the setpoint improves the observation speed responsiveness, but a too large value can cause oscillation. | During operation | Immediately |
| C02.31 | Speed observer inertia correction | 10 to 8000 | 1000 | 0.1% | When the inertia ratio setpoint is not accurate, this value can be used for correction. | During operation | Immediately |
| C02.32 | Speed observer speed feedback cutoff frequency | 0 to 16000 | 0 | Hz | Set the cutoff frequency of the speed observer low-pass filter. | During operation | Immediately |

7.12 Disturbance Observer

The disturbance observer effectively observes the external disturbance. Disturbances within the frequency range can be observed and suppressed with different cutoff frequencies and compensation values.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C02.60 | Disturbance observer gain | 0 to 40000 | 0 | 0.1Hz | Increasing the value improves the responsiveness to disturbances, but a too large value can cause vibration more easily. | During operation | Immediately |
| C02.61 | Disturbance observer inertia correction coefficient | 1 to 10000 | 1000 | 0.1% | When the inertia ratio setpoint is not accurate, this value can be used for correction. | During operation | Immediately |
| C02.62 | Disturbance observer low-pass cutoff frequency | 0 to 16000 | 0 | Hz | Set the cutoff frequency of the speed observer low-pass filter. | During operation | Immediately |
| C02.63 | Disturbance observer compensation torque percentage | 0 to 2000 | 0 | 0.1% | Set the percentage for observation compensation. | During operation | Immediately |

7.13 Friction Compensation

The friction compensation function is used to compensate for changes in viscous friction and variations in fixed loads.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C02.68 | Friction compensation switch and relevant setting | 0 to 0xFF | 0 | - | Bit 0: 0: Disabled 1: Enabled Bit 4: 0: Speed threshold from speed reference 1: Speed threshold from speed feedback | During operation | Immediately |
| C02.69 | Friction compensation speed threshold | 0 to 5000 | 20 | 0.1rpm | Set it to the coulomb friction compensation speed threshold. | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C02.6A | Static friction compensation | 0 to 2000 | 0 | 0.1% | Set it to the static friction compensation value. | During operation | Immediately |
| C02.6B | Forward friction compensation of coulomb friction | 0 to 2000 | 0 | 0.1% | Set it to the friction force of compensation for the position reference in the forward direction. | During operation | Immediately |
| C02.6C | Reverse friction compensation of coulomb friction | -2000 to 0 | 0 | 0.1% | Set it to the friction force of compensation for the position reference in the reverse direction. | During operation | Immediately |
| C02.6D | Viscous friction torque for rated speed | 0 to 2000 | 0 | 0.1% | Set it to the viscous friction torque for rated speed. | During operation | Immediately |
| C02.6E | Friction compensation filter time | 0 to 65535 | 0 | 0.01ms | Determine the speed after overcoming resistance friction. | During operation | Immediately |
| C02.6F | Friction compensation threshold for zero speed | 0 to 1000 | 10 | 0.1rpm | Set it to the friction compensation threshold for the zero speed. | During operation | Immediately |

7.14 Vibration Suppression

The notch can suppress mechanical resonance by reducing the gain at a specific frequency. After the notch is correctly set, vibration can be effectively suppressed, and it may be possible to continue increasing the servo gain. The notch principle is shown in the figure below.

Amplitude frequency

characteristics of the mechanical system

Mechanical resonance

frequency

Frequency

Notch characteristics

Notch width

Notch depth

fL

fH

Central frequency

of the notch fO

Frequency

Figure 7-1    Notch suppression

The servo drive has a total of five notches, and each is defined by three parameters: notch frequency, width level, and depth level. The 1st and 2nd notches can be set manually or configured as adaptive notches (C01.30 = 1 or 2). In this case, the parameters are automatically set by the drive, while the other three notches can be set manually.

Steps to use adaptive notches:

①Set C01.30 (adaptive notch mode) to 1 or 2 based on the number of resonance points.

②When resonance occurs, set C01.30 to 1 to enable one adaptive notch. If resonance occurs again after gain tuning, set C01.30 to 2 to enable two adaptive notches. Parameters of the 1st and 2nd notches are updated automatically during servo operation.

③If resonance is suppressed, the adaptive notch functions well. If resonance persists, use the backend tool to observe waveforms of related variables and use the other three notches to suppress resonance.

Related parameters:

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.30 | Adaptive notch mode | 0 to 4 | 0 | - | 0: Disabled 1: 1st notch 2: 2nd notch 3: Notch parameter reset 4: Resonance frequency tested only | During operation | Immediately |

| Parameter | Name | Value Range | Default | Unit | Options | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01.31 | Adaptive notch test times | 0 to 65535 | 0 | Times | - | During operation | Immediately |
| C01.40 | Frequency of the 1st notch | 10 to 8000 | 8000 | Hz | Set it to the frequency of the 1st notch. | During operation | Immediately |
| C01.41 | Width level of the 1st notch | 0 to 4000 | 0 | 0.1% | Set it to the width level of the 1st notch. | During operation | Immediately |
| C01.42 | Depth level of the 1st notch | 10 to 1000 | 1000 | 0.1% | Set it to the depth level of the 1st notch. | During operation | Immediately |
| C01.43 | Frequency of the 2nd notch | 10 to 8000 | 8000 | Hz | Set it to the frequency of the 2nd notch. | During operation | Immediately |
| C01.44 | Width level of the 2nd notch | 0 to 4000 | 0 | 0.1% | Set it to the width level of the 2nd notch. | During operation | Immediately |
| C01.45 | Depth level of the 2nd notch | 10 to 1000 | 1000 | 0.1% | Set it to the depth level of the 2nd notch. | During operation | Immediately |
| C01.46 | Frequency of the 3rd notch | 10 to 8000 | 8000 | Hz | Set it to the frequency of the 3rd notch. | During operation | Immediately |
| C01.47 | Width level of the 3rd notch | 0 to 4000 | 0 | 0.1% | Set it to the width level of the 3rd notch. | During operation | Immediately |
| C01.48 | Depth level of the 3rd notch | 10 to 1000 | 1000 | 0.1% | Set it to the depth level of the 3rd notch. | During operation | Immediately |
| C01.49 | Frequency of the 4th notch | 10 to 8000 | 8000 | Hz | Set it to the frequency of the 4th notch. | During operation | Immediately |
| C01.4A | Width level of the 4th notch | 0 to 4000 | 0 | 0.1% | Set it to the width level of the 4th notch. | During operation | Immediately |
| C01.4B | Depth level of the 4th notch | 10 to 1000 | 1000 | 0.1% | Set it to the depth level of the 4th notch. | During operation | Immediately |
| C01.4C | Frequency of the 5th notch | 10 to 8000 | 8000 | Hz | Set it to the frequency of the 5th notch. | During operation | Immediately |
| C01.4D | Width level of the 5th notch | 0 to 4000 | 0 | 0.1% | Set it to the width level of the 5th notch. | During operation | Immediately |
| C01.4E | Depth level of the 5th notch | 10 to 1000 | 1000 | 0.1% | Set it to the depth level of the 5th notch. | During operation | Immediately |

Chapter 8	Communication Description

8.1   Overview

    - EtherCAT Overview

EtherCAT is an easy-to-use, industrial Ethernet technology featuring the high-performance, low cost, and flexible topology. It can be used for ultra-high-speed I/O networks at the industrial field level, and uses the standard Ethernet physical layer, with transmission media being twisted pair or optical fiber (100Base-TX or 100Base-FX).

Start

Set system parameters

Import the XML file

Refer  to  th e  documents related to the host controller.

Refer to the section of setting system parameters.

Set communication parameters

Configure the PDO

End

Start the remote node

Refer to the section of  status

indicators.

The EtherCAT system consists of a master and several slaves. The master only requires a regular network card, while a slave requires a dedicated slave control chip, such as ET1100, ET1200, and FPGA.

EtherCAT provides an end-to-end connection over one network, with protocol processing reaching directly to the I/O layer:

y   No need for any underlying sub-bus

y   No gateway delay

y   A single system covering all devices, including the I/O, sensor, activator, drive, and display

y   Transmission rate: 2 x 100 Mbit/s (high speed Ethernet, full duplex mode)

y Synchronism: 300 nodes between two devices, cable of 120 m, and synchronization jitter less than 1 μs

y   Update time:

256 digital I/O: 11 μs

1000 digital I/O distributed over 100 nodes: 30 μs = 0.03 ms 200 analog I/O (16-bit): 50 μs, sampling rate of 20 kHz

100 servo axes (each 8-byte IN+OUT): 100 μs = 0.1 ms 12000 digital I/O: 350 μs

To support a wider variety of devices and application layers, EtherCAT provides the following application protocols:

y  CANopen over EtherCAT (CoE) (CAN application protocol based on EtherCAT)

y   SoE (Servo drive specification compliant with IEC 61800-7-204 standard)

y   EoE (Ethernet implemented by EtherCAT)

y   FoE (File reading implemented by EtherCAT)

Slave devices do not need to support all communication protocols. Instead, only the most suitable communication protocols need to be selected for their applications.

PLC

Remote I/O

……

PE

PE

PE

PE

PE

Figure 8-1    EtherCAT networking

NOTICE

y EtherCAT is a registered trademark and patented technology, licensed by Beckhoff Automation GmbH, Germany.

    - EtherCAT communication technical specifications

| Item |  | Specifications |
| --- | --- | --- |
| Basic performance of EtherCAT slaves | Communication protocol | EtherCAT protocol |
|  | Service supported | CoE (Process Data Object (PDO) and Service Data Object (SDO)) |
|  | Synchronization mode | Distributed clock (DC) |
|  | Physical layer | 100BASE-TX |
|  | Baud Rate | 100 Mbit/s (100Base-TX) |
|  | Duplex mode | Full duplex |
|  | Topology | Linear |
|  | Transmission medium | Cat5 shielded cables or Cat6 and above cables with electrical performance specifications |
|  | Transmission distance | Less than 100 m between two nodes (with proper environment and cables) |
|  | Number of slaves | Up to 65535 supported by protocol, not exceeding 100 in actual use |
|  | EtherCAT frame length | 44 bytes to 1,498 bytes |
|  | Process data | Up to 1486 bytes per Ethernet frame |
|  | Synchronization jitter of two slaves | <1us |
|  | Update time | About 30 μs for 1000 digital input/output About 100 μs for 100 servo axes Update time varying with the different interface types |
|  | Communication bit error ratio | 10-10 Ethernet standard |
| EtherCAT configuration units | Number of FMMU units | 8 |
|  | Number of storage synchronization management units | 8 |
|  | Process data RAM | 8 KB |
|  | DC | 64 bits |
|  | E2PROM capacity | 32 kbit |

    - EtherCAT communication specification

| Item |  | Specifications |
| --- | --- | --- |
| Communication protocol |  | IEC 61158 Type 12, IEC 61800-7 CiA 402 Drive Profile |
| Application Layer | SDO | SDO request and response |
|  | PDO | Variable PDO mapping |
|  | CiA402 | PP mode |
|  |  | PV mode |
|  |  | PT mode |
|  |  | HM mode |
|  |  | CSP mode |
|  |  | CSV mode |
|  |  | CST mode |
| Physical layer | Transmission protocol | 100BASE-TX (IEEE802.3) |
|  | Max. distance | 100M |
|  | Interface | RJ45*2 (IN, OUT) |

8.2   Communication Transmission Modes

    - EtherCAT Communication Structure

Various application layer protocols can be used in EtherCAT communication. The A6-EC series servo drive adopts the IEC 61800-7 (CiA 402)-CANopen motion control sub-protocol.

The following figure shows the EtherCAT communication structure based on the CANopen application layer.

Application layer

ESC DPRAM

Link layer

Object dictionary

EtherCAT state machine

SDO

PDO

COE

Register

Mailbox

Process data

Physical layer

Figure 8-2    EtherCAT communication structure based on the CANopen application layer

As shown in the figure, the object dictionary of the application layer contains the following: Communication parameters, application data, and PDO mapping data PDOs contain real-time data during the operation of the servo drive, and are accessed for reading and writing periodically. SDO mailbox communication accesses and modifies some communication parameter objects and PDOs in a non-periodic manner.

    - Communication State Machine

CiA402 control introduction

Stop at fault

Initialization

Fault

No fault for servo

Servo ready

Waiting for the S﹣ON signal

Quick stop

Servo running

⑬

Start

⓪

⑭

①

⑮

②	⑦

⑫	⑩

③

⑥

⑨

⑧

④	⑤

⑯	⑪

Figure 8-3    CiA402 state machine switchover

The A6-EC series servo drive should be operated according to the process specified by the standard CiA402 protocol, so that the servo drive can operate in the specified state. The following table lists each status.

| State | Description |
| --- | --- |
| Initialization | y Initialization of the servo drive and internal self-inspection are done. y Parameters of the servo drive cannot be set. Drive functions cannot be executed. |
| No fault for servo | y No fault exists in the servo drive or the fault has been cleared. y Parameters of the servo drive can be set. |

| State | Description |
| --- | --- |
| Servo ready | y The servo drive is ready to run. y Parameters of the servo drive can be set. |
| Waiting for the S-ON signal | y The servo drive is waiting for the S-ON signal. y Parameters of the servo drive can be set. |
| Servo running | y The servo drive is running properly and a certain operation mode has been enabled. The motor is energized and starts rotating when the speed reference value inputted is not 0. y Only parameters whose "Setting Condition" is "During running" can be set. |
| Quick stop | y Quick stop is activated and the servo drive is in the process of quick stop. y Only parameters whose "Setting Condition" is "During running" can be set. |
| Stop at fault | y A fault occurs and the servo drive is in the process of stop. y Only parameters whose "Setting Condition" is "During running" can be set. |
| Fault | y The stop process is done and all the drive functions are disabled. Parameters can be modified for the troubleshooting purpose. |

EtherCAT status switchover

Init

Pre﹣Operational

Safe﹣Operational

Operational

(IP)	(PI)

(SI)

(OI)

(PS)

(SP)

(OP)

(SO)

(OS)

Figure 8-4 EtherCAT state machine

EtherCAT devices must support four statuses to coordinate the status relationship between the master and slave applications during initialization and operation.

y   Init: Initialization, short as I

y   Pre-Operational: Pre-operational, short as P y  Safe-Operational: Safe-operational, short as S y   Operational: Operational, short as O

When the initialized state is switched to the operational state, it must follow the sequence "Initialized → Pre-operational → Safe-operational → Operational", and running-through is not allowed. However, when the state is returned from the operational state, running-through is allowed.

The following table describes the status switchover and initialization process.

| State | SDO | RPDO | TPDO | Description |
| --- | --- | --- | --- | --- |
| Initialization (I) | No | No | No | y Communication is initialized. y There is no communication at the application layer, and the master can only read data from and write data to the ESC register. |
| IP | No | No | No | y The master configures the slave addresses. y The mailbox channel is configured. y The DC is configured. y The pre-operational state is requested. |
| Pre-operational (P) | zes | No | No | y There is mailbox data communication (SDO) at the application layer. |
| PS | zes | No | No | y The master uses SDOs to initialize process data mapping. y The master configures the SM channel used for process data communication. y The master configures FMMU. y The safe-operational state is requested. |
| Safe-operational (S) | zes | No | zes | y Both SDOs and Transmission PDOs (TPDOs) are available. y The DC mode is available. |
| SO | zes | No | zes | y The master sends valid output data. y The operational state is requested. |
| Operational (O) | zes | zes | zes | y It is in the normal operation state. y All the input and output data is valid. y Mailbox communication is available. |

    - DC

The DC allows all EtherCAT devices to use the same system time, thereby controlling the synchronous execution of tasks on each device. Slave devices can generate sync signals based on the synchronized system time. The A6-EC series servo drive only supports the DC sync mode. The synchronization cycle is controlled by SYNC0. The period range varies with different motion modes.

NOTICE

y The SYNC signal can be used for synchronization of all slaves and can achieve an error of less than 1 μs. Before the SYNC signal starts, the master needs to synchronize all slaves to the same clock. In the operational state, it also needs to continuously synchronize the slaves to the same clock to prevent the difference in the crystal oscillator from causing a clock offset. This is generally manifested as synchronizing the 0x910 register of the ESC. y The SYNC starting time is the time of the ESC 0x990 register minus the 0x920 time. Enable the DC mode (0x981 = 0x03) before the 0x910 reaches the starting time. If the SYNC starting time setting is incorrect, the ESC 0x134 status register will report a fault code of 0x2D.

8.3    Communication Data Frame Structure

    - Process Data

EtherCAT real-time data is transmitted through PDOs. According to the data transmission direction, PDOs can be divided into Reception PDOs (RPDOs) and TPDOs. RPDOs send master data to slaves, and TPDOs feed back slave data to the master.

RxPDO:

Control word and target position

TxPDO:

Status word and position feedback

Master

Slave

The A6-EC series servo drive supports user-defined PDO lists and PDO mapping objects.

PDO mapping

PDO mapping is used to establish the mappings between the object dictionary and PDOs. 1600h to 17FFh are RPDOs, and 1A00h to 1BFFh are TPDOs. The A6-EC series servo drive provides six RPDOs and five TPDOs, which are described in the following table.

| RPDO (6) | 1600h | Variable mapping |
| --- | --- | --- |
|  | 1701h to 1705h | Fixed mapping |
| TPDO (5) | 1A00h | Variable mapping |
|  | 1B01h to 0x1B04h | Fixed mapping |

Fixed PDO mapping

The A6-EC series servo drive provides five fixed RPDOs and four fixed TPDOs. The following table lists some typical uses of RPDOs and TPDOs.

| Available Servo Mode | PP CSP |
| --- | --- |
| 1701h (Output) | Mapping object (four 12-byte objects) |
|  | 6040h (control word) 607Ah (target position) 60B8h (touch probe function) 60FEh sub-index 1 (forced physical DO) |
| 1B01h (Input) | Mapping object (nine 28-byte objects) |
|  | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 6077h (torque feedback) 60F4h (position deviation) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60BCh (touch probe 2 positive edge) 60FDh (DI status) |

| Available Servo Mode | PP PV PT CSP CSV CST |
| --- | --- |
| 1702h (Output) | Mapping object (seven 19-byte objects) |
|  | 6040h (control word) 607Ah (target position) 60FFh (target velocity) 6071h (target torque) 6060h (mode selection) 60B8h (touch probe function) 607Fh (Max. speed) |
| 1B02h (Input) | Mapping object (nine 25-byte objects) |
|  | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 6077h (torque feedback) 6061h (mode display) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60BCh (touch probe 2 positive edge) 60FDh (DI status) |

| Available Servo Mode | PP PV CSP CSV |
| --- | --- |
| 1703h (Output) | Mapping object (seven 17-byte objects) |
|  | 6040h (control word) 607Ah (target position) 60FFh (target velocity) 6060h (mode selection) 60B8h (touch probe function) 60E0h (positive torque limit) 60E1h (negative torque limit) |
| 1B03h (Input) | Mapping object (ten 29-byte objects) |
|  | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 6077h (torque feedback) 60F4h (position deviation) 6061h (mode display) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60BCh (touch probe 2 positive edge) 60FDh (DI status) |

| Available Servo Mode | PP PV PT CSP CSV CST |
| --- | --- |
| 1704h (Output) | Mapping object (nine 23-byte objects) |
|  | 6040h (control word) 607Ah (target position) 60FFh (target velocity) 6071h (target torque) 6060h (mode selection) 60B8h (touch probe function) 607Fh (Max. speed) 60E0h (positive torque limit) 60E1h (negative torque limit) |
| 1B02h (Input) | Mapping object (nine 25-byte objects) |
|  | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 6077h (torque feedback) 6061h (mode display) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60BCh (touch probe 2 positive edge) 60FDh (DI status) |

| Available Servo Mode | PP PV CSP CSV |
| --- | --- |
| 1705h (Output) | Mapping object (eight 19-byte objects) |
|  | 6040h (control word) 607Ah (target position) 60FFh (target velocity) 6060h (mode selection) 60B8h (touch probe function) 60E0h (positive torque limit) 60E1h (negative torque limit) 60B2h (torque offset) |
| 1B04h (Input) | Mapping object (ten 29-byte objects) |
|  | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 6077h (torque feedback) 6061h (mode display) 60F4h (position deviation) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60BCh (touch probe 2 positive edge) 606Ch (speed feedback) |

Variable PDO mapping

The A6-EC series servo drive provides one variable RPDO and one variable TPDO.

| Variable PDO | Index | Max. Number of Mappings | Max. Number of Bytes | Default Mapping Object |
| --- | --- | --- | --- | --- |
| RPDO1 | 1600h | 10 | 40 | 6040h (control word) 607Ah (target position) 60B8h (touch probe function) |
| TPDO1 | 1A00h | 10 | 40 | 603Fh (fault code) 6041h (status word) 6064h (position actual value) 60BCh (touch probe 2 positive edge) 60B9h (touch probe status) 60BAh (touch probe 1 positive edge) 60FDh (DI status) |

Sync manager PDO assignment setting

In EtherCAT periodic data communication, process data can contain multiple PDOs. the CoE protocol uses

data objects 0x1C10 to 0x1C2F to define the PDO mapping object list of the corresponding sync manager (SM) channel. Multiple PDOs can be mapped in different sub-indexes. The A6-EC series servo drive supports one RPDO assignment and one TPDO assignment, which are listed in the following table.

| Index | Sub-index | Item |
| --- | --- | --- |
| 0x1C12 | 01h | Choose one from 0x1600 and 0x1701 to 0x1705 as the actual RPDO used. |
| 0x1C13 | 01h | Choose one from 0x1A00 and 0x1B01 to 0x1B04 as the actual TPDO used. |

PDO configuration

PDO mapping parameters contain pointers to PDO-related process data that PDOs need to send or receive, including the index, sub-index, and mapping object length. Among them, sub-index 0 records the number N of specific objects mapped by the PDO. Each PDO data length can be up to 4 x N bytes and can map one or more objects at the same time. Sub-indexes 1 to N are the mapping content. The following table defines the mapping parameter content.

| Bit | 31 | …… | 16 | 15 | …… | 8 | 7 | …… | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meaning | Index |  |  | Sub-index |  |  | Object length |  |  |

The index and sub-index together determine the position of the object in the object dictionary, and the object length specifies the specific number of bits for the object, represented in hexadecimal.

| Object length | Bits |
| --- | --- |
| 08h | 8 bits |
| 10h | 16 bits |
| 20h | 32 bits |

For example, the mapping parameter for the 16-bit control word 6040h-00 is 60400010h. The PDO configuration of the A6-EC series servo drive follows the process:

①Configure PDO mapping groups. Write 0 to the 00h sub-index of 1C12h (or 1C13h).

      - Clear the original mapping group: Writing "0" to the 00h sub-index of 1C12h (or 1C13h) can clear this PDO configuration group.

      - Write the PDO mapping group: Write the mapping configuration group based on the site requirements. Pre-write the values of 1600h and 1701h to 1705h in 1C12h, and pre-write the values of 1A00h and 1B01h to 1B04h in 1C13h. (Note: Only 1600h and 1A00h are configurable mapping groups. Others are fixed mapping configurations.)

      - Write the total number of the PDO mapping groups to sub-index 0 of the 1C12h (or 0x1C13h) object.

②Configure PDO mapping objects. Write 0 to the 00h sub-index of 1600h (or 1A00h).

        - Clear the original mapping object: Writing "0" to the 00h sub-index of 1600h (or 1A00h) can clear this PDO mapping configuration.

        - Write the PDO mapping content: According to the object parameter definition in the XML file, write the content to the mapping parameter sub-indexes 1 to 10 respectively. Only the objects that support mapping can be configured as the PDO mapping content.

        - Write the total number of PDO mapping objects, and write the number of mappings written in Step b to sub-index 0.

CAUTION

y The PDO configuration can be set only when the EtherCAT communication state machine is in pre-operational (2 displayed on the panel). Otherwise, an error will be reported. y PDO configuration parameters cannot be stored in E2PROM. Be sure to reconfigure the mapping object after each power-on. Otherwise, the mapping object will be the default parameter of the servo drive.

The following operations will return an SDO fault code:

y   Modifying PDO parameters in a non-pre-operational state.

y   Pre-writing values other than 1600h and 1701h to 1705h in 1C12, or pre-writing values other than

1A00h and 1B01h to 1B04h in 1C13.

    - Mailbox Data

The EtherCAT mailbox data SDO is used to transmit non-periodic data, such as the configuration of communication parameters and servo drive operation parameters. CoE service types of EtherCAT include the following:

y   Emergency event information

y   SDO request

y   SDO response

y   Transmit PDO (TxPDO)

y   Receive PDO (RxPDO)

y  Remote request to send TxPDOs y  Remote request to send RxPDOs y   SDO information

The A6-EC series servo drive supports SDO requests and responses.

| Code | Name | Function Name | Description | Remarks |
| --- | --- | --- | --- | --- |
| Input signal functions |  |  |  |  |
| FunIN.1 | S-ON | S-ON | Inactive: Servo motor disabled in local mode Active: Servo motor enabled in local mode | The S-ON function is only active in non-bus control mode. The logic of the corresponding terminal must be set to level valid. |
| FunIN.2 | ALM-RST | Alarm reset signal | Active: Fault reset executed in local mode Inactive: Fault reset not executed in local mode | The ALM-RST function is only active in non-bus control mode. It is recommended to set the logic of the corresponding terminal to level valid. |
| FunIN.6 | P-OT | PL switch | Active: Forward drive disabled Inactive: Forward drive enabled | When the mechanical movement is beyond the movable range, the overtravel prevention function will be activated. It is recommended to set the logic of the corresponding terminal to level valid. |
| FunIN.7 | N-OT | Negative limit switch | Active: Reverse drive disabled Inactive: Reverse drive enabled | When the mechanical movement is beyond the movable range, the overtravel prevention function will be activated. It is recommended to set the logic of the corresponding terminal to level valid. |
| FunIN.5 | HomeSwitch | Home switch | Inactive: Mechanical load beyond the home switch range Active: Mechanical load within the home switch range | The logic of the corresponding terminal must be set to level valid. |
| FunIN.4 | Emergency Stop | Emergency stop | Active: Position locked after stop at zero speed Inactive: Current operating state unaffected | It is recommended to set the logic of the corresponding terminal to level valid. |

| Code | Name | Function Name | Description | Remarks |
| --- | --- | --- | --- | --- |
| Input signal functions |  |  |  |  |
| FunIN.30 | TouchProbe1 | Probe 1 | Inactive: Touch probe not triggered Active: Touch probe triggerable | The touch probe logic is only related to the touch probe function (60B8h). |
| FunIN.31 | TouchProbe2 | Probe 2 | Inactive: Touch probe not triggered Active: Touch probe triggerable | The touch probe logic is only related to the touch probe function (60B8h). |

| Code | Name | Function Name | Description | Remarks |
| --- | --- | --- | --- | --- |
| Functions of output signals |  |  |  |  |
| FunOUT.1 | S-RDY | Servo ready | The servo drive is ready to receive the S-ON signal. Active: Servo ready Inactive: Servo not ready | - |
| FunOUT.2 | TGON | Motor rotation signal | Inactive: Absolute value of filtered motor speed lower than the setpoint of C03.2D Active: Absolute value of filtered motor speed reaching the setpoint of C03.2D | - |
| FunOUT.3 | BK | Brake output | Active: Brake signal outputted Inactive: Brake signal not outputted | - |
| FunOUT.5 | WARN | Alarm | Active: Warning occurred on the servo drive Inactive: No warning occurred on the servo drive or the warning has been reset | - |
| FunOUT.4 | ALM | Fault | Active: Fault occurred on the servo drive Inactive: No fault occurred on the servo drive or the fault has been reset | - |

10.1 Fault Alarms

    - Fault display and category

The servo drive provides various protection functions, and triggers an alarm when a protection functions acts. Then, the LED panel displays the fault and its alarm code.

Figure 10-1    Fault code display

NOTICE

y The servo drive can record the latest 10 faults/alarms and the servo drive status parameters upon occurrence of the faults/alarms. Repeated faults or alarms among the latest 5 logs are logged as one fault or alarm, and the servo drive status upon its occurrence is logged only once. y When a single fault or an alarm occurs, the panel displays the fault or alarm code. When multiple faults or alarms occur, the panel displays the fault code of the highest level. y After a fault or an alarm is reset, the servo drive still keeps the log of the fault or alarm. You can set F31.04 (Initialize fault record) to 1 to clear the fault and alarm records.

Alarm codes are divided into three categories (category 1 indicates the most severe level) based on the fault and alarm severity, which are specified by fault codes.

| Fault Category | Fault Code | Resettable |
| --- | --- | --- |
| Class 1 | Er0x.x to Er3x.x | Non-resettable |
|  | Er4x.x to Er7x.x | Resettable |
| Class 2 | Er8x.x to ErCx.x | Resettable |
| Class 3 | ALFxx | Resettable |

NOTICE

y "Resettable" means that the panel stops displaying the fault/alarm when a "reset signal" is input.

    - Troubleshooting and reset

Checklist:

Whether the parameters are set properly?

Whether a fault code is displayed?

Whether the cable of the control signal CN1 is wired properly

or is disconnected?

Whether the cable of the encoder signal CN2 is wired properly

or is disconnected?

Whether the power supply voltage changes?

Whether there is power input? Whether the connection is loose?

Whether the wiring is correct? Whether the cable is disconnected?

Whether the short circuit cable is disconnected?

Whether the motor has noise?

Whether the connection is loose?

Mechanical

A6

System ground

Figure 10-2    Troubleshooting checklist

Reset operation:

y   Set F31.00 (Fault reset) to 1 to stop displaying the fault on the panel.

y To reset resettable faults, switch off the S-ON signal first and then send the fault reset signal (set F31.00 to 1).

y To reset resettable alarms, eliminate the alarm source and then the servo drive automatically resets the alarms.

CAUTION

y Some faults/alarms can only be reset after the causes are rectified through setting modifications. However, a reset operation does not activate modifications. y If the modification can be effective only after power-on, power on the device again. y If the modification can be effective only after stop, turn off the S-ON signal. The servo drive can operate properly only after modifications are activated.

    - List of faults and alarms

Table 10-1    List of factory fault codes

| Fault Group | Fault Code | Fault Name | Fault Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 1 | Er01.0 | Mismatch of software versions | 0x010 | 0x6100 | Non-resettable |
|  | Er01.1 | Mismatch of motor parameters | 0x011 | 0x7122 | Non-resettable |
|  | Er02.0 | Product matching fault. No specified drive | 0x020 | 0x6100 | Non-resettable |
|  | Er02.1 | Product matching fault. No specified motor | 0x021 | 0x6100 | Non-resettable |
|  | Er02.2 | Product matching fault. No specified encoder | 0x022 | 0x6100 | Non-resettable |
|  | Er03.0 | System parameter error | 0x030 | 0x6320 | Non-resettable |
|  | Er03.1 | Parameter out-of-range | 0x031 | 0x6320 | Non-resettable |
|  | Er03.2 | Parameter writing error | 0x032 | 0x6320 | Non-resettable |
|  | Er03.3 | Parameter reading error | 0x033 | 0x6320 | Non-resettable |
|  | Er05.0 | Current loop timeout | 0x050 | 0x7500 | Non-resettable |
|  | Er05.1 | Speed loop timeout | 0x051 | 0x7500 | Non-resettable |
|  | Er05.2 | Position loop timeout | 0x052 | 0x7500 | Non-resettable |
|  | Er05.3 | Serial port data check failure | 0x053 | 0x7500 | Non-resettable |
|  | Er06.0 | Protection from out of control | 0x060 | 0x8400 | Non-resettable |
|  | Er10.0 | P-hardware overcurrent | 0x100 | 0x2312 | Non-resettable |
|  | Er10.1 | N-hardware overcurrent | 0x101 | 0x2312 | Non-resettable |

| Fault Group | Fault Code | Fault Name | Fault Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 1 | Er10.2 | U phase software overcurrent | 0x102 | 0x2312 | Non-resettable |
|  | Er10.3 | V phase software overcurrent | 0x103 | 0x2312 | Non-resettable |
|  | Er10.4 | Output short circuited to ground | 0x104 | 0x2330 | Non-resettable |
|  | Er10.5 | Current sampling failure | 0x105 | 0x6100 | Non-resettable |
|  | Er10.6 | Incorrect current parameter setting | 0x106 | 0x6320 | Non-resettable |
|  | Er10.7 | UV current correction failure | 0x107 | 0x6100 | Non-resettable |
|  | Er10.8 | Excessive current zero drift | 0x108 | 0x6100 | Non-resettable |
|  | Er10.9 | Current exception during enabling | 0x109 | 0x2312 | Non-resettable |
|  | Er11.0 | Excessive motor speed upon servo drive power-on | 0x110 | 0xFF00 | Non-resettable |
|  | Er11.1 | Drive over-temperature | 0x111 | 0x2312 | Non-resettable |
|  | Er20.1 | Encoder internal fault | 0x201 | 0x7305 | Non-resettable |
|  | Er20.2 | Encoder reading/writing error | 0x202 | 0x7305 | Non-resettable |
|  | Er20.3 | Encoder data frame loss | 0x203 | 0x7305 | Non-resettable |
|  | Er20.4 | Excessive encoder incremental position | 0x204 | 0x7305 | Non-resettable |
|  | Er20.5 | Abnormal encoder data | 0x205 | 0x7305 | Non-resettable |
|  | Er20.6 | Mismatch of encoder type | 0x206 | 0x7305 | Non-resettable |
|  | Er20.7 | Encoder model not supported | 0x207 | 0x7305 | Non-resettable |
|  | Er20.8 | Encoder battery failure | 0x208 | 0x7305 | Non-resettable |
|  | Er20.9 | Encoder multi-turn error | 0x209 | 0x7305 | Non-resettable |
|  | Er21.0 | Mismatch between encoder pulses per revolution and drive pulses per revolution | 0x210 | 0x7305 | Non-resettable |
|  | Er31.0 | More than ten PDO mapping objects | 0x310 | 0x8220 | Non-resettable |
|  | Er32.0 | EtherCAT peripheral error | 0x320 | 0x6100 | Non-resettable |
|  | Er32.1 | ESI check error in FLASH | 0x321 | 0x7600 | Non-resettable |
|  | Er32.2 | Failure to read data from EEPROM through bus | 0x322 | 0x7600 | Non-resettable |

| Fault Group | Fault Code | Fault Name | Fault Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 1 | Er32.3 | Failure of update to EEPROM through bus | 0x323 | 0x7600 | Non-resettable |
|  | Er32.4 | Correctness of checksum in ESC configuration area | 0x324 | 0x7600 | Non-resettable |
|  | Er32.5 | EtherCAT failed to obtain valid XML information | 0x325 | 0x7600 | Non-resettable |
|  | Er40.0 | Drive overload | 0x400 | 0x3230 | Resettable |
|  | Er41.0 | Motor overload | 0x410 | 0x3230 | Resettable |
|  | Er41.1 | Motor over-temperature due to locked- rotor | 0x411 | 0x7121 | Resettable |
|  | Er41.2 | Motor over-temperature | 0x412 | 0x4210 | Resettable |
|  | Er42.1 | Discharge tube temperature too high | 0x421 | 0x4210 | Resettable |
|  | Er42.2 | Heatsink temperature too high | 0x422 | 0x4210 | Resettable |
|  | Er43.0 | Overvoltage | 0x430 | 0x3210 | Resettable |
|  | Er43.1 | Undervoltage | 0x431 | 0x3220 | Resettable |
|  | Er45.0 | S-ON enabling failure | 0x450 | 0xFF00 | Resettable |
|  | Er46.0 | Motor overspeed | 0x460 | 0x8400 | Resettable |
|  | Er47.0 | Excessive position deviation | 0x470 | 0x8611 | Resettable |
|  | Er47.1 | Position deviation overflow | 0x471 | 0x8611 | Resettable |
|  | Er50.1 | D/Q current overflow | 0x501 | 0x6100 | Resettable |
|  | Er51.0 | Offline inertia auto-tuning failure | 0x510 | 0x6310 | Resettable |
|  | Er51.1 | Offline inertia parameter error | 0x511 | 0x6310 | Resettable |
|  | Er52.0 | Angle auto-tuning failure | 0x520 | 0x7122 | Resettable |
|  | Er53.0 | Motor parameter auto-tuning timeout | 0x530 | 0x7122 | Resettable |
|  | Er53.1 | Resistance parameter auto-tuning failure | 0x531 | 0x7122 | Resettable |

| Fault Group | Fault Code | Fault Name | Fault Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 1 | Er53.2 | Inductance parameter auto-tuning failure | 0x532 | 0x7122 | Resettable |
|  | Er53.3 | Back EMF parameter auto-tuning failure | 0x533 | 0x7122 | Resettable |
|  | Er54.0 | Current loop auto-tuning failure | 0x540 | 0x7122 | Resettable |
|  | Er55.0 | Excessive vibration | 0x550 | 0x7122 | Resettable |
|  | Er74.0 | EtherCAT synchronization cycle setting error | 0x740 | 0x6320 | Resettable |
|  | Er74.1 | No sync signal | 0x741 | 0x8700 | Resettable |
|  | Er74.2 | Chip synchronization process uncompleted in OP | 0x742 | 0x8700 | Resettable |
| Class 2 | Er80.0 | Control power undervoltage | 0x800 | 0x3120 | Resettable |
|  | Er81.0 | Input phase loss 1 | 0x810 | 0x3130 | Resettable |
|  | Er81.1 | Input phase loss 2 | 0x811 | 0x3130 | Resettable |
|  | Er81.2 | Output phase loss (reserved) | 0x812 | - | Resettable |
|  | Er82.0 | DI function allocation fault | 0x820 | 0x6320 | Resettable |
|  | Er82.1 | DO function allocation fault | 0x821 | 0x6320 | Resettable |
|  | Er84.0 | Electronic gear ratio setting error | 0x840 | 0x6320 | Resettable |
|  | Er84.1 | Software limit setting error | 0x841 | 0x6320 | Resettable |
|  | Er84.2 | Encoder resolution setting error | 0x842 | 0x7122 | Resettable |
|  | Er84.3 | Home position setting error | 0x843 | 0xFF00 | Resettable |
|  | Er87.1 | One-time excessive position reference increment (One-time increment of the target position is over 5 times of the maximum speed) | 0x871 | 0xFF00 | Resettable |
|  | Er87.2 | Continuous excessive position reference increment (Increment of the target position exceeds the maximum speed for 3 consecutive times) | 0x872 | 0xFF00 | Resettable |
|  | Er87.3 | Overflow of 32-bit sign bit of the target position during limiting | 0x873 | 0xFF00 | Resettable |

| Fault Group | Fault Code | Fault Name | Fault Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 2 | Er87.4 | Target position exceeding maximum value of mechanical single-turn position in rotating mode | 0x874 | 0xFF00 | Resettable |
|  | ErA0.1 | Multi-turn overflow fault | 0xA01 | 0x7305 | Resettable |
|  | ErC1.0 | Excessive EtherCAT synchronization period error | 0xC10 | 0x8700 | Resettable |
|  | ErC1.1 | Synchronization loss | 0xC11 | 0x8700 | Resettable |
|  | ErC1.2 | Network status switchover error | 0xC12 | 0x8700 | Resettable |
|  | ErC1.4 | Network cable connection unreliable | 0xC14 | 0x8700 | Resettable |
|  | ErC1.5 | Data frame loss protection error | 0xC15 | 0x8700 | Resettable |
|  | ErC1.6 | Data frame forwarding error | 0xC16 | 0x8700 | Resettable |
|  | ErC1.7 | Data update timeout | 0xC17 | 0x8700 | Resettable |
|  | ErC1.8 | Watchdog expired | 0xC18 | 0x8700 | Resettable |
|  | ErC2.0 | SYNC signal loss | 0xC20 | 0x8700 | Resettable |

Table 10-2    List of factory alarm codes

| Fault Group | Alarm Code | Alarm Name | Alarm Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 3 | ALF0.0 | Emergency stop alarm | 0x0F00 | 0x0F00 | Resettable |
|  | ALF1.0 | Re-power-on required for parameter settings to take effect | 0xF10 | 0x6320 | Resettable |
|  | ALF1.1 | Frequent parameter storage alarm | 0xF11 | 0x5530 | Resettable |
|  | ALF1.2 | Torque reached parameter error | 0xF12 | 0x6320 | Resettable |
|  | ALF1.3 | Too frequent writing of EEPROM by host controller SDO | 0xF13 | 0x7600 | Resettable |
|  | ALF2.0 | Forward overtravel alarm | 0xF20 | 0x5443 | Resettable |
|  | ALF2.1 | Reverse overtravel alarm | 0xF21 | 0x5444 | Resettable |
|  | ALF4.0 | Homing timeout | 0xF40 | 0x6320 | Resettable |
|  | ALF4.1 | Homing DI conflict | 0xF41 | 0x6320 | Resettable |

| Fault Group | Alarm Code | Alarm Name | Alarm Code (203F) | Bus Fault Code (603F) | Resettable |
| --- | --- | --- | --- | --- | --- |
| Class 3 | ALF4.2 | Homing mode conflict | 0xF42 | 0x6320 | Resettable |
|  | ALF5.0 | Braking resistor overload | 0xF50 | 0x3210 | Resettable |
|  | ALF5.1 | Too small resistance of external regenerative resistor | 0xF51 | 0x6320 | Resettable |
|  | ALF6.1 | Output phase loss | 0xF61 | 0x3230 | Resettable |
|  | ALF8.0 | Vibration occurred during auto-tuning | 0xF80 | 0x7122 | Resettable |
|  | ALF9.0 | Encoder battery voltage low | 0xF90 | 0x7305 | Resettable |
|  | ALFA.0 | Drive high temperature warning | 0xFA0 | 0x7305 | Resettable |
|  | xxnr | Servo not ready | 0xFFFF | - | Resettable |

Table 10-3    List of bus fault codes

| Bus Fault SN | Bus Fault Code | Bus Fault Name |
| --- | --- | --- |
| 0 | 0x0000 | No fault |
| 1 | 0x2312 | Continuous current fault |
| 2 | 0x2330 | Short circuit to ground |
| 3 | 0x3120 | Control power overvoltage |
| 4 | 0x3130 | Phase loss |
| 5 | 0x3210 | Main circuit overvoltage |
| 6 | 0x3220 | Main circuit undervoltage |
| 7 | 0x3230 | Overload |
| 8 | 0x4210 | Over-temperature |
| 9 | 0x5443 | Forward overtravel |
| 10 | 0x5444 | Reverse overtravel |
| 11 | 0x5530 | Storage fault |
| 12 | 0x6320 | Parameter error |
| 13 | 0x7121 | Motor locked-rotor |
| 14 | 0x7122 | Motor mismatch |
| 15 | 0x7305 | Encoder error |

| Bus Fault SN | Bus Fault Code | Bus Fault Name |
| --- | --- | --- |
| 16 | 0x7500 | Communication fault |
| 17 | 0x7600 | Data storage |
| 18 | 0x8400 | Speed control |
| 19 | 0x8611 | Following fault |
| 20 | 0x8220 | Length error |
| 21 | 0x8700 | Synchronization controller |
| 22 | 0x8900 | Process data monitoring |
| 23 | 0x0FFF | Factory fault |

10.2 Solutions

Table 10-4    List of causes and solutions for faults and alarms

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er01.0 | Mismatch of software versions | y The MCU and FPGA versions are incorrect. | y Check whether the software versions are consistent. y Contact technical support personnel or update the FPGA or MCU software. |
| Er01.1 | Mismatch of motor parameters | y Incorrect motor parameters | y Replace with a servo drive or motor of the matching power. y Contact our company for technical support. |
| Er02.0 | Product matching fault. No specified drive | y The set servo drive model is incorrect. | y Check whether the model of the U42.10 servo drive is correct. If not, contact technical support and correct the model. |
| Er02.1 | Product matching fault. No specified motor | y The set motor model is incorrect. | y Read the motor model U42.11 and contact technical support. |
| Er03.0 | System parameter error | y The software is updated. | y Check whether the software is updated. y Reset the servo drive model and the motor model, and restore default settings (set F31.02 to 1). |
|  |  | y The control power voltage drops instantaneously. y Instantaneous power failure occurs during parameter storage. | y Check whether the voltage drops during control power cutoff or instantaneous power failure occurs. y Restore default settings (set F31.02 to 1) and write the parameters again. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
|  |  | y The times of parameter writing within a certain period of time exceeds the limit. | y Check whether parameter update is performed frequently from the host controller. y Change the way of parameter writing and write the parameters again. |
|  |  | y The servo drive is faulty. | y If the fault persists after several times of power-on and parameter initialization, replace the servo drive. |
| Er03.1 | Parameter out-of- range | y The number of software parameters changes after upgrade. An address error occurs during reading and writing the change. | y Check whether the parameter access address is out of range. You can view the group number and offset of the error code in U41-06 and U41-07. y Restore factory settings. |
| Er03.2 | Parameter writing error | y Parameter writing is frequent. y The control power is unreliable. y The servo drive is faulty. | y Check whether the communication program contains a command that frequently modifies and writes parameters. y Check the wiring of the control power and ensure that the control power voltage is within the limit. y If the fault still persists after several times of power-on, replace the servo drive. |
| Er03.3 | Parameter reading error | y Parameter reading is frequent. y The servo drive is faulty. | y Check whether the communication program contains a command that frequently reads parameters. y Modify a parameter, power on the servo drive again, and check whether the modification is saved. y If the modification is not saved and the fault still persists after several times of power-on replace the servo drive. |
| Er05.0 | Current loop timeout | y The interval for MCU torque interruption scheduling is abnormal. | y If the fault persists after several times of power-on, replace the servo drive. |
| Er05.1 | Speed loop timeout | y The interval for MCU speed scheduling is abnormal. | y If the fault persists after several times of power-on, replace the servo drive. |
| Er05.2 | Position loop timeout | y The interval for MCU position interruption scheduling is abnormal. | y If the fault persists after several times of power-on, replace the servo drive. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er06.0 | Runaway protection | y The control circuit is abnormal due to incorrect wiring, resulting in motor runaway and stall. | y Check whether the servo drive power cables are connected to UVW terminals of the motor and servo drive in the correct sequence on both sides. y Connect the U, V, and W phases according to the correct sequence. |
|  |  | y The interference signal causes an error in the initial phase detection of the motor rotor upon power-on. | y The U, V, and W phase sequence is correct, but Er06.0 occurs when the servo drive is enabled. Power on the device again. |
|  |  | y The encoder model is set incorrectly. | y Check the motor model and encoder type. y Use the matching products. |
|  |  | y The encoder cable is connected incorrectly, aging, or corroded, or the encoder connector is loose. | y Check whether the encoder cable is aging, corroded, or loosened. y Re-solder, tighten, or replace the encoder cable. |
|  |  | y The gravity load is too large when the motor controls a vertical axis. | y Check whether the load of the vertical axis is too large. Reduce the load of the vertical axis, increase the rigidity, or shield this fault without affecting safety and use. |
|  |  | y The servo vibration is too large due to improper parameters setting. | y Set the parameters properly to avoid large servo vibration. |
|  |  | y The motor is dragged by an external force in the reverse direction. | y If the motor runs properly and is actually dragged by an external force, consider to shield the protection from out of control (set C06.20 to 0 with caution). |
| Er10.0 | P-hardware overcurrent | y The gain is set improperly and the motor oscillates. | y Adjust the gain after determining the cause. |
|  |  | y The encoder cable is connected incorrectly, aging, or corroded, or the encoder connector is loose. | y Re-solder, tighten, or replace the encoder cable. |
|  |  | y Braking resistor overcurrent | y Select a discharge resistor of proper resistance and model and route it again. |
|  |  | y The servo drive is faulty. | y Replace the servo drive. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er10.1 | N-hardware overcurrent | y The gain is set improperly and the motor oscillates. | y Adjust the gain after determining the cause. |
|  |  | y The encoder cable is connected incorrectly, aging, or corroded, or the encoder connector is loose. | y Re-solder, tighten, or replace the encoder cable. |
|  |  | y Braking resistor overcurrent | y Select a braking resistor of proper resistance and model and route it again. |
|  |  | y The motor U/V/W cables are short circuited. | y Connect motor cables correctly or replace the motor with unbalanced resistance. |
|  |  | y The servo drive is faulty. | y Replace the servo drive. |
| Er10.2 | U phase software overcurrent | y The motor cables are in poor contact. | y Tighten the cables that are loosened or disconnected. |
|  |  | y The motor cables are grounded. | y Replace the motor in the case of poor insulation. |
|  |  | y The motor U/V/W cables are short circuited. | y Unplug the motor cables and check whether short-circuit occurs among U, V, and W phases and whether burrs exist on the cable connections. Unplug the motor cables and measure whether the resistance among U, V, and W phases of the motor cable is balanced. |
|  |  | y The motor is damaged. | y Connect motor cables correctly or replace the motor with unbalanced resistance. |
| Er10.3 | V phase software overcurrent | y The motor cables are in poor contact. | y Tighten the cables that are loosened or disconnected. |
|  |  | y The motor cables are grounded. | y Replace the motor in the case of poor insulation. |
|  |  | y The motor U/V/W cables are short circuited. | y Unplug the motor cables and check whether short-circuit occurs among U, V, and W phases and whether burrs exist on the cable connections. Unplug the motor cables and measure whether the resistance among U, V, and W phases of the motor cable is balanced. |
|  |  | y The motor is damaged. | y Connect motor cables correctly or replace the motor with unbalanced resistance. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er10.4 | Output short circuited to ground | y The servo drive power cables (U/V/W) are short- circuited to ground. | y Re-connect or replace the power cables of the servo drive. |
|  |  | y The motor is short-circuited to ground. | y Replace the motor. |
|  |  | y The servo drive is faulty. | y Replace the servo drive. |
| Er10.5 | Current sampling failure | y Current sampling of phase U or V is abnormal. | y Check for interference sources on the site. y Check the grounding of the servo drive and motor and whether anti-interference measures such as shielding are properly applied. y Add magnetic rings to the power cable and encoder cable of the motor. |
|  |  | y The internal current sampling chip is damaged. | y Replace the servo drive. |
| Er10.6 | Incorrect current parameter setting | y Incorrect setting of current sampling parameters | y If the fault persists upon next power-on, replace the servo drive. |
| Er10.7 | UV current correction failure | y The current correction detection accuracy error is greater than 5%. | y If the fault persists upon next power-on, replace the servo drive. |
| Er10.8 | Excessive current zero drift | y The current zero drift detected upon power on is greater than the threshold. | y If the fault persists upon next power-on, replace the servo drive. |
| Er10.9 | The sampled current during enabling is too large | y The sampled current during enabling is too large. | y If the fault persists after several times of drive enabling, replace the servo drive. |
| Er11.0 | Excessive motor speed upon servo drive power-on | y The motor is rotating when the servo drive is powered on. | y Keep the motor stationary when the servo drive is powered on. |
| Er11.1 | Drive over- temperature | y Drive over-temperature | y Check whether the fan is abnormal or whether the ambient temperature is too high. y Improve the installation conditions of the servo unit to reduce the ambient temperature. y If the fault persists upon next power-on, replace the servo drive. |
| Er20.1 | Encoder internal fault | y The encoder has an internal fault. | y Replace the motor. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er20.2 | Encoder reading/ writing error | y Encoder data exchange exception upon power-on. | y Use a new encoder cable. If the fault no longer occurs after cable replacement, the original encoder cable is damaged. If the fault persists after the encoder cable is replaced, the encoder may be faulty. In this case, replace the servo motor. y Add magnetic rings to the power cable and encoder cable of the motor. |
| Er20.3 | Encoder data frame loss | y The encoder cable is abnormal. | y Replace the encoder cable. |
|  |  | y Intensive interference to the encoder | y Add magnetic rings to the power cable and encoder cable of the motor. y If the fault persists after the servo drive is powered off and on several times, the encoder is faulty. In this case, replace the servo motor. |
| Er20.4 | Excessive encoder incremental position | y Abnormal single-turn position of the encoder | y Route the motor cables and encoder cables through different routes if they are bundled together. y If the fault persists after the servo drive is powered off and on several times, the encoder is faulty. In this case, replace the servo motor. |
| Er20.5 | Abnormal encoder data | y Internal parameters of the encoder are abnormal. | y Route the motor cables and encoder cables through different routes if they are bundled together. y If the fault persists after the servo drive is powered off and on several times, the encoder is faulty. In this case, replace the servo motor. |
| Er20.6 | Mismatch of encoder type | y Motor model mismatch | y Use the motor that matches the drive. |
| Er20.7 | Encoder model not supported | y Encoder model not supported | y Use the motor that matches the drive. |
| Er20.8 | Encoder battery failure | y The encoder battery voltage is too low. | y Replace with a new battery of matching voltage. |
|  |  | y The battery is replaced or no battery is connected during power-off. y C00.07 is set to the absolute value for the first time. | y Set F31-10 to 4 to reset the encoder, and power on the machine again. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er20.9 | Encoder multi-turn error | y An encoder multi-turn counting error occurs. | y Set F31.10 to 4 to reset the encoder, and power on the machine again. y If the fault persists after multiple times of power-on, replace the motor. |
| Er21.0 | Mismatch between encoder pulses per revolution and drive pulses per revolution | y Mismatch between encoder pulses per revolution and drive pulses per revolution | y Distribute parameters again for the encoder. |
| Er31.0 | More than ten PDO mapping objects | y The number of mapping objects in TPDO or RPDO exceeds 10. | y Modify the number of PDO mapping objects to a value smaller than or equal to 10. |
| Er32.0 | EtherCAT peripheral error | y EEPROM or IIC bus error. | y Replace the servo drive. |
| Er32.1 | ESI check error in FLASH | y The XML configuration file is not programmed. | y Check whether the XML version in U42.0B is normal. Program the XML file. |
| Er32.2 | Failure to read data from EEPROM through bus | y Reading EtherCAT data from EEPROM fails. | y If the fault persists after the servo drive is powered off and on several times, replace the servo drive. |
| Er32.3 | Failure of update to EEPROM through bus | y The bus fails to update EtherCAT data to EEPROM. | y If the fault persists after the servo drive is powered off and on several times, replace the servo drive. |
| Er32.4 | Correctness of checksum in ESC configuration area | y An error occurs during XML load check. | y If the fault persists after the servo drive is powered off and on several times, replace the servo drive. |
| Er32.5 | EtherCAT failed to obtain valid XML information | y Failed to load the XML file during EtherCAT communication. | y Check whether the XML version in U42.0B is normal. Program the XML file. Contact the technical support personnel. |
| Er40.0 | Drive overload | y The servo drive overloads. | y Check whether the load rate (U40.07) and current feedback during drive running are too large. If large load is required by the operating conditions, use a servo drive of a higher power. |
| Er41.0 | Motor overload | y The motor and encoder cables are connected improperly or in poor contact. | y Connect the cables according to the correct wiring diagram. y When customized cables are used, prepare and connect the cables according to the wiring instructions. |
|  |  | y The load is too large. The motor keeps outputting effective torque higher than the rated torque for a long time. | y Check whether the average load factor of the servo drive is greater than 100.0% for a long time. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
|  |  | y Acceleration/deceleration is too frequent or the load inertia is too large. | y Use a large-capacity servo drive and a matching motor, or reduce the load and increase the acceleration/deceleration time. y Check the mechanical inertia ratio or perform inertia auto-tuning, and view the value. Check the single running cycle when the servo motor runs cyclically. Increase the acceleration/deceleration time during single-cycle running. |
|  |  | y The gain is improper or the rigidity is too high. | y Check whether the motor vibrates and generates abnormal noise during running. y Adjust the gain again. |
|  |  | y The servo drive or motor model is set incorrectly. | y View the servo drive nameplate, set the servo drive and motor models correctly, and use a matching servo motor. |
|  |  | y Motor locked-rotor occurs due to mechanical factors, resulting in overload during running. | y Eliminate mechanical factors. |
|  |  | y The servo drive is faulty. | y If the fault persists after the servo drive is powered off and on again, replace the servo drive. |
| Er41.1 | Motor over- temperature due to locked-rotor | y Power output (UVW) phase loss, disconnection, or incorrect phase sequence occurs on the servo drive. | y Perform motor trial run without load and check cable connections with a multimeter. Check whether the cable phase sequence is correct. y Connect cables again according to the correct wiring diagram or replace the cables. |
|  |  | y Motor parameters are set incorrectly. | y Read parameters in group R20 and check whether the number of pole pairs is correct. y Auto-tune the motor angle multiple times and check whether the obtained values are consistent. y Correct motor parameters. |
|  |  | y The communication command is interfered. | y Check whether commands from the host controller jitters and eliminate EtherCAT communication interference. |
|  |  | y Motor locked-rotor occurs due to mechanical factors. | y Check for mechanical factors such as locking, occasional jamming, or eccentricity. y If the fault persists after the servo drive is powered off and on several times, contact our company for technical support. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er41.2 | Motor over- temperature | y The PTC temperature sensor of the motor detects motor over-temperature. | y Check for the PTC motor and check whether the PTC cable is connected to the servo drive. y If the servo drive or motor does not support PTC, disable the PTC function (C06-16=0). |
| Er42.2 | Heatsink temperature too high | y The ambient temperature is too high. | y Improve the cooling conditions of the servo drive to reduce the ambient temperature. |
|  |  | y The servo drive is powered off and on for several times to reset the overload fault. | y Change the fault reset method. After overload occurs, wait for 30s before reset. Increase the capacity of the servo drive and motor, increase the acceleration/ deceleration time, and reduce the load. |
|  |  | y The fan is damaged. | y Check whether the fan works when the motor runs. Replace the servo drive. |
|  |  | y The installation direction or clearance of the servo drive is improper. | y Install the servo drive according to the installation requirements. |
|  |  | y The servo drive is faulty. | y If the fault persists even though the servo drive is restarted 5 minutes after power-off, replace the servo drive. |
| Er43.0 | Overvoltage | y The main circuit input voltage is too high. | y Replace or adjust the power supply according to the specifications. |
|  |  | y The power supply is unstable or affected by lightning. | y Monitor whether the power supply of the servo drive is stable, affected by lightning or satisfies the specifications. y Connect an SPD and then switch on the power supplies of the control circuit and the main circuit. If the fault persists, replace the servo drive. |
|  |  | y The braking resistor fails. | y Check the wiring of the braking resistor. Measure the resistance of the external braking resistor between P and C. If the resistance is ∞, the internal cables of the braking resistor are broken. In this case, replace the resistor. y Set the power and resistance of the external braking resistor according to the specifications of the external braking resistor in use. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
|  |  | y The resistance of the external braking resistor is too large, and energy absorption during braking is insufficient. | y Measure the resistance of the external braking resistor between P and C and compare the measured value with the recommended value. Connect a new external braking resistor of recommended resistance. y Set the power and resistance of the external braking resistor according to the specifications of the external braking resistor in use. |
|  |  | y The motor is in abrupt acceleration/deceleration status. The maximum braking energy exceeds the energy absorption value. | y Confirm the acceleration/deceleration time during running and measure whether the DC bus voltage exceeds the fault threshold during deceleration. |
|  |  | y The bus voltage sampling value deviates greatly from the actual measured value. | y Ensure the input voltage of the main circuit is within the specified range, and then increase the acceleration/deceleration time within the allowable range. y Contact our company for technical support. |
|  |  | y The servo drive is faulty. | y If the fault persists after the main circuit is powered off and on several times. y Replace the servo drive. |
| Er43.1 | Undervoltage | y The power supply of the main circuit is unstable or power failure occurs. y Instantaneous power failure occurs. | y Measure whether the input voltages at the main circuit cables and servo drive comply with the specifications. Increase the power capacity. |
|  |  | y The power voltage drops during operation. | y Monitor the power input voltage of the servo drive and check whether the main circuit power supply is applied to other devices, resulting in insufficient power capacity and voltage drop. Increase the power capacity. |
|  |  | y Phase loss: A single-phase power supply is used for a three-phase servo drive. | y Check whether the main circuit wiring is correct and secure. y Replace the cables and connect the main circuit cables properly. |
|  |  | y The servo drive is faulty. | y If the fault persists after the main circuit is powered off and on several times, replace the servo drive. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er45.0 | S-ON enabling failure | y The multiple S-ON enabling methods conflict. | y Do not turn on the S-ON signal simultaneously for multiple control modes (such as servo background and host controller). |
| Er46.0 | Motor overspeed | y The motor cable U, V, and W phase sequence is incorrect. | y Check whether the servo drive power cables are connected to UVW terminals of the motor and servo drive in the correct sequence on both sides. Connect the U, V, and W phases according to the correct sequence. |
|  |  | y The overspeed threshold is set incorrectly. | y Check whether the overspeed threshold is smaller than the actual maximum motor speed. Reset the overspeed threshold according to the mechanical requirements. When C06.03 is set to 0, the overspeed threshold is the maximum speed of the motor. |
|  |  | y The input reference exceeds the overspeed threshold. | y Check whether the motor speed corresponding to the input reference exceeds the overspeed threshold. Set the speed limit to a value smaller than the overspeed threshold. |
|  |  | y The motor speed overshoots. | y Check whether the speed feedback exceeds the overspeed threshold through the commissioning platform. Adjust the gain or mechanical operating conditions. |
|  |  | y The servo drive is faulty. | y If the fault persists after the servo drive is powered off and on, replace the servo drive. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er47.0 | Excessive position deviation | y Power output (UVW) phase loss or incorrect phase sequence occurs on the servo drive. | y Perform motor trial run without load and check cable connections. y Connect cables again according to the correct wiring diagram or replace the cables. |
|  |  | y The servo drive UVW output cable or the encoder cable breaks. | y Check and connect the cables again. Check whether the servo motor power cables (UVW) are in the same phase sequence as the servo drive cables. y Replace all the cables with new cables if necessary and ensure all the cables are connected securely. |
|  |  | y Motor locked-rotor occurs due to mechanical factors. | y Eliminate mechanical factors. |
|  |  | y The servo drive gain is low. | y Adjust the gain manually or perform gain auto-tuning. |
|  |  | y The position reference increment is too large. | y Increase the acceleration/deceleration ramp.Decrease the gear ratio according to the actual conditions. |
|  |  | y The fault value is too small in relative to the operating conditions. | y Check whether the position deviation fault value is set to a too small value. Increase the position deviation alarm threshold (6065h). |
|  |  | y The servo drive or motor is faulty. | y Monitor the operating waveform through the oscilloscope function in the drive commissioning platform: position reference, position feedback, speed reference, and torque reference. If the position reference is not 0, but the position feedback is always 0, replace the servo drive or motor. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er47.1 | Position deviation overflow | y Power output (UVW) phase loss or incorrect phase sequence occurs on the servo drive. | y Perform motor trial run without load and check cable connections. y Connect cables again according to the correct wiring diagram or replace the cables. |
|  |  | y The servo drive UVW output cable or the encoder cable breaks. | y Check and connect the cables again. Check whether the servo motor power cables (UVW) are in the same phase sequence as the servo drive cables. y Replace all the cables with new cables if necessary and ensure all the cables are connected securely. |
|  |  | y Motor locked-rotor occurs due to mechanical factors. | y Eliminate mechanical factors. |
|  |  | y The servo drive gain is low. | y Adjust the gain manually or perform gain auto-tuning. |
|  |  | y The position reference increment is too large. | y Adjust the position reference. Decrease the gear ratio according to the actual conditions. |
|  |  | y The fault value is too small in relative to the operating conditions. | y Check whether the position deviation fault value is set to a too small value. Increase the setpoint. |
|  |  | y The servo drive or motor is faulty. | y Monitor the operating waveform through the oscilloscope function in the drive commissioning platform: position reference, position feedback, speed reference, and torque reference. If the position reference is not 0, but the position feedback is always 0, replace the servo drive or motor. |
| Er50.1 | D/Q current overflow | y A current sampling error occurs. | y If the fault persists after several times of power-on, replace the servo drive. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er51.0 | Offline inertia auto- tuning failure | y Continuous vibration occurs during auto-tuning. The auto-tuning result fluctuates greatly. y The mechanical connection of the load is loose due to offset of the machinery. y An alarm is reported during the auto-tuning process, leading to operation interruption. y Vibration of the load with large inertia cannot be suppressed. The acceleration/deceleration time must be increased to ensure that the motor current is not saturated. | y Enable the vibration suppression function to eliminate vibration if vibration cannot be automatically suppressed. y Troubleshoot and remove the alarm. After that, perform auto-tuning again. y Increase the maximum running speed, decrease the acceleration/deceleration time, and shorten the travel of the screw machinery. |
| Er51.1 | Offline inertia parameter error | y The torque during auto- tuning is too large. | y Decrease the auto-tuning speed (C07.01) and auto-tuning target torque (C07.03), and increase the number of auto-tuning turns (C07.04). |
| Er52.0 | Angle auto-tuning failure | y Angle auto-tuning failure | y Set the motor parameters correctly. y Perform motor wiring again. |
| Er53.0 | Motor parameter auto-tuning timeout | y Motor parameter auto- tuning timeout | y Contact our company for technical support. |
| Er53.1 | Resistance parameter auto-tuning failure | y Resistance parameter auto- tuning failure | y Contact our company for technical support. |
| Er53.2 | Inductance parameter auto- tuning failure | y Inductance parameter auto- tuning failure | y Contact our company for technical support. |
| Er53.3 | Back EMF parameter auto-tuning failure | y Back EMF parameter auto- tuning failure | y Contact our company for technical support. |
| Er54.0 | Current loop auto- tuning failure | y Current loop auto-tuning failure | y Contact our company for technical support. |
| Er55.0 | Excessive vibration | y The vibration is excessive. | y Reset the gain parameters. |
| Er74.0 | EtherCAT synchronization cycle setting error | y After the system switches over to the operation mode, the synchronization cycle is not an integral multiple of 250 μs. | y Change the synchronization cycle of the host controller. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er74.1 | No sync signal | y The communication synchronization clock is configured incorrectly for the master. | y Correct the master communication configuration. |
| Er74.2 | Chip synchronization process uncompleted in OP | y Synchronization interrupt is not detected in several position loop periods of the servo drive. | y If the fault persists after re-power-on, contact us. |
| Er80.0 | Control power undervoltage | y The control power supply is unstable or power failure occurs. | y Check whether the voltage drops during control power cutoff or instantaneous power failure occurs. Power on the device again. If the fault is caused by abnormal power failure, ensure stable power supply. Check whether the input voltage of the control power cables satisfies the specifications. Increase the power capacity. |
|  |  | y The control power cables are in poor contact. | y Check whether control power cables are connected and whether voltage of control power cables on the servo drive side satisfies the specifications. Connect the cables again or replace the cables. |
| Er81.0 | Input phase loss 1 | y Input phase loss | y Check whether the input three-phase AC power supply is normal. If the power supply is normal, replace the drive. |
| Er81.1 | Input phase loss 2 | y Input phase loss | y Check whether the input three-phase AC power supply is normal. If the power supply is normal, replace the drive. |
| Er81.2 | Output phase loss (reserved) | y Output UVW disconnection | y Replace the motor cable. |
| Er82.0 | DI function allocation fault | y One function is allocated to multiple DI terminals. | y Allocate different function numbers to parameters allocated with the same non- zero function number, and turn on the control power supply to make the settings take effect. Or, disable the S-ON signal and then send a reset signal to make the settings take effect. |
|  |  | y The function number set for the DI terminal exceeds the maximum value. | y Check whether the MCU program is updated. Restore default settings (F31.02=1) and power on the system again. |
| Er82.1 | DO function allocation fault | y The function number set for the DO terminal exceeds the maximum value. | y Set the correct DO function number. Restore default settings (F31.02=1) and power on the system again. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er84.0 | Electronic gear ratio setting error | y The electronic gear ratio exceeds the limit. | y Set the electronic gear ratio correctly (0.001, 4000 x Encoder resolution/10000). |
| Er84.1 | Software limit setting error | y The software limit lower limit is greater than or equal to the upper limit. | y Reset the value to make the minimum software absolute position limit is smaller than the maximum one. |
| Er84.2 | Encoder resolution setting error | y The encoder resolution is abnormal. | y Restore default settings (C31.02=1) and power on the system again. |
| Er84.3 | Home position setting error | y The home offset is beyond the software limits. | y When the encoder works in the incremental, absolute linear, or single-turn absolute value mode, set the home offset to be within the software limits. |
|  |  | y The home offset is beyond the limits in rotating mode. | y When the encoder works in the rotating mode, set the home offset to a value between the upper and lower limits of the mechanical single-turn. |
| Er87.1 | One-time excessive position reference increment (one- time increment of the target position is over 5 times of the maximum speed) | y The target position increment is too large. One- time increment of the target position is over 5 times of the maximum speed. | y Check the change of two adjacent target positions through the drive commissioning platform. y Check whether the maximum speed of the motor meets application requirements. If yes, decrease the target position reference increment to lower the planned reference speed. If no, replace the motor. y Before switching the mode or enabling the S-ON signal, align the target position with the current position feedback. y Check the encoder resolution and electronic gear ratio set for the host controller. Check whether the encoder resolution matches the actual motor encoder resolution. y If data received from the slave is incorrect due to communication time sequence error of the host controller, check the communication sequence of the host controller. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| Er87.2 | Continuous excessive position reference increment (increment of the target position exceeds the maximum speed for 3 consecutive times) | y The target position increment is too large. Increment of the target position exceeds the maximum speed for 3 consecutive times. | y Check the change of two adjacent target positions through the drive commissioning platform. y Check whether the maximum speed of the motor meets application requirements. If yes, decrease the target position reference increment to lower the planned reference speed. If no, replace the motor. y Before switching the mode or enabling the S-ON signal, align the target position with the current position feedback. y If data received from the slave is incorrect due to communication time sequence error of the host controller, check the communication sequence of the host controller. |
| Er87.3 | Overflow of 32-bit sign bit of the target position during limiting | y 32-bit sign bit of the target position overflows during limiting. | y The target position reference at the limit is too large. |
| Er87.4 | Target position exceeding maximum value of mechanical single-turn position in rotating mode | y The target position exceeds the single-turn position upper/lower limit in absolute value rotation mode or single-turn rotation mode. | y Set the target position to a value between the single-turn upper and lower limits. |
| ErA0.1 | Multi-turn overflow fault | y The number of turns of the absolute encoder in the forward direction or reverse direction exceeds 32767 or 32768, respectively. | y Set F31-10 to 4 to reset the fault and multi- turn data, and power on the machine again. Perform homing again when necessary. |
| ErC1.0 | Excessive EtherCAT synchronization period error | y The synchronization period error of the controller is too large. | y Increase the value of the manufacturer parameter C13.06. If the fault persists, replace the drive. |
| ErC1.1 | Synchronization loss | y The communication synchronization clock is configured incorrectly for the master. | y Perform the test on another master. Correct the master communication configuration. |
|  |  | y The IN and OUT ports for EtherCAT communication are connected inversely. | y Connect the IN and OUT ports in the correct sequence. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
|  |  | y The controller chip of the slave is damaged. | y If the fault persists after the master is replaced, use an oscilloscope to measure the synchronization signal generated by the slave controller chip. If there is no signal, the slave controller chip is damaged. Return to factory for repair or replace the controller chip of the slave. |
|  |  | y The MCU pin is damaged. | y Use an oscilloscope to measure the synchronization signal generated by the slave controller chip. If there is signal, the MCU chip pin is damaged. Return to factory for repair or replace the MCU chip. |
| ErC1.2 | Network status switchover error | y Master malfunction or manual malfunction occurs. | y Check the network status switchover program of the host controller. y Use a shielded twisted-pair communication cable. y Ground the servo drive according to the standard. y Check the network connection status according to the LED. |
| ErC1.4 | Network cable connection unreliable | y The physical connection of the data link is unstable, or the process data is lost due to network cable connection and removal. | y Check whether the network cable connection of the drive is reliable and firm, and whether there is severe vibration on site. |
| ErC1.5 | Data frame loss protection error | y Data loss occurs due to EMC interference, poor network cable quality, or poor connection. | y Ensure reliable grounding and correct EMC. y Check whether the network cable is the one specified by STEPPERONLINE. y Check whether the network cable connection is reliable. |
| ErC1.6 | Data frame forwarding error | y An upper station has detected and marked that the data frame has been damaged. When the data frame is forwarded to the current slave, an alarm is reported. | y Ensure reliable grounding and correct EMC. y Check whether the network cable is the one specified by STEPPERONLINE.. y Check whether the network cable connection is reliable. |
| ErC1.7 | Data update timeout | y The data frame has been lost or discarded at an upper station or the master performance is poor. | y Check whether the load of the master CPU is excessive. Change the sync signal offset value. |
| ErC1.8 | Watchdog expired | y The master configuration is incorrect. | y Modify the watchdog configuration of the host controller. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| ErC2.0 | SYNC signal loss | y The physical connection of the data link is unstable, or the process data is lost due to network cable connection and removal. | y Replace the network cable with a more reliable one. If the fault persists, contact technical support. |
| ALF0.0 | Emergency stop alarm | y Check whether the logic of the DI allocated with function 4 (Emergency stop) is valid. | y Check the operation mode and clear DI emergency stop valid signal when safety is guaranteed. |
| ALF1.0 | Re-power-on required for parameter settings to take effect | y Modifications of some servo drive parameters take effect only after the servo drive is powered on again. After these parameters are modified, the servo drive reminds users to restart it. | y Power on the device again. |
| ALF1.2 | Torque reached parameter error | y In torque control mode, the DO setting for torque reach is invalid. | y Set the torque output when torque reached DO signal turned on to be greater than torque output when torque reached DO signal turned off. Set C03.4A to be greater than C03.4B. |
| ALF1.3 | Too frequent writing of EEPROM by host controller SDO | y A large number of parameters are modified and saved frequently. | y Do not frequently write parameters into cloud platform EEPROM on the host controller. |
| ALF2.0 | Forward overtravel alarm | y The positive limit (PL) DI is active. | y Run the motor in the reverse direction to the restricted range. |
|  |  | y The drive position feedback is at the forward software position limit. | y Run the motor in the reverse direction to the re-stricted range or increase the positive software position limit. |
|  |  | y The home offset setting exceeds the software position limit. | y Set the home offset within the software position limit range. |
| ALF2.1 | Reverse overtravel alarm | y The negative limit (NL) DI is active. | y Run the motor in the forward direction to the restricted range. |
|  |  | y The drive position feedback is at the reverse software position limit. | y Run the motor in the forward direction to the restricted range or decrease the positive software position limit. |
|  |  | y The home offset setting exceeds the software position limit. | y Set the home offset within the software position limit range. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| ALF4.0 | Homing timeout | y The homing time exceeds the setpoint. | y Appropriately adjust the homing speed and homing time, and ensure that the external home signal connection is reliable (if used). |
| ALF4.1 | Homing DI conflict | y During homing, both the forward and reverse limits are valid or both the home signal and limit signal are valid. | y Check whether the home signal and limit signal are correct. |
| ALF4.2 | Homing mode conflict | y The homing mode is set incorrectly. | y Check whether the homing mode set in object dictionary 6098h on the host controller is correct. |
| ALF5.0 | Braking resistor overload | y The cable connected to the external braking resistor is in poor contact, disconnected, or broken. | y Connect the external braking resistor between P and C with a new cable. y Replace the external braking resistor with a new one. Ensure that the resistance measured is the same as the nominal value, and then connect the resistor between P and C. |
|  |  | y The resistance of the external braking resistor used is too large. | y Select a resistor with a proper resistance according to the specification requirements. |
|  |  | y The resistance setpoint is greater than the resistance of the external braking resistor used. | y Set the value according to the resistance of the external bleeder resistor in use. |
|  |  | y The input voltage of the main circuit is beyond the specification. | y Replace or adjust the power supply according to the specifications. |
|  |  | y The load moment of inertia ratio is too large. | y Select an external braking resistor with large capacity and set its resistance to the actual one. |
|  |  | y The motor speed is too high and the deceleration process is not completed within the set deceleration time. The motor is in continuous deceleration state during cyclic running. | y Select a servo drive with a large capacity. |
|  |  | y The capacity of the servo drive or the braking resistor is insufficient. | y Reduce the load if allowed. y Increase the acceleration/deceleration time if allowed. y Increase the motor running cycle if allowed. |

| Code | Name | Cause | Solution |
| --- | --- | --- | --- |
| ALF5.1 | External braking resistance too small | y The resistance of the external braking resistor is smaller than permissible minimum resistance by the servo drive. | y Replace with an external braking resistor matching the servo drive, with the resistance greater than the minimum value, and set C00.10 to the resistance value. |
| ALF6.1 | Output phase loss | y The output current is abnormal. | y Check whether the power cable is broken. If so, replace the cable. |
| ALF8.0 | Vibration occurred during auto-tuning | y Continuous vibration occurs during auto-tuning. y The mechanical connection of the load is loose due to offset of the machinery. | y Check the mechanical installation clearance and connection reliability. |
|  |  | y Vibration of the load with large inertia cannot be suppressed. The acceleration/deceleration time must be increased to ensure that the mo-tor current is not saturated. | y Appropriately adjust the inertia auto-tuning parameters (C07.00, C07.01, C07.03, and C07.04), reduce the values of the auto- tuning speed (C07.01) and auto-tuning target torque (C07.03), and increase the number of auto-tuning turns (C07.04). |
| ALF9.0 | Encoder battery voltage low | y The encoder battery voltage is too low. | y Replace the encoder battery. |
| ALFA.0 | Drive high temperature warning | y Drive high temperature warning | y Check whether the fan is abnormal or whether the ambient temperature is too high. y Improve the installation conditions of the servo unit to reduce the ambient temperature. |
| xxnr | Servo not ready | y The voltage of the control power is too low. | y Check the U40.35 parameter for the control bus voltage, and ensure that the power supply is normal. |
|  |  | y The main circuit voltage is too low. | y Check the U40.36 parameter for the primary bus voltage, and ensure that the power supply is normal. |
|  |  | y The input AC signal is abnormal. | y Check the input AC power supply and the three- phase AC main power supply, and ensure that the power supply is normal. |
|  |  | y The encoder battery voltage is too low. | y Measure the voltage of the encoder battery. If the voltage is less than 2.9 V, replace the battery. |

Chapter 11	Parameter List

11.1 Parameter Group Description

Parameter access address: index+subindex, both in hexadecimal format

The CiA402 protocol has the following constraints on the address of system parameters.

| Index | Description |
| --- | --- |
| 0001h—0FFFh | Data type description |
| 1000h—1FFFh | CoE communication object |
| 2000h—5FFFh | Manufacturer specific object |
| 6000h—9FFFh | Sub-protocol object |
| A000h—FFFFh | Reserved |

11.2 Parameter List

    - Common Parameters in Group 2000h

Parameters (2000h/C00)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C00.00 | Control mode | 10: EtherCAT | 0-10 | 10 | - | U16 | At stop | Immedi- ately |
| 02h | C00.01 | Motor rotating direction | 0: CCW 1: CW | 0-1 | 0 | - | U16 | At stop | Upon re- power-on |
| 05h | C00.04 | Auto-tuning mode | 0: Manual mode 1: Standard mode 2: Positioning mode | 0-2 | 1 | - | U16 | During operation | Immedi- ately |
| 06h | C00.05 | Stiffness level | - | 1-31 | 12 | - | U16 | During operation | Immedi- ately |
| 07h | C00.06 | Load inertia ratio | - | 0-12000 | 100 | % | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08h | C00.07 | Absolute mode | 0: Incremental position mode 1: Absolute position linear mode 2: Absolute position linear infinite mode 3: Absolute position single- turn mode 4: Absolute position rotation mode 5: Absolute mechanical single- turn mode (operating direction selectable) | 0-5 | 0 | - | U16 | At stop | Upon re- power-on |
| 11h | C00.10 | Bleeder resistor selection | 0: Internal bleeder resistor 1: External bleeder resistor 2: No bleeder resistor 3: Capacitor bleeder resistor | 0-3 | 0 | - | U16 | At stop | Immedi- ately |
| 12h | C00.11 | Bleeder resistor power | - | 1-65535 | 50 | W | U16 | At stop | Immedi- ately |
| 13h | C00.12 | Bleeder resistor resistance | - | 1-65535 | 50 | Ω | U16 | At stop | Immedi- ately |
| 14h | C00.13 | Bleeder resistor heat dissipation coeffi-cient | - | 1-100 | 30 | - | U16 | During operation | Immedi- ately |
| 15h | C00.14 | Brake enable switch | - | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 17h | C00.16 | Panel display | 0: Default display 1: Speed display 2: Torque display 3: Voltage display 4: Load rate display | 0-4 | 0 | - | U16 | During operation | Immedi- ately |
| 32h | C00.31 | Super user | - | 0-65535 | 0 | - | U16 | During operation | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.1 "Group C00".

Basic Gain Parameters (2001h/C01)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C01.00 | 1st position loop gain | - | 0-20000 | 400 | 0.1rad/ s | U16 | During operation | Immedi- ately |
| 02h | C01.01 | 1st speed loop gain | - | 1-20000 | 250 | 0.1Hz | U16 | During operation | Immedi- ately |
| 03h | C01.02 | 1st speed loop integral time parameter | - | 1-51200 | 3184 | 0.01ms | U16 | During operation | Immedi- ately |
| 04h | C01.03 | 1st torque reference filter cutoff frequency | - | 5-16000 | 200 | Hz | U16 | During operation | Immedi- ately |
| 09h | C01.08 | 2nd position loop gain | - | 0-20000 | 560 | 0.1rad/ s | U16 | During operation | Immedi- ately |
| 0Ah | C01.09 | 2nd speed loop gain | - | 1-20000 | 350 | 0.1Hz | U16 | During operation | Immedi- ately |
| 0Bh | C01.0A | 2nd speed loop integral time parameter | - | 1-51200 | 2274 | 0.01ms | U16 | During operation | Immedi- ately |
| 0Ch | C01.0B | 2nd torque reference filter cutoff frequency | - | 5-16000 | 280 | Hz | U16 | During operation | Immedi- ately |
| 11h | C01.10 | Speed feedback filter | 0: Internal setting 1: Low-pass filter 2: Overlapping average filter 3: Speed observer 4: No filter | 0-4 | 0 | - | U16 | At stop | Immedi- ately |
| 12h | C01.11 | Cutoff frequency of speed feedback low-pass filter | - | 10-16000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 13h | C01.12 | Speed feedback overlapping average filter time constant | 0: No filter 1: 2 times filter 2: 4 times filter 3: 8 times filter 4: 16 times filter 5: 32 times filter 6: 64 times filter | 0-6 | 0 | - | U16 | During operation | Immedi- ately |
| 14h | C01.13 | Speed feedforward source | 0: No feedforward 1: Internal reference 2: Model tracking 5: Communication | 0-5 | 0 | - | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15h | C01.14 | Speed feedforward percentage | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 16h | C01.15 | Speed feedforward filter cutoff frequency | - | 5-16000 | 318 | Hz | U16 | During operation | Immedi- ately |
| 17h | C01.16 | Torque feedforward source | 0: No feedforward 1: Internal reference 2: Model tracking 5: Communication | 0-5 | 0 | - | U16 | During operation | Immedi- ately |
| 18h | C01.17 | Torque feedforward percentage | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 19h | C01.18 | Torque feedforward filter cutoff frequency | - | 5-16000 | 318 | Hz | U16 | During operation | Immedi- ately |
| 1Ch | C01.1B | PDFF control coefficient | - | 0-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 1Dh | C01.1C | Damping factor control coefficient | - | 0-1000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 21h | C01.20 | Position reference overlapping average filter time constant A | - | 0-1280 | 0 | 0.1ms | U16 | At stop | Immedi- ately |
| 22h | C01.21 | Position reference overlapping average filter time constant B | - | 0-1280 | 0 | 0.1ms | U16 | At stop | Immedi- ately |
| 23h | C01.22 | Position reference low-pass filter time constant A | - | 0-65535 | 0 | 0.1ms | U16 | At stop | Immedi- ately |
| 24h | C01.23 | Position reference low-pass filter time constant B | - | 0-65535 | 0 | 0.1ms | U16 | At stop | Immedi- ately |
| 25h | C01.24 | 1st notch filter frequency of position reference | - | 0-2000 | 0 | 0.1Hz | U16 | At stop | Immedi- ately |
| 26h | C01.25 | 1st notch filter width of position reference | - | 0-1000 | 0 | 0.1% | U16 | At stop | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27h | C01.26 | 1st notch filter depth of position reference | - | 10-1000 | 1000 | 0.1% | U16 | At stop | Immedi- ately |
| 28h | C01.27 | 2nd notch filter frequency of position reference | - | 0-2000 | 0 | 0.1Hz | U16 | At stop | Immedi- ately |
| 29h | C01.28 | 2nd notch filter width of position reference | - | 0-1000 | 0 | 0.1% | U16 | At stop | Immedi- ately |
| 2Ah | C01.29 | 2nd notch filter depth of position reference | - | 10-1000 | 1000 | 0.1% | U16 | At stop | Immedi- ately |
| 2Bh | C01.2A | Position reference pre-charge filter time constant | - | 0-1280 | 0 | 0.1ms | U16 | At stop | Immedi- ately |
| 31h | C01.30 | Adaptive notch mode | 0: Disabled 1: 1st notch 2: 2nd notch 3: Notch parameter reset 4: Resonance frequency tested only | 0-4 | 0 | - | U16 | During operation | Immedi- ately |
| 32h | C01.31 | Adaptive notch test times | - | 0-65535 | 0 | Times | U16 | At stop | Immedi- ately |
| 39h | C01.38 | Gain switchover mode | 0: Fixed to the 1st gain set 1: DI switchover 2: DI P-PI switchover 3: Torque reference 4: Speed reference 5: Speed feedback 6: Speed reference change rate 7: Position deviation 8: Position reference | 0-8 | 0 | - | U16 | At stop | Immedi- ately |
| 3Ah | C01.39 | Gain switchover time | - | 10-10000 | 50 | 0.1ms | U16 | During operation | Immedi- ately |
| 3Bh | C01.3A | Gain switchover threshold | - | 0-65535 | 10 | - | U16 | During operation | Immedi- ately |
| 3Ch | C01.3B | Gain switchover loop width | - | 0-65535 | 10 | - | U16 | During operation | Immedi- ately |
| 41h | C01.40 | Frequency of the 1st notch | - | 10-8000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 42h | C01.41 | Width level of the 1st notch | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43h | C01.42 | Depth level of the 1st notch | - | 10-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 44h | C01.43 | Frequency of the 2nd notch | - | 10-8000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 45h | C01.44 | Width level of the 2nd notch | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 46h | C01.45 | Depth level of the 2nd notch | - | 10-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 47h | C01.46 | Frequency of the 3rd notch | - | 10-8000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 48h | C01.47 | Width level of the 3rd notch | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 49h | C01.48 | Depth level of the 3rd notch | - | 10-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 4Ah | C01.49 | Frequency of the 4th notch | - | 10-8000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 4Bh | C01.4A | Width level of the 4th notch | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 4Ch | C01.4B | Depth level of the 4th notch | - | 10-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 4Dh | C01.4C | Frequency of the 5th notch | - | 10-8000 | 8000 | Hz | U16 | During operation | Immedi- ately |
| 4Eh | C01.4D | Width level of the 5th notch | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 4Fh | C01.4E | Depth level of the 5th notch | - | 10-1000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.2 "Group C01".

Advanced Gain Parameters (2002h/C02)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C02.00 | Model tracking control | 0: Disabled 1: Single mass model tracking | 0-1 | 0 | - | U16 | At stop | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02h | C02.01 | Model tracking control gain | - | 10-20000 | 500 | 0.1rad/ s | U16 | During operation | Immedi- ately |
| 03h | C02.02 | Model tracking inertia correction coefficient | - | 10-8000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 31h | C02.30 | Speed observer gain | - | 0-40000 | 0 | 0.1Hz | U16 | During operation | Immedi- ately |
| 32h | C02.31 | Speed observer inertia correction | - | 10-8000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 33h | C02.32 | Speed observer speed feedback cutoff frequency | - | 0-16000 | 0 | Hz | U16 | During operation | Immedi- ately |
| 39h | C02.38 | Frequency for vibration suppression 1 | - | 10-20000 | 1000 | 0.1Hz | U16 | During operation | Immedi- ately |
| 3Ah | C02.39 | Inertia correction for vibration suppression 1 | - | 10-8000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 3Bh | C02.3A | Low-pass filter correction for vibration suppression 1 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 3Ch | C02.3B | Correction of high-pass filter 1 for vibration sup- pression 1 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 3Dh | C02.3C | Frequency of high- pass filter 2 for vibration sup- pression 1 | - | 10-50000 | 20000 | 0.1Hz | U16 | During operation | Immedi- ately |
| 3Eh | C02.3D | Ratio of compensation 1 for vibration suppression 1 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 3Fh | C02.3E | Ratio of compensation 2 for vibration suppression 1 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 41h | C02.40 | Frequency for vibration suppression 2 | - | 10-20000 | 1000 | 0.1Hz | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42h | C02.41 | Inertia correction for vibration suppression 2 | - | 10-8000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 43h | C02.42 | Low-pass filter correction for vibration suppression 2 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 44h | C02.43 | Correction of high-pass filter 1 for vibration sup- pression 2 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 45h | C02.44 | Frequency of high- pass filter 2 for vibration sup- pression 2 | - | 10-50000 | 20000 | 0.1Hz | U16 | During operation | Immedi- ately |
| 46h | C02.45 | Ratio of compensation 1 for vibration suppression 2 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 47h | C02.46 | Ratio of compensation 2 for vibration suppression 2 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 49h | C02.48 | Frequency for vibration suppression 3 | - | 10-20000 | 1000 | 0.1Hz | U16 | During operation | Immedi- ately |
| 4Ah | C02.49 | Inertia correction for vibration suppression 3 | - | 10-8000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 4Bh | C02.4A | Low-pass filter correction for vibration suppression 3 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 4Ch | C02.4B | Correction of high-pass filter 1 for vibration sup- pression 3 | - | -9999-9999 | 0 | 0.1Hz | I16 | During operation | Immedi- ately |
| 4Dh | C02.4C | Frequency of high- pass filter 2 for vibration sup- pression 3 | - | 10-50000 | 20000 | 0.1Hz | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4Eh | C02.4D | Ratio of compensation 1 for vibration suppression 3 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 4Fh | C02.4E | Ratio of compensation 2 for vibration suppression 3 | - | 0-20000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 61h | C02.60 | Disturbance observer gain | - | 0-40000 | 0 | 0.1Hz | U16 | During operation | Immedi- ately |
| 62h | C02.61 | Disturbance observer inertia correction coefficient | - | 1-10000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 63h | C02.62 | Disturbance observer low-pass cutoff frequency | - | 0-16000 | 0 | Hz | U16 | During operation | Immedi- ately |
| 64h | C02.63 | Disturbance observer compensation torque per- centage | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 69h | C02.68 | Friction compensation switch and relevant setting | - | 0-255 | 0 | - | U16 | During operation | Immedi- ately |
| 6Ah | C02.69 | Friction compensation speed threshold | - | 0-5000 | 20 | 0.1rpm | U16 | During operation | Immedi- ately |
| 6Bh | C02.6A | Static friction compensation | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 6Ch | C02.6B | Forward friction compensation of coulomb friction | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 6Dh | C02.6C | Reverse friction compensation of coulomb friction | - | -2000-0 | 0 | 0.1% | I16 | During operation | Immedi- ately |
| 6Eh | C02.6D | Viscous friction torque for rated speed | - | 0-2000 | 0 | 0.1% | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6Fh | C02.6E | Friction compensation filter time | - | 0-65535 | 0 | 0.01ms | U16 | During operation | Immedi- ately |
| 70h | C02.6F | Friction compensation threshold for zero speed | - | 0-1000 | 10 | 0.1rpm | U16 | During operation | Immedi- ately |

Instruction Parameters (2003h/C03)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22h | C03.21 | Speed reference | - | -8000-8000 | 100 | rpm | I16 | During operation | Immedi- ately |
| 23h | C03.22 | Acceleration rate | - | 0-3600000 | 10 | ms | U32 | During operation | Immedi- ately |
| 25h | C03.24 | Deceleration rate | - | 0-3600000 | 10 | ms | U32 | During operation | Immedi- ately |
| 28h | C03.27 | Internal positive speed limit | - | 0-8000 | 6000 | rpm | U16 | During operation | Immedi- ately |
| 29h | C03.28 | Internal negative speed limit | - | 0-8000 | 6000 | rpm | U16 | During operation | Immedi- ately |
| 2Ch | C03.2B | Speed reach threshold | - | 0-8000 | 1000 | rpm | U16 | During operation | Immedi- ately |
| 2Dh | C03.2C | Speed synchronization threshold | - | 0-1000 | 10 | rpm | U16 | During operation | Immedi- ately |
| 2Eh | C03.2D | Speed rotation threshold | - | 0-1000 | 20 | rpm | U16 | During operation | Immedi- ately |
| 2Fh | C03.2E | Zero speed output threshold | - | 0-1000 | 10 | rpm | U16 | During operation | Immedi- ately |
| 42h | C03.41 | Torque reference | - | -4000-4000 | 0 | 0.1% | I16 | During operation | Immedi- ately |
| 44h | C03.43 | Internal positive torque limit | - | 0-4000 | 3000 | 0.1% | U16 | During operation | Immedi- ately |
| 45h | C03.44 | Internal negative torque limit | - | 0-4000 | 3000 | 0.1% | U16 | During operation | Immedi- ately |
| 48h | C03.47 | Positive speed limit in torque mode | - | 0-8000 | 3000 | rpm | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49h | C03.48 | Negative speed limit in torque mode | - | 0-8000 | 3000 | rpm | U16 | During operation | Immedi- ately |
| 4Ah | C03.49 | Reference value for torque reach | - | 0-4000 | 0 | 0.1% | U16 | During operation | Immedi- ately |
| 4Bh | C03.4A | Valid value for torque reached | - | 0-4000 | 200 | 0.1% | U16 | During operation | Immedi- ately |
| 4Ch | C03.4B | Invalid value for torque reached | - | 0-4000 | 100 | 0.1% | U16 | During operation | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.3 "Group C03".

I/O Parameters (2004h/C04)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C04.00 | DI1 function selection | 0: No definition 1: S-ON 2: Fault reset 4: Emergency stop 5: Home switch 6: Forward overtravel 7: Reverse overtravel 30: Probe 1 31: Probe 2 | 0-32 | 6 | - | U16 | At stop | Immedi- ately |
| 02h | C04.01 | DI1 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 03h | C04.02 | DI1 filter time | - | 0-65535 | 150 | 0.01ms | U16 | During operation | Immedi- ately |
| 05h | C04.04 | DI2 function selection | 0: No definition 1: S-ON 2: Fault reset 4: Emergency stop 5: Home switch 6: Forward overtravel 7: Reverse overtravel 30: Probe 1 31: Probe 2 | 0-32 | 7 | - | U16 | At stop | Immedi- ately |
| 06h | C04.05 | DI2 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07h | C04.06 | DI2 filter time | - | 0-65535 | 150 | 0.01ms | U16 | During operation | Immedi- ately |
| 09h | C04.08 | DI3 function selection | 0: No definition 1: S-ON 2: Fault reset 4: Emergency stop 5: Home switch 6: Forward overtravel 7: Reverse overtravel 30: Probe 1 31: Probe 2 | 0-32 | 5 | - | U16 | At stop | Immedi- ately |
| 0Ah | C04.09 | DI3 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 0Bh | C04.0A | DI3 filter time | - | 0-65535 | 150 | 0.01ms | U16 | During operation | Immedi- ately |
| 0Dh | C04.0C | DI4 function selection | 0: No definition 1: S-ON 2: Fault reset 4: Emergency stop 5: Home switch 6: Forward overtravel 7: Reverse overtravel 30: Probe 1 31: Probe 2 | 0-32 | 31 | - | U16 | At stop | Immedi- ately |
| 0Eh | C04.0D | DI4 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 0Fh | C04.0E | DI4 filter time | - | 0-65535 | 150 | 0.01ms | U16 | During operation | Immedi- ately |
| 11h | C04.10 | DI5 function selection | 0: No definition 1: S-ON 2: Fault reset 4: Emergency stop 5: Home switch 6: Forward overtravel 7: Reverse overtravel 30: Probe 1 31: Probe 2 | 0-32 | 30 | - | U16 | At stop | Immedi- ately |
| 12h | C04.11 | DI5 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 13h | C04.12 | DI5 filter time | - | 0-65535 | 150 | 0.01ms | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31h | C04.30 | DO1 function selection | 0: No definition 1: Servo ready 2: Motor rotation 3: Brake 4: Fault 5: Alarm 7: Position reach 17: Torque reached 18: EMD output | 0-20 | 1 | - | U16 | At stop | Immedi- ately |
| 32h | C04.31 | DO1 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 33h | C04.32 | DO2 function selection | 0: No definition 1: Servo ready 2: Motor rotation 3: Brake 4: Fault 5: Alarm 7: Position reach 17: Torque reached 18: EMD output | 0-20 | 4 | - | U16 | At stop | Immedi- ately |
| 34h | C04.33 | DO2 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |
| 35h | C04.34 | DO3 function selection | 0: No definition 1: Servo ready 2: Motor rotation 3: Brake 4: Fault 5: Alarm 7: Position reach 17: Torque reached 18: EMD output | 0-20 | 3 | - | U16 | At stop | Immedi- ately |
| 36h | C04.35 | DO3 logic selection | 0: Active low 1: Active high | 0-1 | 0 | - | U16 | During operation | Immedi- ately |

Stop Mode (2005h/C05)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 03h | C05.02 | Stop mode at overtravel | 0: Coast to stop, keeping de-energized status 1: Stop at zero speed, keeping position lock status 2: Stop at zero speed, keeping de-energized status 3: Ramp to stop as defined by 6085h, keeping de- energized status 4: Ramp to stop as defined by 6085h, keeping position lock status 5: Dynamic braking stop, keeping de-energized status 6: Dynamic braking stop, keeping dynamic braking status 7: Not responding to overtravel | 0-7 | 1 | - | U16 | At stop | Immedi- ately |
| 04h | C05.03 | Stop mode at No. 1 fault | 0: Coast to stop, keeping de-energized status 1: Dynamic braking stop, keeping de-energized status 2: Dynamic braking stop, keeping dynamic braking status | 0-2 | 2 | - | U16 | At stop | Immedi- ately |
| 0Dh | C05.0C | Limit for stop at emergency-stop torque | - | 0-3000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 0Eh | C05.0D | Maximum downtime | - | 0-65535 | 10000 | ms | U16 | At stop | Immedi- ately |
| 11h | C05.10 | Delay from brake close to motor de-energized | - | 0-65535 | 100 | ms | U16 | During operation | Immedi- ately |
| 12h | C05.11 | Speed threshold at brake closing | - | 10-3000 | 30 | rpm | U16 | During operation | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13h | C05.12 | Maximum waiting time with S-ON off at brake closing | - | 0-65535 | 100 | ms | U16 | During operation | Immedi- ately |
| 14h | C05.13 | Delay from brake on to command received | - | 0-65535 | 100 | ms | U16 | At stop | Immedi- ately |
| 15h | C05.14 | Energizing delay of DB relay | - | 0-65535 | 20 | ms | U16 | At stop | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.4 "Group C05".

Protection Parameters (2006h/C06)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 04h | C06.03 | Threshold of excessive speed | - | 0-9000 | 0 | rpm | U16 | During operation | Immedi- ately |
| 05h | C06.04 | Input phase loss detection | 0: Enabled 1: Disabled | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 06h | C06.05 | Retentive at power failure | 0: Non-retentive 1: Retentive | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 08h | C06.07 | Mechanical limit position | 0: Inactive 1: Enabled 2: Enabled after homing | 0-2 | 0 | - | U16 | During operation | Immedi- ately |
| 09h | C06.08 | Mechanical PL | - | ﹣231- (231-1) | 231-1 | Unit in application | I32 | During operation | Immedi- ately |
| 0Bh | C06.0A | Mechanical NL | - | ﹣231- (231-1) | ﹣231 | Unit in application | I32 | During operation | Immedi- ately |
| 11h | C06.10 | Drive overload protection threshold | - | 0-3500 | 1150 | 0.1% | U16 | At stop | Immedi- ately |
| 12h | C06.11 | Motor overload protection threshold | - | 0-3500 | 1150 | 0.1% | U16 | At stop | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13h | C06.12 | Motor locked- rotor detection | 0: Inactive 1: Enabled | 0-1 | 1 | - | U16 | At stop | Immedi- ately |
| 14h | C06.13 | Motor locked- rotor detection time | - | 0-3000 | 200 | ms | U16 | At stop | Immedi- ately |
| 15h | C06.14 | Motor locked- rotor detection speed | - | 0-1000 | 10 | rpm | U16 | During operation | Immedi- ately |
| 16h | C06.15 | Output phase loss detection | 0: Inactive 1: Enabled | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 1Dh | C06.1C | Encoder communication fault tolerance threshold | - | 0-88 | 3 | - | U16 | At stop | Immedi- ately |
| 21h | C06.20 | Protection from out of control | 0: Inactive 1: Enabled | 0-1 | 1 | - | U16 | At stop | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.5 "Group C06".

Auto-tuning Parameters (2007h/C07)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C07.00 | Offline inertia auto- tuning mode setting | - | 0-785 | 769 | - | U16 | At stop | Immedi- ately |
| 02h | C07.01 | Offline inertia auto-tuning speed reference | - | 50-1000 | 500 | rpm | U16 | At stop | Immedi- ately |
| 03h | C07.02 | Acceleration/ Deceleration time for offline inertia auto- tuning | - | 0-65535 | 100 | ms | U16 | At stop | Immedi- ately |
| 04h | C07.03 | Offline inertia auto- tuning target torque | - | 1-1500 | 150 | 0.1% | U16 | At stop | Immedi- ately |
| 05h | C07.04 | Offline inertia auto- tuning revolutions | - | 10-65535 | 200 | 0.01r | U16 | At stop | Immedi- ately |

Communication Parameters (200Ah/C0A)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09h | C0A.08 | Commissioning software communication station ID | - | 1-255 | 1 | - | U16 | At stop | Immedi- ately |
| 0Ah | C0A.09 | Commissioning software communication baud rate | 0: 1200bps 1: 2400bps 2: 4800bps 3: 9600bps 4: 19200bps 5: 38400bps 6: 57600bps 7: 115200bps | 0-7 | 7 | - | U16 | At stop | Upon re- power-on |
| 0Bh | C0A.0A | Commissioning software communication format | 0: No parity, 1 stop bit 1: Odd parity, 1 stop bit 2: Even parity, 1 stop bit 3: No parity, 2 stop bits 4: Odd parity, 2 stop bits 5: Even parity, 2 stop bits | 0-5 | 0 | - | U16 | At stop | Upon re- power-on |
| 0Ch | C0A.0B | Commissioning software communication response time | - | 1-1000 | 1 | ms | U16 | At stop | Immedi- ately |
| 0Dh | C0A.0C | Commissioning software communication timeout | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Eh | C0A.0D | Commissioning software communication storage | 0: No storage 1: Storage | 0-1 | 1 | - | U16 | At stop | Immedi- ately |
| 0Fh | C0A.0E | Commissioning software data format | 0: Low 16 bits before high 16 bits 1: High 16 bits before low 16 bits | 0-1 | 0 | - | U16 | At stop | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.6 "Group C0A".

Homing Touch Probe Parameters (2010h/C10)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C10.00 | Homing enable | 0: Inactive 1: Written through communication 2: DI trigger 3: Current position as home | 0-3 | 0 | - | U16 | During operation | Immedi- ately |
| 09h | C10.08 | Homing timeout interval | - | 0-(232-1) | 60000 | ms | U32 | During operation | Immedi- ately |
| 11h | C10.10 | Multi-turn absolute position offset (low 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | At stop | Upon re- power- on |
| 13h | C10.12 | Multi-turn absolute position offset (high 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | At stop | Upon re- power- on |
| 15h | C10.14 | Multi-turn revolutions data offset | - | 0-65535 | 0 | Rev | U16 | Read only | Immedi- ately |
| 16h | C10.15 | Multi-turn overflow flag | - | 0-1 | 0 | - | U16 | Read only | Immedi- ately |
| 17h | C10.16 | Reference running mode in rotation mode | 0: Nearest 1: Always in forward direction 2: Always in reverse direction 3: Always in current direction 4: Not specified | 0-4 | 0 | - | U16 | At stop | Immedi- ately |
| 19h | C10.18 | Numerator of electronic gear ratio in rotation mode | - | 1-65535 | 1 | - | U16 | At stop | Immedi- ately |
| 1Ah | C10.19 | Denominator of electronic gear ratio in rotation mode | - | 1-65535 | 1 | - | U16 | At stop | Immedi- ately |
| 1Bh | C10.1A | Upper limit of mechanical absolute position in ro-tation mode (low 32 bits) | - | 0-(232-1) | 0 | P | U32 | At stop | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Dh | C10.1C | Upper limit of mechanical absolute position in ro-tation mode (high 32 bits) | - | 0-(232-1) | 0 | P | U32 | At stop | Immedi- ately |
| 1Fh | C10.1E | Single-turn homing absolute value offset | - | ﹣231- (231-1) | 0 | Unit in application | I32 | At stop | Upon re- power- on |
| 31h | C10.30 | Torque limit of homing upon hit- and-stop | - | 0-3000 | 1000 | 0.1% | U16 | During operation | Immedi- ately |
| 32h | C10.31 | Speed for homing upon hit-and-stop | - | 0-1000 | 10 | rpm | U16 | During operation | Immedi- ately |
| 33h | C10.32 | Number of times for homing upon hit- and-stop | - | 0-65535 | 30 | - | U16 | During operation | Immedi- ately |

EtherCAT Parameters (2013h/C13)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | C13.00 | EtherCAT slave name | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 02h | C13.01 | EtherCAT slave alias | - | 0-65535 | 0 | - | U16 | During operation | Immedi- ately |
| 03h | C13.02 | EtherCAT sync loss threshold | - | 1-20 | 8 | - | U16 | During operation | Immedi- ately |
| 04h | C13.03 | EtherCAT synchronization detection mode | - | 0-65535 | 0 | - | U16 | At stop | Immedi- ately |
| 05h | C13.04 | EtherCAT sync loss count | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 06h | C13.05 | EtherCAT synchronization mode setting | - | 0-2 | 1 | - | U16 | During operation | Immedi- ately |
| 07h | C13.06 | EtherCAT synchronization error threshold | - | 0-6000 | 3000 | ns | U16 | At stop | Immedi- ately |
| 08h | C13.07 | Occurrence count of excessive position ref- erence increment in sync position mode | - | 1-30 | 5 | - | U16 | During operation | Immedi- ately |
| 09h | C13.08 | EtherCAT enhanced link selection | 0: Inactive 1: Enabled | 0-1 | 0 | - | U16 | During operation | Upon re- power-on |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0Ah | C13.09 | Maximum errors and invalid frames of EtherCAT port 0 per unit time | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Bh | C13.0A | Maximum errors and invalid frames of EtherCAT port 1 per unit time | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Ch | C13.0B | Max. transfer error of EtherCAT port per unit time | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Dh | C13.0C | Max. EtherCAT data frame processing unit error per unit time | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Eh | C13.0D | Max. link loss value of EtherCAT port per unit time | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 0Fh | C13.0E | EtherCAT state machine status and port connection status | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 10h | C13.0F | EtherCAT AL status code | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 11h | C13.10 | EtherCAT parameter storage | 0: No storage 1: Storage | 0-1 | 1 | - | U16 | During operation | Immedi- ately |
| 12h | C13.11 | EtherCAT IRQ loss threshold | - | 0-10 | 5 | - | U16 | During operation | Immedi- ately |
| 13h | C13.12 | EtherCAT IRQ loss count | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 1Ah | C13.19 | Use of the loop network | - | 0-1 | 0 | - | U16 | At stop | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.7 "Group C13".

Motor Parameters (2020h/R20)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | R20.00 | Motor model | - | 0-65535 | 20000 | - | U16 | At stop | Upon re- power-on |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23h | R20.22 | Encoder type | - | 0-65535 | 0 | - | U16 | Read only | Upon re- power-on |

Drive Parameters (2021h/R21)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | R21.00 | Drive model | - | 0-65535 | 3 | - | U16 | At stop | Upon re- power-on |
| 02h | R21.01 | Internal drive model | - | 0-65535 | 3 | - | U16 | Read only | Immediately |
| 0Dh | R21.0C | Drive voltage class | - | 0-2 | 0 | - | U16 | Read only | Immediately |
| 0Eh | R21.0D | Rated drive power | - | 1-(232-1) | 40 | 0.01kW | U32 | Read only | Immediately |
| 10h | R21.0F | Rated output current of drive | - | 1-(232-1) | 280 | 0.01A | U32 | Read only | Immediately |
| 12h | R21.11 | Maximum output current of drive | - | 1-(232-1) | 980 | 0.01A | U32 | Read only | Immediately |
| 14h | R21.13 | Internal bleeder resistor power | - | 1-65535 | 40 | W | U16 | Read only | Immediately |
| 15h | R21.14 | Internal bleeder resistor resistance | - | 1-65535 | 50 | Ω | U16 | Read only | Immediately |

NOTICE

y For details about parameters above, refer to section 11.3.8 "Group R21".

Motor Gain Parameters (2022h/R22)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | R22.00 | Current loop mode | 0: Standard mode 1: Performance mode | 0-1 | 0 | - | U16 | At stop | Upon re- power-on |
| 02h | R22.01 | Current loop response level | - | 0-4000 | 0 | 0.1% | U16 | At stop | Upon re- power-on |
| 21h | R22.20 | MTPA field-weakening switch | - | 0-65535 | 256 | - | U16 | At stop | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22h | R22.21 | Field-weakening depth | - | 500-2000 | 1000 | 0.1% | U16 | At stop | Immedi- ately |
| 23h | R22.22 | Field-weakening proportional gain | - | 10-1000 | 100 | Hz | U16 | At stop | Immedi- ately |
| 24h | R22.23 | Field-weakening integral gain | - | 0-8000 | 100 | 0.1% | U16 | At stop | Immedi- ately |
| 25h | R22.24 | Cutoff frequency of d axis current low-pass filter | - | 0-16000 | 0 | Hz | U16 | At stop | Immedi- ately |
| 26h | R22.25 | Field-weakening d axis current limit | - | 0-3000 | 1500 | 0.1% | U16 | At stop | Immedi- ately |
| 31h | R22.30 | Dead zone compensation | - | 0-2000 | 1000 | 0.1% | U16 | At stop | Immedi- ately |

Parameters of Control in Progress (2030h/F30)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | F30.00 | JOG enabling in velocity mode | - | 0~8000 | 0 | - | U16 | During operation | Immedi- ately |
| 02h | F30.01 | JOG enabling in position mode | - | 0~8000 | 0 | - | U16 | During operation | Immedi- ately |
| 03h | F30.02 | JOG velocity reference | - | 0~8000 | 100 | rpm | U16 | During operation | Immedi- ately |
| 04h | F30.03 | JOG acceleration/ deceleration time | - | 0~3600000 | 100 | ms | U32 | During operation | Immedi- ately |
| 06h | F30.05 | JOG distance in position mode | - | ﹣231- (231-1) | 20000 | Unit in application | I32 | During operation | Immedi- ately |
| 11h | F30.10 | Inertia auto-tuning selection | 0: Disabled 1: Enabled | 0-65535 | 0 | - | U16 | During operation | Immedi- ately |
| 12h | F30.11 | Initial angle auto-tuning selection | 0: Disabled 1: Enabled | 0-1 | 0 | - | U16 | During operation | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.9 "Group F30".

Parameters of Control in Progress (2031h/F31)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | F31.00 | Fault reset | 0: Inactive 1: Reset | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 02h | F31.01 | Software reset | 0: Inactive 1: Reset | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 03h | F31.02 | Parameter initialization | 0: Inactive 1: Restore default settings of parameters 2: Restore default settings of the object dictionary | 0-2 | 0 | - | U16 | At stop | Immedi- ately |
| 04h | F31.03 | Drive motor parameter reset | 0: Inactive 1: Factory reset drive parameters 2: Factory reset motor parameters | 0-2 | 0 | - | U16 | At stop | Immedi- ately |
| 05h | F31.04 | Fault record initialization | 0: Inactive 1: Fault record clearing | 0-1 | 0 | - | U16 | At stop | Immedi- ately |
| 11h | F31.10 | Encoder data reset | 0: Inactive 1: Read encoder 2: Write encoder 3: Reset encoder fault 4: Reset encoder fault and multi-turn data 16: Operation failed | 0-31 | 0 | - | U16 | At stop | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.10 "Group F31".

Running Monitoring Parameters (2040h/U40)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | U40.00 | Speed reference | - | -9000- 9000 | 0 | rpm | I16 | Read only | Immedi- ately |
| 02h | U40.01 | Speed feedback | - | -9000- 9000 | 0 | rpm | I16 | Read only | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 03h | U40.02 | Torque reference | - | -4000- 4000 | 0 | 0.1% | I16 | Read only | Immedi- ately |
| 04h | U40.03 | Torque feedback | - | -4000- 4000 | 0 | 0.1% | I16 | Read only | Immedi- ately |
| 05h | U40.04 | DI status | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 06h | U40.05 | DO status | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 07h | U40.06 | Bus voltage | - | 0-9000 | 0 | 0.1V | U16 | Read only | Immedi- ately |
| 08h | U40.07 | Average load ratio | - | 0-4000 | 0 | 0.1% | U16 | Read only | Immedi- ately |
| 09h | U40.08 | Electrical angle | - | 0-36000 | 0 | 0.01° | U16 | Read only | Immedi- ately |
| 0Ah | U40.09 | Mechanical angle | - | 0-36000 | 0 | 0.01° | U16 | Read only | Immedi- ately |
| 0Dh | U40.0C | RMS value of phase current | - | -9000- 9000 | 0 | 0.1A | I16 | Read only | Immedi- ately |
| 11h | U40.10 | Position deviation counter | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 15h | U40.14 | Absolute position reference | - | ﹣231- (231-1) | 0 | Unit in application | I32 | Read only | Immedi- ately |
| 17h | U40.16 | Absolute position feedback (reference unit) | - | ﹣231- (231-1) | 0 | Unit in application | I32 | Read only | Immedi- ately |
| 19h | U40.18 | Absolute position feedback (encoder unit) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 1Bh | U40.1A | Absolute position feedback (encoder unit) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 1Dh | U40.1C | Encoder single-turn data | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 1Fh | U40.1E | Encoder multi-turn position data | - | 0-65535 | 0 | Rev | U16 | Read only | Immedi- ately |
| 20h | U40.1F | Encoder initial angle | - | 0-36000 | 0 | 0.01° | U16 | Read only | Immedi- ately |
| 21h | U40.20 | Encoder multi-turn data (low 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 23h | U40.22 | Encoder multi-turn data (high 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25h | U40.24 | Absolute position feedback (encoder unit) (low 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 27h | U40.26 | Absolute position feedback (encoder unit) (high 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 29h | U40.28 | Position feedback in rotation mode (reference unit) (low 32 bits) | - | ﹣231- (231-1) | 0 | Unit in application | I32 | Read only | Immedi- ately |
| 2Bh | U40.2A | Position feedback in rotation mode (encoder unit) (low 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 2Dh | U40.2C | Position feedback in rotation mode (encoder unit) (high 32 bits) | - | ﹣231- (231-1) | 0 | P | I32 | Read only | Immedi- ately |
| 31h | U40.30 | Heatsink temperature | - | -9000- 9000 | 0 | 0.1℃ | I16 | Read only | Immedi- ately |
| 35h | U40.34 | Offline inertia auto- tuning value | - | 0-12000 | 0 | % | U16 | Read only | Immedi- ately |
| 37h | U40.36 | Instantaneous value in phase U current | - | ﹣231- (231-1) | 0 | 0.001A | I32 | Read only | Immedi- ately |
| 39h | U40.38 | Instantaneous value in phase V current | - | ﹣231- (231-1) | 0 | 0.001A | I32 | Read only | Immedi- ately |
| 3Bh | U40.3A | Synchronization cycle measured value | - | 0-(231-1) | 0 | 10ns | U32 | Read only | Immedi- ately |
| 3Dh | U40.3C | SYNC and IRQ phase value | - | ﹣231- (231-1) | 0 | 10ns | I32 | Read only | Immedi- ately |
| 3Fh | U40.3E | Drive accumulated heat | - | 0-2000 | 0 | 0.1% | U16 | Read only | Immedi- ately |
| 40h | U40.3F | Motor accumulated heat | - | 0-2000 | 0 | 0.1% | U16 | Read only | Immedi- ately |

NOTICE

y For details about parameters above, refer to section 11.3.11 "Group U40".

Status Monitoring Parameters (2041h/U41)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | U41.00 | MCU system status | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 02h | U41.01 | MCU fault state | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 05h | U41.04 | Encoder system status | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 06h | U41.05 | Encoder fault state | - | 0-65535 | 0 | - | U16 | Read only | Immedi- ately |
| 07h | U41.06 | Group number of abnormal parameter | - | 0-255 | 0 | - | U16 | Read only | Immedi- ately |
| 08h | U41.07 | Offset of the abnormal parameter within the parameter group | - | 0-255 | 0 | - | U16 | Read only | Immedi- ately |
| 0Bh | U41.0A | Servo Status | - | 0-3 | 0 | - | U16 | Read only | Immedi- ately |
| 0Ch | U41.0B | Servo running mode | - | 0-9 | 0 | - | U16 | Read only | Immedi- ately |
| 0Dh | U41.0C | Servo running time | - | 0-(232-1) | 0 | 0.1s | U32 | Read only | Immedi- ately |

Version Parameters (2042h/U42)

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01h | U42.00 | ARM version | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 03h | U42.02 | Encoder version | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 04h | U42.03 | ARM-based machine | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 06h | U42.05 | Internal software version | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 0Bh | U42.0A | EtherCAT CoE version | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 0Ch | U42.0B | EtherCAT XML version | - | 0-65535 | 0 | 0.01 | U16 | Read only | Immediately |
| 11h | U42.10 | Drive model | - | 0-65535 | 0 | - | U16 | Read only | Immediately |
| 12h | U42.11 | Motor model | - | 0-65535 | 0 | - | U16 | Read only | Immediately |
| 13h | U42.12 | Encoder model | - | 0-65535 | 0 | - | U16 | Read only | Immediately |
| 14h | U42.13 | Power supply unit model identification | - | 0-65535 | 0 | - | U16 | Read only | Immediately |

| Index | Param- eter | Name | Options | Value Range | Default | Unit | Data Type | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15h | U42.14 | Inverter model identification 1 | - | 0-65535 | 0 | - | U16 | Read only | Immediately |
| 16h | U42.15 | Inverter model identification 2 | - | 0-65535 | 0 | - | U16 | Read only | Immediately |
| 17h | U42.16 | Servo version | - | 0-65535 | 0 | - | U16 | Read only | Immediately |

    - Common Parameters in Group 6000h

Parameter group 6000h contains supported sub-protocol DSP 402 related objects.

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 603Fh | 0 | Error code | RO | TPDO | U16 | - | - | - | - | - |
| 6040h | 0 | Control word | RW | RPDO | U16 | - | 0-65535 | 0 | During operation | Immedi- ately |
| 6041h | 0 | Status word | RO | TPDO | U16 | - | - | - | - | - |
| 605Ah | 0 | Quick stop option code | RW | NO | I16 | - | 0-7 | 2 | During operation | Upon re- power-on |
| 605Ch | 0 | Stop mode upon servo-off | RW | NO | I16 | - | -4-1 | 0 | During operation | Upon re- power-on |
| 605Dh | 0 | Halt option code | RW | NO | I16 | - | 1-3 | 1 | During operation | Upon re- power-on |
| 605Eh | 0 | Stop mode at No. 2 fault | RW | NO | I16 | - | 5-3 | 2 | During operation | Upon re- power-on |
| 6060h | 0 | Servo mode | RW | RPDO | I8 | - | 0-10 | 0 | During operation | Immedi- ately |
| 6061h | 0 | Modes of operation display | RO | TPDO | I8 | - | - | - | - | - |
| 6062h | 0 | Position reference | RO | TPDO | I32 | Reference unit | - | - | - | - |
| 6063h | 0 | Position feedback | RO | TPDO | I32 | Encoder unit | - | - | - | - |
| 6064h | 0 | Position feedback | RO | TPDO | I32 | Reference unit | - | - | - | - |
| 6065h | 0 | Following error window | RW | RPDO | U32 | Reference unit | 0-(232-1) | 0 | During operation | Immedi- ately |
| 6066h | 0 | Following error time out | RW | RPDO | U16 | ms | 0-65535 | 0 | During operation | Immedi- ately |
| 6067h | 0 | Position reach threshold | RW | RPDO | U32 | Reference unit | 0-(232-1) | 734 | During operation | Immedi- ately |

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6068h | 0 | Position window time | RW | RPDO | U16 | ms | 0-65535 | 0 | During operation | Immedi- ately |
| 606Ch | 0 | Actual speed | RO | TPDO | I32 | Reference unit/s | - | - | - | - |
| 606Dh | 0 | Speed reach threshold | RW | RPDO | U16 | rpm | 0-65535 | 10 | During operation | Immedi- ately |
| 606Eh | 0 | Velocity window time | RW | RPDO | U16 | ms | 0-65535 | 0 | During operation | Immedi- ately |
| 606Fh | 0 | Velocity threshold | RW | RPDO | U16 | rpm | 0-65535 | 10 | During operation | Immedi- ately |
| 6070h | 0 | Velocity threshold time | RW | RPDO | U16 | ms | 0-65535 | 0 | During operation | Immedi- ately |
| 6071h | 0 | Target torque | RW | RPDO | I16 | 0.1% | 4000- 4000 | 0 | During operation | Immedi- ately |
| 6072h | 0 | Max. torque | RW | RPDO | U16 | 0.1% | 0-4000 | 3500 | During operation | Immedi- ately |
| 6074h | 0 | Torque reference | RO | TPDO | I16 | 0.1% | - | 0 | - | - |
| 6077h | 0 | Actual torque | RO | TPDO | I16 | 0.1% | - | 0 | - | - |
| 607Ah | 0 | Target position | RW | RPDO | I32 | Reference unit | -231- (231-1) | 0 | During operation | Immedi- ately |
| 607Ch | 0 | Home offset | RW | RPDO | I32 | Reference unit | -231- (231-1) | 0 | During operation | Immedi- ately |
| 607D | Software absolute position limit |  |  |  |  |  |  |  |  |  |
|  | 0 | Highest sub-index supported | RO | NO | U8 | - | - | 0x02 | - | - |
|  | 1 | Minimum software position limit | RW | RPDO | I32 | Reference unit | -231- (231-1) | -231 | During operation | Immedi- ately |
|  | 2 | Maximum software position limit | RW | RPDO | I32 | Reference unit | -231- (231-1) | 231-1 | During operation | Immedi- ately |
| 607Eh | 0 | Reference polarity | RW | RPDO | U8 | - | 0-255 | 0 | During operation | Immedi- ately |
| 607Fh | 0 | Maximum speed | RW | RPDO | U32 | Reference unit/s | 0-(232-1) | 104857600 | During operation | Immedi- ately |
| 6081h | 0 | Profile operating speed | RW | RPDO | U32 | User velocity | 0-(232-1) | 1747627 | During operation | Immedi- ately |
| 6083h | 0 | Profile acceleration rate | RW | RPDO | U32 | Reference unit/s2 | 0-(232-1) | 174762666 | During operation | Immedi- ately |

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6084h | 0 | Profile deceleration rate | RW | RPDO | U32 | Reference unit/s2 | 0-(232-1) | 174762666 | During operation | Immedi- ately |
| 6085h | 0 | Quick stop deceleration | RW | RPDO | U32 | Reference unit/s2 | 0-(232-1) | 231-1 | During operation | Immedi- ately |
| 6086h | 0 | Motion profile type | RW | RPDO | I16 | - | 32767- 32767 | 0 | During operation | Immedi- ately |
| 6087h | 0 | Torque slope | RW | RPDO | U32 | 0.1%/s | 0-(232-1) | 232-1 | During operation | Immedi- ately |
| 6091h | Gear ratio |  |  |  |  |  |  |  |  |  |
|  | 0 | Highest sub-index supported | RO | NO | U8 | U8 | - | 0x02 | - | - |
|  | 1 | Motor revolutions | RW | RPDO | U32 | - | 0-(232-1) | 1 | During operation | Immedi- ately |
|  | 2 | Shaft revolutions | RW | RPDO | U32 | - | 1-(232-1) | 1 | During operation | Immedi- ately |
| 6098h | 0 | Homing method | RW | RPDO | I8 | - | 2-35 | 1 | During operation | Immedi- ately |
| 6099h | Homing speed |  |  |  |  |  |  |  |  |  |
|  | 0 | Highest sub-index supported | RO | NO | U8 | - | - | 2 | - | - |
|  | 1 | Speed during search for switch | RW | RPDO | U32 | Reference unit/s | 0-(232-1) | 1747627 | During operation | Immedi- ately |
|  | 2 | Speed during search for zero | RW | RPDO | U32 | Reference unit/s | 10-(232-1) | 174763 | During operation | Immedi- ately |
| 609Ah | 0 | Homing acceleration | RW | RPDO | U32 | Reference unit/s2 | 0-(232-1) | 1747626667 | During operation | Immedi- ately |
| 60B0h | 0 | Position offset | RW | RPDO | I32 | Reference unit | -231- (231-1) | 0 | During operation | Immedi- ately |
| 60B1h | 0 | Speed deviation | RW | RPDO | I32 | Reference unit/s | -231- (231-1) | 0 | During operation | Immedi- ately |
| 60B2h | 0 | Torque offset | RW | RPDO | I16 | 0.10% | 4000- 4000 | 0 | During operation | Immedi- ately |
| 60B8h | 0 | Touch probe function | RW | RPDO | U16 | - | 0-65535 | 0 | During operation | Immedi- ately |
| 60B9h | 0 | Touch probe status | RW | TPDO | U16 | - | - | 0 | - | - |
| 60BAh | 0 | Touch probe 1 positive edge | RW | TPDO | I32 | Reference unit | - | 0 | - | - |

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60BBh | 0 | Touch probe 1 negative edge | RW | TPDO | I32 | Reference unit | - | 0 | - | - |
| 60BCh | 0 | Touch probe 2 positive edge | RW | TPDO | I32 | Reference unit | - | 0 | - | - |
| 60BDh | 0 | Touch probe 2 negative edge | RW | TPDO | I32 | Reference unit | - | 0 | - | - |
| 60C5h | 0 | Max. acceleration | RW | RPDO | U32 | User acceleration unit | 0-232-1 | 231-1 | During operation | Immedi- ately |
| 60C6h | 0 | Max. deceleration | RW | RPDO | U32 | User acceleration unit | 0-232-1 | 231-1 | During operation | Immedi- ately |
| 60D5h | 0 | Touch probe 1 positive edge counter | RO | TPDO | U16 | - | - | 0 | - | - |
| 60D6h | 0 | Touch probe 1 negative edge counter | RO | TPDO | U16 | - | - | 0 | - | - |
| 60D7h | 0 | Touch probe 2 positive edge counter | RO | TPDO | U16 | - | - | 0 | - | - |
| 60D8h | 0 | Touch probe 2 negative edge counter | RO | TPDO | U16 | - | - | 0 | - | - |
| 60E0h | 0 | Positive torque limit | RW | RPDO | U16 | 0.1% | 0-4000 | 3500 | During operation | Immedi- ately |
| 60E1h | 0 | Negative torque limit | RW | RPDO | U16 | 0.1% | 0-4000 | 3500 | During operation | Immedi- ately |
| 60E3h | Supported homing method |  |  |  |  |  |  |  |  |  |
|  | 0 | Highest sub-index supported | RO | NO | U8 | - | - | 22 | - | - |
|  | 1 | 1st supported homing method | RO | NO | I16 | - | - | 1 | - | - |
|  | 2 | 2nd supported homing method | RO | NO | I16 | - | - | 2 | - | - |
|  | 3 | 3rd supported homing method | RO | NO | I16 | - | - | 3 | - | - |
|  | 4 | 4th supported homing method | RO | NO | I16 | - | - | 4 | - | - |
|  | 5 | 5th supported homing method | RO | NO | I16 | - | - | 5 | - | - |
|  | 6 | 6th supported homing method | RO | NO | I16 | - | - | 6 | - | - |

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7 | 7th supported homing method | RO | NO | I16 | - | - | 7 | - | - |
|  | 8 | 8th supported homing method | RO | NO | I16 | - | - | 8 | - | - |
|  | 9 | 9th supported homing method | RO | NO | I16 | - | - | 9 | - | - |
|  | A | 10th supported homing method | RO | NO | I16 | - | - | 10 | - | - |
|  | B | 11th supported homing method | RO | NO | I16 | - | - | 11 | - | - |
|  | C | 12th supported homing method | RO | NO | I16 | - | - | 12 | - | - |
|  | D | 13th supported homing method | RO | NO | I16 | - | - | 13 | - | - |
|  | E | 14th supported homing method | RO | NO | I16 | - | - | 14 | - | - |
|  | F | 15th supported homing method | RO | NO | I16 | - | - | 17 | - | - |
|  | 10 | 16th supported homing method | RO | NO | I16 | - | - | 18 | - | - |
|  | 11 | 17th supported homing method | RO | NO | I16 | - | - | 19 | - | - |
|  | 12 | 18th supported homing method | RO | NO | I16 | - | - | 20 | - | - |
|  | 13 | 19th supported homing method | RO | NO | I16 | - | - | 21 | - | - |
|  | 14 | 20th supported homing method | RO | NO | I16 | - | - | 22 | - | - |
|  | 15 | 21th supported homing method | RO | NO | I16 | - | - | 23 | - | - |
|  | 16 | 22th supported homing method | RO | NO | I16 | - | - | 24 | - | - |
|  | 17 | 23th supported homing method | RO | NO | I16 | - | - | 25 | - | - |
|  | 18 | 24th supported homing method | RO | NO | I16 | - | - | 26 | - | - |
|  | 19 | 25th supported homing method | RO | NO | I16 | - | - | 27 | - | - |

| Index | Sub- index | Name | Access the Platform | PDO Mapping | Data Type | Unit | Value Range | Default | Modifica- tion Mode | Effective Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1A | 26th supported homing method | RO | NO | I16 | - | - | 28 | - | - |
|  | 1B | 27th supported homing method | RO | NO | I16 | - | - | 29 | - | - |
|  | 1C | 28th supported homing method | RO | NO | I16 | - | - | 30 | - | - |
|  | 1D | 29th supported homing method | RO | NO | I16 | - | - | 33 | - | - |
|  | 1E | 30th supported homing method | RO | NO | I16 | - | - | 34 | - | - |
|  | 1F | 31th supported homing method | RO | NO | I16 | - | - | 35 | - | - |
| 60E6h | 0 | Encoder increments for the additional position | RW | NO | U16 | - | 0-1 | 0 | During operation | Immedi- ately |
| 60F4h | 0 | Position deviation | RO | TPDO | I32 | Reference unit | - | - | - | - |
| 60FCh | 0 | Position reference | RO | TPDO | I32 | Encoder unit | - | - | - | - |
| 60FDh | 0 | DI status | RO | TPDO | U32 | - | - | - | - | - |
| 60FFh | 0 | Target velocity | RW | RPDO | I32 | Reference unit/s | (-231-1)- (231-1) | 0 | During operation | Immedi- ately |
| 6502h | 0 | Supported drive modes | RO | NO | U32 | - | - | 941 | - | - |

NOTICE

y For details about parameters above, refer to section 11.3.12 "Group 6000".

11.3 Description of Parameters

    - Group C00

C00.05: Stiffness level

y   Defines the stiffness level of the servo system. The higher the stiffness level, the stronger the gains and the quicker the response will be. But an excessively high stiffness level will cause vibration. The setpoint 0 indicates the lowest stiffness and 41 indicates the highest stiffness.

C00.06: Load inertia ratio

y   Defines the mechanical load inertia ratio relative to the motor moment of inertia.

y When C00.06 is set to 0, it indicates the motor carries no load; if it is set to 1.00, it indicates the mechanical load inertia is the same as the motor moment of inertia.

y When the value of C00.06 is equal to the actual inertia ratio, the value of speed loop gain can represent the maximum follow-up frequency of actual speed loop.

    - Group C01

C01.00: 1st position loop gain

y   Defines the proportional gain of the position loop.

y This parameter determines the responsiveness of the position loop. A high setpoint shortens the positioning time. Note that an excessively high setpoint may cause vibration.

y    The 1st gain set includes C01.00, C01.01, C01.02, and C01.03.

C01.01: 1st speed loop gain

y   Defines the speed loop proportional gain.

y This parameter determines the responsiveness of the speed loop. The higher the setpoint, the faster the speed loop response is. Note that an excessively high setpoint may cause vibration.

y   In the position control mode, the position loop gain must be increased together with the speed loop gain.

C01.02: 1st speed loop integral time

y   Defines the speed loop integral time constant.

y   The lower the setpoint, the better the integral action, and the quicker will the deviation value be close to 0.

y   There is no integral action when C01.02 is set to 512.00 ms

C01.08: 2nd position loop gain

y   Defines the 2nd gain of the position loop.

y    The 2nd gain set includes C01.08, C01.09, C01.0A, and C01.0B.

y   For details about gain switchover, see section 7.5 "Gain Switchover".

C01.0B: 2nd torque reference filter cutoff frequency

y   Defines the torque reference filter time constant.

y  Low-pass filtering of torque references helps to smoothen torque references and reduce vibration.

y   Pay attention to the responsiveness during setting as an excessively high setpoint lowers down the

responsiveness.

NOTICE

y The servo drive offers two low-pass filters for torque references. By default, the 1st filter is used. y Gain switchover can be used in the position or speed control mode. Once certain conditions are satisfied, the servo drive can switch to filter 2.

C01.11: Cutoff frequency of speed feedback low-pass filter

y   Defines the cutoff frequency for first-order low-pass filtering on the speed feedback.

y   The lower the setpoint, the weaker the speed feedback fluctuation, and the longer the feedback delay will

be.

y   Setting this parameter to 8000 Hz negates the filtering effect.

C01.12: Speed feedback overlapping average filter time constant

y   Defines the moving average filtering times for speed feedback.

y   The higher the setpoint, the weaker the speed feedback fluctuation, but the longer the feedback delay will

be.

y When C01.12 is set to a value higher than 0, C01.11 (Cutoff frequency of speed feedback low-pass filter) is invalid.

C01.13: Speed feedforward source

y   Defines the source of the speed loop feedforward signal.

y   In the position control mode, the speed feedforward control can improve the position reference

responsiveness.

| Setpoint | Speed Feedforward Source | Remarks |
| --- | --- | --- |
| 0 | No feedforward | - |
| 1 | Internal reference | The speed corresponding to the position reference (encoder unit) is defined as the speed feedforward source. |
| 2 | Model tracking | Model tracking control can improve the responsiveness and shorten the positioning time. It is only available in the position control mode. It must be used with C02.00. When C02.00 is set to 1, the speed feedforward is sourced from the speed feedforward output of model tracking. |
| 5 | Communication | In CSP, 60B1h is used as the source of the external speed feedforward signal. Bit 6 of 607Eh can specify the polarity of the speed feedforward signal (60B1h). |

C01.14: Speed feedforward percentage

y In the position control mode, speed feedforward is the value of C01.14 multiplied by the speed feedforward signal, which is part of the speed reference. Increasing the setpoint improves the responsiveness to position references and reduces the position deviation during operation at a constant speed.

y Set C01.15 to a fixed value first, and then gradually increase the value of C01.14 from 0 to a certain setpoint at which speed feedforward achieves the desired effect.

y   Adjust C01.15 and C01.14 repeatedly until a balanced setting is achieved.

y For the speed feedforward function and speed feedforward signal selection, see C01.13 (Speed feedforward source selection).

C01.15: Speed feedforward filter cutoff frequency

y   Defines the speed feedforward smoothing filter time.

C01.16: Torque feedforward source

y   Defines whether to enable the internal torque feedforward function in a non-torque control mode.

y The torque feedforward function can improve the torque reference responsiveness and reduce the position deviation during operation at constant acceleration/deceleration rate.

| Setpoint | Torque Feedforward Source | Remarks |
| --- | --- | --- |
| 0 | No feedforward | - |
| 1 | Internal reference | The torque feedforward signal source is the speed reference. In the position control mode, the speed reference is output from the position controller. In the speed control mode, the speed reference is output from the user speed reference. |
| 2 | Model tracking | It must be used with C02.00. When C02.00 is set to 1, the torque feedforward is sourced from the torque feedforward output of model tracking. |
| 5 | Communication | In CSP, 60B1h is used as the source of the external torque feedforward signal. Bit 6 of 607Eh can specify the polarity of the torque feedforward signal (60B1h). |

y Torque feedforward parameters include C01.17 (Torque feedforward percentage) and C01.18 (Torque feedforward cutoff frequency).

y   In a non-torque control mode, the control block diagram of torque feedforward is as follows:

Torque feedforward control

Speed detection

Speed reference

Motor

−

Speed feedback

Encoder

Current loop control

Speed loop control

Figure 11-1    Torque feedforward control

C01.17: Torque feedforward percentage

y In control modes other than torque control, torque feedforward is the product of torque feedforward signal multiplied by C01.17 and is part of the torque reference. Increasing the setpoint improves the responsiveness to variable speed references and position references and reduces the position deviation during operation at a constant speed.

C01.18: Torque feedforward filter cutoff frequency

y   Defines the filter time constant of torque feedforward.

C01.1B: PDFF control coefficient

y   Defines the control method of the speed loop.

y When the setpoint is 100.0, PI control (default control mode of the speed loop) is applied to the speed loop, which features fast dynamic response.

y When the setpoint is 0.0, speed loop integral action is enhanced, which filters out low-frequency interference but also slows down the dynamic response.

y C01.1B can be used to keep a good responsiveness of the speed loop, with the anti-interference capacity in low-frequency bands improved and the speed feedback overshoot not increased.

C01.30: Adaptive notch mode

Setpoint:

0: Adaptive notch not updated

1: One adaptive notch activated (3rd notch)

2: Two adaptive notches activated (3rd and 4th notches)

3: Adaptive notch cleared, values of the 3rd and 4th notches restored to default settings 4: Resonance point tested only, displayed in C01.31, C01.32, and C01.33

Description:

y   Defines the operation mode of the adaptive notch.

C01.38: Gain switchover mode

| Setpoint | Gain Switchover Condition | Remarks |
| --- | --- | --- |
| 0 | Fixed to the 1st gain set | The 1st gain set applies. |
| 1 | DI switchover | Gains are switched through bit 26 of 60FE. Bit 26 signal inactive: 1st gain set Bit 26 signal active: 2nd gain set If the bit 26 signal cannot be allocated to a DI terminal, the 1st gain set applies. |
| 2 | DI P-PI switchover | Gains are switched through bit 26 of 60FE. Bit 26 signal inactive: 1st gain set Bit 26 signal active: 2nd gain set (The 2nd speed loop integral (C01.0A) is forced to be 512 ms.) If the bit 26 signal cannot be allocated to a DI terminal, the 1st gain set applies. |
| 3 | Torque reference | When the absolute value of the torque reference exceeds (threshold + loop width, %) in the last 1st gain set, the drive switches to the 2nd gain set. When the absolute value of the torque reference is less than (threshold – loop width, %) and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. |
| 4 | Speed reference | When the absolute value of the speed reference exceeds (threshold + loop width, rpm) in the last 1st gain set, the drive switches to the 2nd gain set. When the absolute value of the speed reference is less than (threshold – loop width, rpm) and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. |
| 5 | Speed feedback | It is valid only in the position control mode. When the absolute value of the actual speed exceeds (threshold + loop width, rpm) in the last 1st gain set, the drive switches to the 2nd gain set. When the absolute value of the actual speed is less than (threshold – loop width, rpm) and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. The 1st gain set applies when the drive is not in the position control mode. |
| 6 | Speed reference change rate | It is valid only in non-speed control modes. When the absolute value of the change rate in the speed reference exceeds (threshold + loop width, 10 rpm/s) in the last 1st gain set, the drive switches to the 2nd gain set. When the absolute value of the change rate in the speed reference is less than (threshold – loop width, 10 rpm/s) and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. The 1st gain set applies in the speed control mode. |

| Setpoint | Gain Switchover Condition | Remarks |
| --- | --- | --- |
| 7 | Position deviation | When the absolute value of the position deviation exceeds (threshold + loop width, encoder unit) in the last 1st gain set, the drive switches to the 2nd gain set. When the absolute value of the position deviation is less than (threshold + loop width, encoder unit) and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. The 1st gain set applies when the drive is not in the position control mode. |
| 8 | Position reference | It is valid only in the position control mode. When the position reference is not 0 in the last 1st gain set, the drive switches to the 2nd gain set. When the position reference is 0 and this status lasts within the delay (C01.39) in the last 2nd gain set, the drive returns to the 1st gain set. The 1st gain set applies when the drive is not in the position control mode. |

C01.39: Gain switchover time

Defines the duration when the drive switches from the 2nd gain set to the 1st gain set.

C01.3A: Gain switchover threshold

y   Defines the gain switchover threshold.

y Gain switchover is affected by both the threshold and the loop width, as defined by C01.38. The unit of gain switchover threshold varies with the switchover condition.

y Set C01.3A to a value greater than or equal to C01.3B. If C01.3A is set to a value less than C01.3B, the servo drive sets C01.3A to the same value as C01.3B.

C01.3B: Gain switchover loop width

y   Defines the gain switchover loop width.

y Gain switchover is affected by both the threshold and the loop width. The unit of gain switchover threshold varies with the switchover condition.

y Set C01.3A to a value greater than or equal to C01.3B. If C01.3A is set to a value less than C01.3B, the servo drive sets C01.3A to the same value as C01.3B.

C01.40: Frequency of the 1st notch

y   Defines the center frequency of the notch, which is the mechanical resonance frequency.

y   In the torque control mode, setting the notch frequency to 8000 Hz deactivates the notch function.

C01.41: Width level of the 1st notch

y   Defines the width level of the notch. Use the default value in general cases.

y   Width level is the ratio of the notch width to the notch center frequency.

C01.42: Depth level of the 1st notch

y   Defines the depth level of the notch.

y   The depth level of the notch is the ratio between the input to the output at the notch center frequency.

y The higher the setpoint, the lower the notch depth and the weaker the mechanical resonance suppression will be. Note that an excessively high setpoint may cause system instability.

y   For the use of notch, see 7.14 "Vibration Suppression".

C01.45: Depth level of the 2nd notch

y   Description of the 2nd notch parameters is the same as that of the 1st notch parameters.

NOTICE

y The 1st and 2nd notches can be set manually or configured as adaptive notches (C01.30 = 1 or 2). In this case, the parameters are automatically set by the drive, while the other three notches can be set manually.

    - Group C03

C03.21: Speed reference

y   It is the speed reference in the local speed mode, which is invalid in EtherCAT mode.

C03.22: Acceleration rate

y It is the acceleration ramp time of the speed reference in the local speed mode, which is invalid in EtherCAT mode.

C03.24: Deceleration rate

y It is the deceleration ramp time of the speed reference in the local speed mode, which is invalid in EtherCAT mode.

C03.27: Internal positive speed limit

y   It is the PL of the speed reference in the local speed mode, which is invalid in EtherCAT mode.

C03.28: Internal negative speed limit

y   It is the NL of the speed reference in the local speed mode, which is invalid in EtherCAT mode.

C03.43: Internal positive torque limit

y   It is valid only in the local torque mode. For torque limit in EtherCAT mode, use 60E0h/60E1h/6072h. Use the torque limit with caution as an excessively low limit value may lead to insufficient motor torque output.

y If the setpoint exceeds the maximum torque of the servo motor and servo drive, the actual torque is limited to the maximum torque of the servo motor and servo drive.

C03.44: Internal negative torque limit

y   It is valid only in the local torque mode. For torque limit in EtherCAT mode, use 60E0h/60E1h/6072h.

Use the torque limit with caution as an excessively low limit value may lead to insufficient motor torque

output.

y If the setpoint exceeds the maximum torque of the servo motor and servo drive, the actual torque is limited to the maximum torque of the servo motor and servo drive.

C03.47: Positive speed limit in torque mode

y    It is valid only in the local torque mode. Use 607F for the speed limit in the EtherCAT, CST, and PT modes.

C03.48: Negative speed limit in torque mode

y    It is valid only in the local torque mode. Use 607F for the speed limit in the EtherCAT, CST, and PT modes.

C03.4B: Invalid value for torque reached

y The torque reached function is used to judge whether the actual torque reference reaches the range of the valid value for torque reached. If yes, the servo drive outputs the corresponding flag (bit 10 of the status word) to the host controller.

A: Actual torque reference (U40.02)

B: Base value for torque reach (C03.49) C: Valid value for torque reach (C03.4A) D: Invalid value for torque reach (C03.4B) C and D are offsets on the basis of B.

The torque reach signal is activated only when the actual torque reference meets the condition: |A| ≥ B +

C. Otherwise, the torque reach signal remains inactive.

The torque reach signal is deactivated only when the actual torque reference meets the condition: |A| < B +

D.

Actual torque

B+C

B+D B 0

﹣B

A

Time

﹣(B+D)

﹣(B+C)

Torque reached output

6041.bit10

OFF

ON

OFF

ON

OFF

    - Group C05

C05.0D: Maximum downtime

y Defines the maximum time taken by the motor in decelerating from 6000 RPM to 0 RPM when the stop mode is set to "Ramp to stop as defined by 6084h/609Ah (HM)" or "Ramp to stop as defined by 6085h".

    - Group C06

C06.04: Input phase loss detection

Servo drives support three-phase 380 V power supplies. When voltage fluctuation or phase loss occurs on the power supply, power input phase loss protection will be triggered by the servo drive based on the setting of C06.04.

y   C06.04 = 0: The servo drive reports Er81.0 (Phase loss fault) when the servo drive is set to 3 kW.

y C06.04 = 1: The servo drive does not report Er81.0 (Phase loss fault) when the servo drive is set to 3 kW, with deration of 80%.

C06.11: Motor overload protection threshold

y   Determines the motor overload duration before Er41.0 (Motor overload) is reported.

y    You can change the setpoint to advance or delay the time when overload protection is triggered based on the motor temperature. The setpoint 50% indicates the time is cut by half; 150% indicates the time is prolonged by 50%.

y   Set this parameter based on the actual temperature of the motor.

C06.20: Protection from out of control

y   Sets whether to enable the runaway protection function.

    - Group C0A

C0A.09: Commissioning software communication baud rate

y   Defines the communication rate between the servo drive and the host controller.

y The baud rate set in the servo drive must be the same as that in the host controller. Otherwise, communication will fail.

C0A.0A: Commissioning software communication format

y   Defines the data check mode between the servo drive and the host controller during communication.

y The data format of the servo drive and the host controller must be the same; otherwise, the communication fails.

    - Group C13

C13.00: EtherCAT slave name

y   Indicates the station number assigned to the slave by the master during EtherCAT communication.

C13.01: EtherCAT slave alias

y Indicates the station number assigned to the slave EtherCAT communication since the master cannot automatically assign station numbers.

y C13.01 = 0: The master assigns the station numbers by default. C13.01 ≠ 0: The set station number applies by default, with the one assigned by master deactivated.

C13.05: EtherCAT synchronization mode setting

y   Defines the synchronization work mode:

| Setpoint | Function | Remarks |
| --- | --- | --- |
| 0 | Manufacturer Function | Manufacturer Function |
| 1 | Sync 1 | Applies to the scenarios where the synchronization performance indicator of the host controller jitters for 1 us. |
| 2 | Sync 2 | Applies to the scenarios where the synchronization performance indicator of the host controller jitters for more than 1 us. |

y In the work mode, the synchronization cycle must be an integer multiple of 250 μs. Otherwise, the serve drive will report Er74.0 (EtherCAT synchronization cycle setting is incorrect.)

C13.06: EtherCAT synchronization error threshold

y Defines the permissible jitter range of synchronization signals when the servo drive works in synchronization mode 1 (C13.05 = 1).

C13.08: EtherCAT enhanced link selection

y When a redundant loop network is used, the EtherCAT Enhanced Link Check function must be enabled (C13.08 = 1), which will take effect upon next power-on of the servo drive.

y   When a loop network is used, both C13.08 and C13.19 need to be set to 1.

    - Group R21

R21.00: Drive model

Setpoint:

2: 200EC	3: 400EC	5: 750EC	6: 1000EC

7: 1500EC	10004: 2000EC    10005:3000EC

Description:

y   Sets the SN of the servo drive. The following table lists the servo drive SNs.

| Setpoint | Servo Drive SN | Remarks |
| --- | --- | --- |
| 2 | 200EC | The rated drive power is 0.2 kW. The main circuit inputs single-phase 220 V. |
| 3 | 400EC | The rated drive power is 0.4 kW. The main circuit inputs single-phase 220 V. |
| 5 | 750EC | The rated drive power is 0.75 kW. The main circuit inputs single-phase 220 V. |
| 6 | 1000EC | The rated drive power is 1.0 kW. The main circuit inputs single-phase or three- phase 220 V. (The main circuit of the servo drive supports single-phase 220 V power supplies without derating.) |

| Setpoint | Servo Drive SN | Remarks |
| --- | --- | --- |
| 7 | 1500EC | The rated drive power is 1.5 kW. The main circuit inputs single-phase or three- phase 220 V. (The main circuit of the servo drive supports single-phase 220 V power supplies without derating.) |
| 10003 | 2000EC | The rated drive power is 2.0 kW. The main circuit inputs three-phase 380 V. |
| 10004 | 3000EC | The rated drive power is 3.0 kW. The main circuit inputs three-phase 380 V. |

If the voltage input to the main circuit of the servo drive does not comply with the preceding specifications, a fault or damage occurs.

    - Group F30

F30.03 JOG acceleration/deceleration time

y Acceleration/Deceleration time setpoint for jog in velocity mode, which can be enabled through parameter F30.00 on the panel or through the software

F30.10: Inertia auto-tuning selection

y   Used to enable offline inertia auto-tuning through the keypad.

y In the parameter display mode, switch to F30.10 and press the SET key to enable offline inertia auto- tuning. For details about offline inertia auto-tuning, see section 7.2 "Inertia Auto-tuning".

11.3.10    Group F31

F31.00: Fault reset

y   Defines whether to enable fault reset.

| Setpoint | Function | Remarks |
| --- | --- | --- |
| 0 | No operation | - |
| 1 | Enable | When a No.1 or No.2 resettable fault occurs, you can enable the fault reset function in the non-operational state after rectifying the fault cause and stopping the keypad from displaying the fault. When a No.3 warning occurs, you can enable the fault reset function directly. |

y   For fault classification, see section 10.1.3 "List of faults and alarms".

y The fault reset function, once enabled, stops the keypad from displaying the fault only. It does not activate modifications made on parameters.

y This function is not applicable to non-resettable faults. Use this function with caution in cases where the fault causes are not rectified.

F31.01 Software reset

y   Defines whether to enable fault reset.

| Setpoint | Function | Remarks |
| --- | --- | --- |
| 0 | No operation | - |
| 1 | Enable | Programs in the drive are reset automatically (similar to the program reset upon power-on) after the software reset function is enabled, without the need for a power cycle. |

y Software reset conditions: The servo drive is disabled, and there is no non-resettable fault such as No.1 fault.

F31.10: Encoder data reset

y The absolute position saved by the encoder changes abruptly after multi-turn data reset. In this case, perform mechanical homing.

    - Group U40

U40.00: Speed reference

y   Indicates the present speed reference (accurate to 1 RPM) of the drive in the position and speed control

modes.

U40.01: Speed feedback

y   Indicates the actual motor speed after round-off, which is accurate to 1 rpm.

y   This parameter is a 32-bit integer, which is displayed as a decimal on the keypad.

U40.02: Actual torque reference

y Indicates the present torque reference (accurate to 0.1%). The value 100.0% corresponds to the rated torque of the motor.

U40.04: DI status

y   Indicates the level status of five DIs without filtering.

y   Upper LED segments ON: high level (indicated by "1")

Lower LED segments ON: low level (indicated by "0") In cases where DI1 is low level and DI2 to DI5 are high level, the corresponding binary value is 11110, and the value of U40.04 read in the software tool is 30.

y   The keypad displays as follows:

DI4    DI2

DI5    DI3   DI1

H H  H  H  L 1   1   1   1   0

H: High L: Low

U40.05: DO state

y   Indicates the level status of three DOs without filtering.

y   Upper LED segments ON: high level (indicated by "1")

Lower LED segments ON: low level (indicated by "0") In cases where DO1 is low level and DO2 to DO3 are high level, the corresponding binary value is 110, and the value of U40.05 read in the software tool is 6.

y   The keypad displays as follows:

DO2 DO3 DO1

H H  L 1   1  0

H: High L: Low

U40.06: Bus voltage

y   Indicates the DC bus voltage of the main circuit input voltage after rectification, which is accurate to 0.1 V.

U40.07: Average load ratio

y Indicates the percentage of the average load torque to the rated torque of the motor, which is accurate to 0.1%. The value 100.0% corresponds to the rated torque of the motor.

U40.08: Electrical angle

y   Indicates the present electrical angle of the motor, which is accurate to 0.1°.

y   The electrical angle variation range is ±360.0° when the motor rotates.

y If the motor has four pairs of poles, each revolution generates four rounds of angle changes from 0° to 359.9°.

y Similarly, if the motor has five pairs of poles, each revolution generates five rounds of angle changes from 0° to 359.9°.

U40.09: Mechanical angle

y Indicates present mechanical angle (encoder unit) of the motor. The value 0 indicates that the mechanical angle is 0°.

U40.0C: RMS value of phase current

y   Indicates the RMS value of the phase current of the servo motor, which is accurate to 0.1 A.

U40.10: Position deviation counter

y   Counts the position pulses fed back by the encoder in any control mode.

y   This parameter is a 32-bit integer, which is displayed as a decimal on the keypad.

U40.30: Heatsink temperature

y Indicates the temperature of the module inside the servo drive, which can be used as a reference for estimating the actual temperature of the servo drive.

    - Group 6000

603Fh: Fault code

y   When a fault described in the DSP402 profile occurs on the drive, 603Fh is as described in DSP402.

y When a fault specified by the user occurs on the servo drive, 603Fh is 0xFF00. The value of 603Fh is in hexadecimal.

y   In addition, the object dictionary 203Fh displays auxiliary bytes of fault code in hexadecimal.

y 203Fh is a UInt32 value, in which the high 16 bits indicate the internal fault code of the manufacturer, and the low 16 bits indicate the external fault code of the manufacturer.

605Ah: Quick stop option code

0: Coast to stop, keeping de-energized status

1: Ramp to stop as defined by 6084h/609Ah (HM), keeping de-energized status 2: Ramp to stop as defined by 6085h, keeping de-energized status

3: Stop at emergency stop torque, keeping de-energized status

5: Ramp to stop as defined by 6084h/609Ah (HM), keeping position lock status 6: Ramp to stop as defined by 6085h, keeping position lock status

7: Stop at emergency stop torque, keeping position lock status

605Ch: Stop mode at S-ON OFF

–4: Ramp to stop as defined by 6085h, keeping dynamic braking status

–3: Stop at zero speed, keeping dynamic braking status

–2: Ramp to stop as defined by 6084h/609Ah (HM), keeping dynamic braking status

–1: Dynamic braking stop, keeping dynamic braking status 0: Coast to stop, keeping de-energized status

1: Ramp to stop as defined by 6084h/609Ah (HM), keeping de-energized status

605Dh: Stop option code

1: Ramp to stop as defined by 6084h/609Ah (HM), keeping position lock status 2: Ramp to stop as defined by 6085h, keeping position lock status

3: Stop at emergency stop torque, keeping position lock status

605Eh: Stop mode at No. 2 fault

-5: Stop at zero speed, keeping dynamic braking status

–4: Stop at emergency stop torque, keeping dynamic braking status

-3: Ramp to stop as defined by 6085h, keeping dynamic braking status

–2: Ramp to stop as defined by 6084h/609Ah (HM), keeping dynamic braking status

–1: Dynamic braking stop, keeping dynamic braking status 0: Coast to stop, keeping de-energized status

1: Ramp to stop as defined by 6084h/609Ah (HM), keeping de-energized status 2: Ramp to stop as defined by 6085h, keeping de-energized status

3: Stop at emergency stop torque, keeping de-energized status

4: Dynamic braking stop, keeping de-energized status

6060h: Modes of operation

Setpoint:

1: Profile position (PP) mode 3: Profile velocity (PV) mode 4: Profile torque (PT) mode 6: Homing mode (HM)

8: Cyclic synchronous position (CSP) mode 9: Cyclic synchronous velocity (CSV) mode 10: Cyclic synchronous torque (CST) mode Others: N/A

Description:

y   If an unsupported operation mode is selected through an SDO, an SDO error will be returned.

y If an unsupported operation mode is selected through a PDO, the change of the operation mode will be invalid.

6061h: Modes of operation display

1: PP mode

3: PV mode

4: PT mode

6: HM

8: CSP mode

9: CSV mode

10: CST mode

6064h: Position actual value

y Position actual value in user-defined unit (6064h) x Gear ratio (6091h) = Position actual value in encoder unit (6063h)

6065h: Following error window

y When the difference value between position reference (6062h) and position actual value (6064h) keeps exceeding ±6065h after the time defined by 6066h elapses, Er47.0 (Position deviation too large) occurs.

6066h: Following error time out

y   Defines the time lapse to trigger excessive position deviation, used with 6065h.

6067h: Max. profile velocity

y   Defines the threshold for position reach.

y If the difference between the position reference value (6062h) and the position actual value (6064h) is within ±6067h and the time reaches 6068h, the position is reached. In this case, bit 10 of 6041h is set to 1 in PP mode.

y    This flag bit is meaningful only when the S-ON signal is active in PP mode.

6068h: Position window time

y   Defines the window time for position reach, which must be used together with 6067h.

606Dh: Velocity window

y   Defines the threshold for speed reach.

y If the difference value between the target speed (60FFh) and the actual speed (606Ch) is within ±606Dh and the time reaches 606Eh, the speed is reached and bit 10 of the status word 6041h is set to 1 in the PV mode.

y    This flag bit is meaningful only when the S-ON signal is active in PV mode.

606Fh: Velocity threshold

y   Defines the threshold for determining whether the user velocity is 0.

y When the velocity actual value (606Ch) is within ±606Fh and the time reaches the value set by 6070h, the user velocity is 0. When either condition is not met, the user velocity is not 0.

y    This flag bit is valid only in PV mode.

y   It is not related to the S-ON state.

6070h: Velocity threshold time

y   Defines the time window for determining whether the user velocity is 0, which must be used together

with 606Fh.

6071h: Target torque

y   Defines the target torque of the servo drive in PT mode.

y   The value 1000 corresponds to the rated torque of the motor.

6072h: Max. torque

y   Defines the maximum torque reference limit.

y   The value 1000 corresponds to the rated torque of the motor.

6074h: Torque reference value

y   Defines the target torque value.

y   The value 1000 corresponds to the rated torque of the motor.

6077h: Torque actual value

y   Indicates the internal torque feedback of the servo drive.

y   The value 1000 corresponds to the rated torque of the motor.

607Ah: Target position

y   Defines the target position of the servo drive in PP mode.

y When bit 6 of 6040h is set to 0, 607Ah indicates the absolute target position of current segment. After positioning of the current segment is done, the value of 6064h will be the same as the value of 607Ah.

y   When bit 6 of 6040h is set to 1, 607Ah indicates the target incremental displacement of the current

segment. After positioning of current segment is done, the incremental displacement will be the same as the value of 607Ah.

607Ch: Home offset

y Defines the physical location of mechanical zero that deviates from the home of the motor in position control modes (profile position mode, interpolation mode, and homing mode).

y The home offset in active under the following conditions: The device is powered on, the homing operation is complete, and bit 15 of 6041h is set to 1.

y   After homing is done, the position actual value (6064h) will be the same as the value of 607Ch.

y   If 607Ch is set to a value outside 607Dh (Software position limit), Er84.3 (Home setting error) will occur.

607D.01h: Minimum software position limit

y   Defines the minimum software position limit relative to the mechanical zero.

y   Minimum software position limit = (607D.01h)

y The software position limit is used to judge the absolute position. When homing is not performed, the internal software position limit is inactivated.

607D.02h: Maximum software position limit

y   Defines the maximum software position limit relative to the mechanical zero.

y   Maximum software position limit = (607D.02h)

607Eh: Polarity

y   Defines the polarity of position or speed references.

y When bit 7 is 1, it indicates the position reference is multiplied by "–1" and the motor direction is reversed in the standard position mode or interpolation mode.

y   When bit 6 is 1, it indicates the speed reference (60FFh) is multiplied by "–1" and the motor direction is

reversed in the speed mode.

y When bit 5 is 1, it indicates the torque reference (6071h) is multiplied by "–1" and the motor direction is reversed in the torque mode.

y    Other bits are meaningless.

607Fh: Max. profile velocity

y   Defines the maximum operating speed in user-defined unit.

6081h: Profile velocity

y   Defines the constant operating speed of the target position in PP mode

y   The setpoint takes effect after the slave receives the displacement reference.

6083h: Profile acceleration

y   Defines the acceleration rate in the acceleration stage of the displacement reference in PP mode.

y The following formula applies if a motor equipped with 17-bit encoder needs to run at 400 RPM (6081h: 400 x 131072/60) with acceleration rate being 400 RPM/s (6083h: 400 x 131072/60) and deceleration

rate being 200 RPM/s (6084h: 200 x 131072/60) under a gear ratio of 1:1: Acceleration time tup = Δ6081h/Δ6083h = 1 (s).

Deceleration time tdown = Δ6081h/Δ6084h=2 (s).

y   The setpoint 0 will be forcibly changed to 1.

6084h: Profile deceleration

y   Defines the deceleration rate in the deceleration stage of the displacement reference in PP mode.

y The following formula applies if a motor equipped with 17-bit encoder needs to run at 400 RPM (6081h: 400 x 131072/60) with acceleration rate being 400 RPM/s (6083h: 400 x 131072/60) and deceleration rate being 200 RPM/s (6084h: 200 x 131072/60) under a gear ratio of 1:1:

Acceleration time tup = Δ6081h/Δ6083h = 1 (s). Deceleration time tdown = Δ6081h/Δ6084h=2 (s).

y   The setpoint 0 will be forcibly changed to 1.

6085h: Quick stop deceleration

y Defines the deceleration rate when the quick stop command (6040h set to 0x0002) is active and 605Ah (Quick stop option code) is set to 2 or 5.

y   The setpoint 0 will be forcibly changed to 1.

6087h: Torque slope

y  Defines the acceleration rate (torque reference increment per second) of the torque reference in PT

mode.

y In PT mode, if 605Ah is set to 1, 2, 5, or 6, or 605Dh is set to 1 or 2, the servo drive decelerates to stop as defined by 6087h.

y   If the setpoint exceeds the torque reference limit, the limit value will be used.

y   The setpoint 0 will be forcibly changed to 1.

- h: Torque slope

y   Defines the numerator of the gear ratio.

y The gear ratio is used to establish the proportional relationship between the load shaft displacement designated by the user and the motor shaft displacement.

y The relationship between motor position feedback (encoder unit) and load shaft position feedback (reference unit) is as follows.

Motor position feedback = Load shaft position feedback x Gear ratio

The relationship between the motor speed (rpm) and the load shaft speed (reference unit/s) is as follows.

Motor speed (rpm) = Load shaft speed x 6091h x 60/Encoder resolutions

y   The relationship between the motor acceleration (rpm/ms) and the load shaft acceleration (reference unit/

s2) is as follows.

Motor acceleration (rpm/ms) = Load shaft acceleration x 6091h x 1000/Encoder resolutions/60

- h: Shaft revolutions

y   Defines the denominator of the gear ratio.

6098h: Homing method

y    For details, see Table 4-1 "Mode lists".

- h: Speed during search for switch

y Defines the speed during search for the deceleration point signal. A large setpoint helps prevent homing timeout.

- h: Speed during search for zero

y Defines the speed in searching for the home signal. Setting this speed to a low value prevents overshoot during stop at high speed, avoiding excessive deviation between the stop position and the set mechanical home.

609Ah: Homing acceleration

y   Defines the acceleration rate in homing mode.

60B8h: Touch Probe Function

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Touch probe 1 function selection 0: Disabled 1: Enabled | bit0 to bit5: Probe 1 related settings When DI is used as the probe trigger signal, the DI source cannot be changed after the probe is enabled. For an absolute encoder, the Z signal refers to the zero point of motor single- turn position feedback. |
| 1 | Touch probe 1 trigger mode 0: Single trigger mode (Latches the position at the first trigger event.) 1: Continuous trigger |  |
| 2 | Touch probe 1 trigger signal selection 0: DI signal 1: Z signal |  |
| 3 | Reserved |  |
| 4 | Touch probe 1 positive edge 0: Switch off latching at positive edge 1: Enable latching at positive edge |  |
| 5 | Touch probe 1 negative edge 0: Switch off latching at negative edge 1: Enable latching at negative edge |  |
| 6 to 7 | Reserved |  |

| Bit | Name | Description |
| --- | --- | --- |
| 8 | Touch probe 2 function selection 0: Disabled 1: Enabled | bit8 to bit13: Probe 2 related settings |
| 9 | Touch probe 2 trigger mode 0: Single trigger mode (Latches the position at the first trigger event.) 1: Continuous trigger |  |
| 10 | Touch probe 2 trigger signal selection 0: DI signal 1: Z signal |  |
| 11 | Reserved |  |
| 12 | Touch probe 2 positive edge 0: Switch off latching at positive edge 1: Enable latching at positive edge | bit8 to bit13: Probe 2 related settings |
| 13 | Touch probe 2 negative edge 0: Switch off latching at negative edge 1: Enable latching at negative edge |  |
| 14 to 15 | Reserved |  |

60BAh: Touch probe 1 positive edge

y   Indicates the position feedback value (reference unit) latched at positive edge of touch probe 1 signal.

60BBh: Touch probe 1 negative edge

y   Indicates the position feedback value (reference unit) latched at negative edge of touch probe 1 signal.

60BCh: Touch probe 2 positive edge

y   Indicates the position feedback value (reference unit) latched at positive edge of touch probe 2 signal.

60BDh: Touch probe 2 negative edge

y   Indicates the position feedback value (reference unit) latched at negative edge of touch probe 2 signal.

60C5h: Max. acceleration

y   Defines the maximum permissible deceleration in PP mode, PV mode, and homing mode.

y   The setpoint 0 will be forcibly changed to 1.

60C6h: Max. deceleration

y   Defines the maximum permissible deceleration in PP mode, PV mode, and homing mode.

y   The setpoint 0 will be forcibly changed to 1.

60D5h: Touch probe 1 positive edge counter

y   The counting value is added by "1" each time this object is triggered.

60D6h: Touch probe 1 negative edge counter

y   The counting value is added by "1" each time this object is triggered.

60D7h: Touch probe 2 positive edge counter

y   The counting value is added by "2" each time this object is triggered.

60D8h: Touch probe 2 negative edge counter

y   The counting value is added by "2" each time this object is triggered.

60E0h: Positive torque limit

y   Defines the maximum torque limit of the servo drive in the forward direction.

60E1h: Negative torque limit

y   Defines the maximum torque limit of the servo drive in the reverse direction.

60E3.01h: 1st supported homing method

y Bit 0 to bit 7: The low 8 bits indicate the supported homing method. Set 6098h to the corresponding value.

y   Bit 8: Relative position homing

0: Not supported

1: Supported

y    Bit 9: Absolute position homing

0: Not supported

1: Supported

y   Bit 10 to bit 15: N/A

60E6h: Actual position calculation method

y Defines the method for calculating the mechanical position after homing is completed. After homing is triggered, changes in 60E6h will be blocked.

60F4h: Position deviation

y   This object indicates the position deviation (in reference unit).

60FCh: Position reference value

y   Indicates the position reference (encoder unit).

y If no warning is detected when the S-ON signal is active, the relationship between the position reference in reference unit and that in encoder unit is as follows:

60FCh (in encoder unit) = 6062h (in reference unit) x 6091h

60FDh: DI status

y   Indicates current DI logic of the drive. 0: Inactive; 1: Active

| Bit | Description |
| --- | --- |
| 0 | Reverse overtravel active |
| 1 | Forward overtravel active |
| 2 | Home signal active |
| 3 to 15 | N/A |
| 16 | DI1 input active |
| 17 | DI2 input active |
| 18 | DI3 input active |
| 19 | DI4 input active |
| 20 | DI5 input active |
| 21 to 26 | NA |

60FFh: Target velocity

y   Defines the target velocity in CSV and PV mode.

6502h: Supported drive modes

y   Defines the target velocity in CSV and PV mode.

| Brake | A: No brake B: brake |
| --- | --- |

| voltage level | 2: 220VAC 3: 380VAC |
| --- | --- |

| Inertial | H: high inertial M: neutral inertial L: low inertial |
| --- | --- |

| Power class | 200: 200W 400: 400W 750: 750W 1000: 1000W ..... |
| --- | --- |

| Motor Size | M40: 40mm End Flange M60: 60mm End Flange M80: 80mm End Flange ...... |
| --- | --- |

| Product series | A6 series |
| --- | --- |

Chapter 12	Motor and Options

12.1 Model

| Number of encoder turns | S for single turn, empty for multiple turns |
| --- | --- |
| Resolution | 17: 17-bit 23: 23-bit |
| Encoder type | E : Magnetic Encoders M: optical encoder |

| Plug Type | 1: Straight Plugs 2: Plastic Plugs 3: Aviation Plugs |
| --- | --- |

A6M60 - 400 H2 A1-M17(S)

12.2 Nameplate

Nameplate

Model

Motor parameters

Manufacturing SN

A6M60-400H2A1-M17

220V	250Hz	1.10A

400W     0.32Nm   Ins:F

rated/max 3000/6000 rpm

S/N:0100000000000000

www.omc-stepperonline.com.

Figure 12-1    Nameplate of the A6 servo motor

12.3 Components

Quality connector

Quality motor bearing

Wear-resistant oil sealing

Shaft extension protection

Figure 12-2    Components of the A6 servo motor

12.4  Terminal Definition

NOTICE

y The figure is for reference only. For details about the motor correspondence and size information, refer to the supporting relationships and drawings. Pay attention to the mirror relationship between the motor side and the cable side.

| Cable Type | Terminal Layout (Cable Side) | Pin No. | Usage |
| --- | --- | --- | --- |
| Power input connector | 1 2 3 A B 4 | 1 | Phase V |
|  |  | 2 | Phase U |
|  |  | 3 | Phase W |
|  |  | 4 | Grounding cable |
|  |  | A | Brake (polarity insensi-tive) |
|  |  | B | Brake (polarity insensi-tive) |
|  | - C - D | A | Phase U |
|  |  | B | Phase V |
|  |  | C | Phase W |
|  |  | D | Grounding cable |
|  | 1 - D 2 - C | A | Phase U |
|  |  | B | Phase V |
|  |  | C | Phase W |
|  |  | D | Grounding cable |
|  |  | 1 | Brake (polarity insensi-tive) |
|  |  | 2 | Brake (polarity insensi-tive) |
| Encoder connector | 5 1 6 2 7 3 4 | 1 | DATA+ |
|  |  | 2 | DATA- |
|  |  | 3 | BAT+ |
|  |  | 4 | BAT- |
|  |  | 5 | +5V |
|  |  | 6 | 0V |
|  |  | 7 | Enclosure |
|  | 1 2 3 4 5 6 7 8 9 10 | 1 | DATA+ |
|  |  | 2 | DATA- |
|  |  | 4 | +5V |
|  |  | 5 | BAT- |
|  |  | 6 | BAT+ |
|  |  | 9 | 0V |
|  |  | 10 | Enclosure |

12.5 General Specifications

    - Mechanical Characteristics

| Item |  | Description |
| --- | --- | --- |
| Duty |  | S1 (Continuous) |
| Vibration class[1] |  | V15 |
| Insulation resistance |  | 500 V DC, above 10 MΩ |
| Excitation mode |  | Permanent magnetic |
| Mounting mode |  | Flange |
| Thermal class |  | Level F |
| Insulation voltage |  | 1500 V AC, 1 minute (220 V level) 1800 V AC, 1 minute (380 V level) |
| IP rating of the enclosure |  | IP67 with an oil seal (oil seal installed on the axis side) |
| Forward direction of rotation |  | The servo drive rotates counterclockwise (CCW) as observed from the axis side under the forward rotation command. |
| Ambient conditions | Ambient temperature | (Non-freezing) (Derate based on the derating curve for temperatures above 40°C.) |
|  | Ambient humidity | 20% to 80% (Non-condensing) |
|  | Installation site | y Free from corrosive or explosive gases y Well ventilated with minimum amount of dust, waste, and moisture y Convenient for inspection and cleaning y Derating required only for altitudes above 1000 m y Away from sources that may generate strong magnetic field y Away from heating sources such as a heating stove y Use a motor with oil seal in places with grinding fluid, oil mist, iron powders or cuttings. |
|  | Storage environment | Observe the following requirements for storage of a de- energized motor: y Storage temperature: (Non-freezing) y Storage humidity: (Non-condensing) |
| Shock resistance[2] | Shock acceleration rate at flange | 490 m/s² |
|  | Number of shocks | 2 |
| Vibration resistance[3] | Vibration acceleration rate at flange | 49 m/s² |

CAUTION

y [1] The vibration class V15 indicates that the vibration amplitude is less than 15 μm when a servo motor rotates at its rated speed. y [2] The value in the table indicates the resistance against shock in the vertical direction with the servo motor shaft mounted horizontally. y [3] The value in the table indicates the resistance against vibration in the vertical, left-right, and forward-backward directions with the servo motor shaft mounted horizontally. y The strength of the vibration that the servo motor can withstand depends on the application. Check the vibration acceleration rate applied to the servo motor through the actual product. Vertical Forward﹣backward Left﹣right Figure 12-3 Directions of short and vibration applied to the servo motor

    - Overload Characteristics

The motor is compliant with NEC and CEC requirements and equipped with protective functions against overload and over-temperature.

To protect different load motors, set the motor overload protection gain based on the overload capacity of the motor. Use the default gain in general conditions. However, when one of the following conditions occurs, change the gain based on actual motor temperature:

y  The motor operates in environments with high temperature.

y The motor is in cyclic motion featuring a short motion cycle and frequent acceleration/ deceleration.

250

200

150

100		 50		 0

120   130   140   150   160   170   180   190   200   210   220   230   240   250   300   350        Load ratio (%)

Run time (s)

See the following inverse time lag curve for motor overload protection.

Figure 12-4    Motor overload protection curve

    - Load moment of inertia

The load moment of inertia represents the inertia of the load. Larger load moment of inertia means slower response, which may result in unstable motion. The allowable load moment of inertia of the motor is subject to a limit. The limit varies with the driving conditions of the servo motor.

An overvoltage warning may occur during deceleration if the load moment of inertia exceeds the allowable value. The servo drive with a built-in braking resistor may generate an overload warning. In case of such warnings, take one of the following measures:

y  Reduce the torque limit value. y   Reduce the deceleration rate. y   Reduce the maximum speed.

y   Install an external braking resistor if the warning cannot be cleared using the above measures.

NOTICE

y Servo motors with the capacity below 400 W do not have built-in braking resistors. y When a built-in braking resistor is used, some energy generated under certain regenerative driving conditions still exceeds the allowable capacity loss (W) of the built-in braking resistor. In this case, an external braking resistor is required.

30

25

20

15

10

5

0

1000

2000

3000

4000

5000

6000

7000

Speed (rpm)

Ratio of allowable load moment of inertia (times)

The following figure shows the relationship between the ratio of allowable load moment of inertia and the speed when a servo is used without a built-in regenerative resistor or an external braking resistor. (The following figure shows the reference values upon deceleration at 200 VAC input and torques greater than the rated torque.)

Figure 12-5    Ratio of allowable load moment of inertia for the speed

When the servo unit is used by load whose moment of inertia exceeds the allowable value, an overvoltage alarm may be triggered.

12.6 Selection Precautions

y   Motors with oil seals must be derated by 10% during use.

y Do not share the power supply of the brake with other electrical devices. Failure to comply may result in malfunction of the brake due to voltage or current drop caused by other devices.

y   Use cables with a cross-sectional area above 0.5 mm2.

y All parameters and torque-speed characteristic values are subject to the conditions that the motor works with a servo drive and the armature coil temperature is 20°C.

y  The torque for fastening the terminal screws must be 0.19 N·m to 0.21 N·m. Excessive torque may

damage the screws.

y   Radial and axial loads of the motor

LF

Radial load

Axial load

Figure 12-6    Radial and axial loads of the motor

16.8

12.7 Technical Specifications

12.7.1   Model of 3000 rpm

100 W (40 frame)

| Item (A6M40--XXXXXXXXXX) | A6M40-100L2B1-M17 (With brake) | A6M40-100L2A1-M17 (Without brake) |
| --- | --- | --- |
| Rated power (W) | 100 |  |
| Rated current (A) | 1.1 |  |
| Maximum current (A) | 3.9 |  |
| Rated torque (N·m) | 0.32 |  |
| Maximum torque (N·m) | 1.12 |  |
| Rotor inertia (10-4·kg·m²) | 0.033 | 0.03 |
| Rated speed (rpm) | 3000 |  |
| Maximum speed (rpm) | 6000 |  |
| Rated voltage (V) | 220 |  |

Ø0.04

A

100L2B1-M17 (With brake)

0.04   A

5.5

Ø8 h6 ﹣0.009

0

↗ 0.02

2﹣Ø4.3

3 h9

Ø46

M3   7

A

3

3

40

100.1

14

20

25

Ø30 h7 ﹣0.021

0

6.2

100L2A1-M17 (Without brake)

0.04   A

5.5

Ø8 h6 ﹣0.009

↗ 0.02

0

2﹣Ø4.3

3  h9

Ø46

M3  7

A

3

3	 14

20

77.4	 	25

Ø30 h7 ﹣0.021

0

6.2

Ø0.04  A

40

16.8

Product Dimensions (unit: mm)

400 W (60 frame)

| Item (A6M60-XXXXXXXXXX) | 400H2B1-M17(With brake) | 400H2A1-M17(Without brake) |
| --- | --- | --- |
| Rated power (W) | 400 |  |
| Rated current (A) | 2.51 |  |
| Maximum current (A) | 8.78 |  |
| Rated torque (N·m) | 1.27 |  |
| Maximum torque (N·m) | 4.45 |  |
| Rotor inertia (10-4·kg·m²) | 0.60 | 0.59 |
| Overload multiplier | 3.5 |  |
| Rated speed (rpm) | 3000 |  |
| Maximum speed (rpm) | 6000 |  |
| Rated voltage (V) | 220 |  |

400H2B1-M17 (With brake)

0.04  A

15.8

6.5

Ø14 h6 -0.011

3       ӭ 0.02

0

4-Ø5.5

5 h9

Ø70

M5  10

A

14

Ø50 h7-0.025

0

5

Ø0.04  A

11	 25.5

60		 	117.2	 30

400H2A1-M17(Without brake)

Ø14  h6 -0.011

0

15.8	0.04   A

6.5

ӭ 0.02

3

4-Ø5.5

5 h9

Ø70

M5  10

A     14

Ø50  h7 -0.025

0

5	 25.5

11

Ø0.04   A

60		 	88	 30

Product Dimensions (unit: mm)

750 W (80 frame)

| Item (A6M80-XXXXXXXXXX) | 750H2B1-M17(With brake) | 750H2A1-M17(Without brake) |
| --- | --- | --- |
| Rated power (W) | 750 |  |
| Rated current (A) | 4.60 |  |
| Maximum current (A) | 16.30 |  |
| Rated torque (N·m) | 2.39 |  |
| Maximum torque (N·m) | 8.36 |  |
| Rotor inertia (10-4·kg·m²) | 1.77 | 1.72 |
| Overload multiplier | 3.5 |  |
| Rated speed (rpm) | 3000 |  |
| Maximum speed (rpm) | 6000 |  |
| Rated voltage (V) | 220 |  |

0.04

A

ӭ

0.02

A 22

Ø0.04

A

750H2B1-M17 (With brake)

15.5

0.04    A

8

Ø19 h6 -

0

0.01

3

3

ӭ

0.02

4-Ø6.4

6 h9

Ø90

M6  12

A

22

6

15.5

30.2

35

Ø70 h7

0

-0.030

80

128.8

Ø0.04   A

750H2A1-M17 (Without brake)

Ø19  h6 -0.013

0

15.5

8

3

44--ØØ66..44

6 h9

Ø90

M6  12

6

Ø70  h7 -0.030

0

15.5

80

30.2

94.6	 35

Product Dimensions (unit: mm)

1 kW (80 frame)

| Item (A6M80-XXXXXXXXXX) | 1000H2B1-M17(With brake) | 1000H2A1-M17(Without brake) |
| --- | --- | --- |
| Rated power (W) | 1000 |  |
| Rated current (A) | 6.3 |  |
| Maximum current (A) | 20.9 |  |
| Rated torque (N·m) | 3.18 |  |
| Maximum torque (N·m) | 11.13 |  |
| Rotor inertia (10-4·kg·m²) | 2.28 | 2.23 |
| Overload multiplier | 3.5 |  |
| Rated speed (rpm) | 3000 |  |
| Maximum speed (rpm) | 6000 |  |
| Rated voltage (V) | 220 |  |

15.5

1000H2B1-M17 (With brake)

15.5

0.04   A

8

Ø19 h6

0

-0.013

3

↗

0.02

4-Ø6.4

6 h9

Ø90

M6  12

A

22

6

30.2

35

Ø70 h7

0

-0.030

80

142.1

Ø0.04  A

1000H2A1-M17(Without brake)

0.04     A

15.5

8

Ø19   h6 ﹣0.013

↗   0.02

0

3

4﹣Ø6.4

6 h9

Ø90

M6  12

A

22

6

Ø70   h7 ﹣0.030

0

15.5

80

107.9

30.2

35

Ø0.04   A

Product Dimensions (unit: mm)

12.8 Motor torque-speed characteristics

Model of 3000 rpm

7000

6000

Speed (rpm)

5000

4000

3000

2000

1000

0

A6M40-100L2***

A

B

0.2   0.4   0.6   0.8   1.0  1.2

Torque (N·m)

7000

6000

Speed (rpm)

5000

4000

3000

2000

1000

0

A6M60-400H2***

A

B

1.2   2.4   3.6   4.8   6.0  7.2

Torque (N·m)

7000

6000

Speed (rpm)

5000

4000

3000

2000

1000

0

A6M80-750H2***

A

B

2.5   5.0   7.5   10  12.5  15

Torque (N·m)

6000

5000

Speed (rpm)

4000

3000

2000

1000

0

A6M80-1000H2***

A

B

2.5     5     7.5   10  12.5   15

Torque (N·m)

- Con�nuous work area

- Short-�me work area

Chapter 13	Peripheries

13.1 List of Peripheries

| Component Name | Installation Location | Applicable Model | Description |
| --- | --- | --- | --- |
| Fuse and circuit breaker | Input side of the drive | All | To comply with EN 61800-5-1 and UL 61800-5-1 standards, install a fuse/circuit breaker on the input side of the servo drive to prevent accidents caused by short circuit in the internal circuit. |
| AC input reactor | Input side of the drive | All | Eliminate harmonics and improves the power factor on the input side. |
| EMC filter | Input side of the drive | All | Reduce the conducted and radiated interference that escapes from the servo drive to the outside. |
| Magnetic ring | Output side of the drive | All | Reduce interference to the outside and the bearing current. |
|  | Signal cable | All | Improve the anti-interference performance of signals. |

13.2 Fuse

To prevent accidents caused by short circuit, install a fuse on the input side of the drive.

| Drive Model |  | Rated Input Current | Recommended Fuse |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Manufacturer | Rated Current | Model |
| Single-phase 220 V |  |  |  |  |  |
| SIZE A | A6-200EC | 2.3A | Bussmann | 5A | FWP-5B |
|  | A6-400EC | 4 A | Bussmann | 10 A | FWP-10B |
| SIZE B | A6-750EC | 7.9 A | Bussmann | 20 A | FWP-20B |
| SIZE C | A6-1000EC | 9.6 A | Bussmann | 20 A | FWP-20B |
| SIZE D | A6-1500EC | 12.8 A | Bussmann | 35 A | FWP-35B |
| Three-phase 220 V |  |  |  |  |  |
| SIZE C | A6-1000EC | 5.1 A | Bussmann | 15 A | FWP-15B |

| Drive Model |  | Rated Input Current | Recommended Fuse |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Manufacturer | Rated Current | Model |
| Three-phase 380 V |  |  |  |  |  |
| SIZE D | A6-2000EC | 5.6 A | Bussmann | 15 A | FWP-15B |
| SIZE D | A6-3000EC | 8 A | Bussmann | 20 A | FWP-20B |

13.3 Electromagnetic Contactor

| Drive Model |  | Rated Input Current | Recommended Contactor |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Manufacturer | Rated Current | Model |
| Single-phase 220 V |  |  |  |  |  |
| SIZE A | A6-200EC | 2.3A | Schneider | 9 A | LC1 D09 |
|  | A6-400EC | 4 A | Schneider | 9 A | LC1 D09 |
| SIZE B | A6-750EC | 7.9 A | Schneider | 9 A | LC1 D09 |
| SIZE C | A6-1000EC | 9.6 A | Schneider | 12 A | LC1 D12 |
| SIZE D | A6-1500EC | 12.8 A | Schneider | 18 A | LC1 D18 |
| Three-phase 220 V |  |  |  |  |  |
| SIZE C | A6-1000EC | 5.1 A | Schneider | 9 A | LC1 D09 |
| SIZE D | A6-1500EC | 8 A | Schneider | 9 A | LC1 D09 |
| Three-phase 380 V |  |  |  |  |  |
| SIZE D | A6-2000EC | 5.6 A | Schneider | 9 A | LC1 D09 |
| SIZE D | A6-3000EC | 8 A | Schneider | 9 A | LC1 D09 |

13.4 Circuit Breaker

| Drive Model |  | Rated Input Current | Recommended Contactor |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Manufacturer | Rated Current | Model |
| Single-phase 220 V |  |  |  |  |  |
| SIZE A | A6-200EC | 2.3A | Schneider | 4A | OSMC32N2D4 |
|  | A6-400EC | 4 A | Schneider | 6 A | OSMC32N2D6 |
| SIZE B | A6-750EC | 7.9 A | Schneider | 16 A | OSMC32N2D16 |
| SIZE C | A6-1000EC | 9.6 A | Schneider | 16 A | OSMC32N2D16 |
| SIZE C | A6-1500EC | 12.8 A | Schneider | 20 A | OSMC32N2D20 |
| Three-phase 220 V |  |  |  |  |  |
| SIZE C | A6-1000EC | 5.1 A | Schneider | 10 A | OSMC32N3D10 |
| SIZE D | A6-1500EC | 8 A | Schneider | 16 A | OSMC32N3D16 |
| Three-phase 380 V |  |  |  |  |  |
| SIZE D | A6-2000EC | 5.6 A | Schneider | 10 A | OSMC32N3D10 |
| SIZE D | A6-3000EC | 8 A | Schneider | 16 A | OSMC32N3D16 |

13.5 Absolute Encoder Battery

Select an appropriate battery according to the following table.

| Battery Spec. | Item and Unit | Ratings |  |  | Requirements |
| --- | --- | --- | --- | --- | --- |
|  |  | Min. | Typical | Max. |  |
| Output Specifications: 3.6V 2500mAh | External battery volt-age (V) | 3.2 | 3.6 | 5 | In standby state[1] |
|  | Circuit fault voltage (V) | - | 2.6 | - | In standby state |
|  | Battery alarm voltage (V) | 2.85 | 3 | 3.15 | - |
|  | Current consumed by the circuit (μA) | - | 2 | - | In normal operation[2] |
|  |  | - | 10 | - | In standby state, shaft at a standstill |
|  |  | - | 80 | - | In standby state, shaft rotating |
|  | Ambient temperature during battery opera-tion (ºC) | 0 | - | 40 | Same as the motor |
|  | Battery storage tem-perature (ºC) | -20 | - | 60 |  |

NOTICE

y The preceding data is measured at an ambient temperature of 20ºC. y [1] The "standby state" means that the encoder performs multi-turn counting by using the power from an external battery when the servo drive power supply is off. In this case, data transmission stops. y [2] The "normal operation" means that the absolute encoder supports single-turn or multi-turn data counting and transceiving. Power on the servo drive after connecting the absolute encoder properly. The encoder starts data transmission after a short delay of about 5s after power-on. The motor speed must be lower than or equal to 10 rpm during transition from the standby state to the normal operation state (upon power-on). Otherwise, E740 (Encoder fault) may be reported. In this case, power off and power on the servo drive again. y Theoretically, a battery can be used for two years, but different working conditions and environments can lead to significant differences.

Chapter 14	Maintenance

14.1 Daily Maintenance

Standard operating conditions:

y   30°C (annual average ambient temperature)

y   Average load rate < 80%

y   Daily operation time < 20 hours

Due to the influence of ambient temperature, humidity, dust, and vibration, the internal components of the device may age and be damaged, causing faults or reducing the service life of the device. Therefore, to ensure the normal function of the device and prevent damage, refer to the following items for daily inspection and cleaning.

Check

y   The ambient temperature and humidity are normal. There is no dust or unwanted objects in the

servo drive.

y   There is no abnormal vibration or noise for the device.

y   The voltage of the power supply is normal.

y   There is no strange smell.

y   There are no fibers adhered to the air inlet.

y   There is no intrusion of unwanted objects on the load end.

Cleaning

y   Clear the dust, especially metallic dust, on the drive surface to prevent the dust from entering the

drive.

y   Keep the drive in a well ventilated environment.

y   Keep the front end of the servo drive and the connectors clean.

CAUTION

y Disconnect the power supply before cleaning. Use a blower gun or dry cloth to clean the equipment. y Do not use gasoline, diluent, alcohol, acidic and alkaline detergents, to prevent discoloration or damage to the enclosure.

14.2 Periodic Maintenance

The electrical and electronic parts inside the servo drive may be mechanically worn out and degraded. Perform periodic maintenance according to the following table.

Contact us or the agent before replacement to double check whether the part needs to be replaced.

| Object | Type | Standard Replacement Interval |
| --- | --- | --- |
| Drive | Bus filter capacitor | About 5 years |
|  | Cooling fan | 2 to 3 years (10,000 h to 30,000 h) |
|  | Aluminum electrolytic capacitor of the circuit board | About 5 years |
|  | Pre-charge relay | 100,000 operations (depending on the operating conditions) |
|  | Pre-charge resistor | 20,000 operations (depending on the operating conditions) |
| Motor | Bearing | 3 to 5 years (20,000 h to 30,000 h) |
|  | Oil Seal | 5000 hours |
|  | Encoder | 3 to 5 years (20,000 h to 30,000 h) |
|  | Absolute encoder battery | Depends on the operating conditions. See the operation instructions for the encoder battery for details. |

14.3 Part Replacement

14.3.1 Plain Key Replacement

CAUTION

y Observe the uninstallation requirements described in this chapter. Failure to comply may result in equipment fault or damage. y Disassembly by force is strictly prohibited to prevent injury to hands from bumps and knocks.

Currently, the plain keys for ASMK2 standard motors with 60/80/130 frames have been unified to B-type plain keys, and come with key extraction holes. The specifications for the key extraction screws are as listed in the following table.

| Motor Mode | Flat Key Size | Screw Specifications (Hexagon Screw Recommended) |
| --- | --- | --- |
| 40 frame | A-type plain key - A3 x 3 x 14 | Without key extraction holes |
| 60 frame | B-type plain key - B5 x 5 x 16.5 | M3x10 and above |

| Motor Mode | Flat Key Size | Screw Specifications (Hexagon Screw Recommended) |
| --- | --- | --- |
| 80 frame | B-type plain key - B6 x 6 x 25 | M3x15 and above |
| 100 frame | C-type plain key - C8 x 7 x 35 | M3x20 and above |
| 130 frame | B-type plain key - B8 x 7 x 35 | M3x20 and above |
| 180 frame | C-type plain key - C10 x 8 x 64 | M3x20 and above |

Tools: Internal hexagonal wrench Disassembly procedure:

① Prepare key extraction screws (hexagon bolts as recommended) of the corresponding specifications according to the motor models.

② Use an Allen key to turn the screws clockwise until the A-A end of the plain key completely disengages from the keyway. This will allow you to remove the plain key.

Key extraction hole

Figure 14-1    Plain key disassembly

15.3.2 Oil Seal Replacement

Tools: Needle-nose pliers, non-slip gloves, and cotton cloth Disassembly procedure:

① Put a cotton cloth at support point B to protect the end bracket against scratch during disassembly.

② Fix the motor and prop the oil seal outer lip at point A with one end of the needle-nose pliers.

③ Pry out the oil seal slowly at point B.

A

B

Figure 14-2    Oil seal disassembly

Troubleshooting for Common

EMC Issues

Chapter 15

15.1 RCD Malfunction

If a residual current device (RCD) is needed, select the RCD according to the following requirements:

y   Use a B-type RCD because the drive may generate DC leakage current in the protective conductor.

y For each drive, use an RCD whose tripping current is not lower than 100 mA to prevent RCD malfunction due to high-frequency leakage current generated by the drive.

y When multiple drives are connected in parallel and share one RCD, select an RCD whose tripping current is not lower than 300 mA.

y   Use Chint or Schneider RCDs (recommended).

When an RCD malfunctions, perform troubleshooting according to the following table.

| Fault | Possible Cause | Solution |
| --- | --- | --- |
| Tripping upon power-on | The anti-interference performance of the RCD is poor. | y Use an RCD from a recommended brand. y Use an RCD with a high tripping current. y Move the unbalanced load to the front end of the earth leakage circuit breaker. |
|  | The rated tripping current of the RCD is too low. |  |
|  | An unbalanced load is connected to the rear end of the RCD. |  |
|  | The ground capacitance at the front end of the drive is high. |  |
| Tripping during operation | The anti-interference performance of the RCD is poor. | y Use an RCD from a recommended brand. y Use an RCD with a high tripping current. y Install a simple filter on the input side of the drive, and wind the LN/RST cable on a magnetic ring near the RCD. y Reduce the carrier frequency without compromising the performance. y Use a shorter motor cable. |
|  | The rated tripping current of the RCD is too low. |  |
|  | An unbalanced load is connected to the rear end of the RCD. |  |
|  | The ground distributed capacitance of the motor cable and the motor is too high. |  |

15.2 Harmonic Suppression

To suppress the harmonic current of the servo drive and improve the power factor, install an AC input reactor on the input side of the servo drive to meet standard requirements.

The following figure shows the reactor mounting mode.

Input reactor

EMC filter

Drive

Power supply

U1

V1 W1

U2

V2 W2

L1

L2 L3

PE

PE

L1’

L2’

L3’ PE

Place the metal enclosure on the grounding backplate.

R

S T

PE

U

V W

PE

Place the metal

Motor

Place the metal enclosure on the grounding backplate.

enclosure on the grounding backplate.

Connect the grounding copper

busbar to the backplate.

Figure 15-1    Reactor mounting mode

15.3 Control Circuit Interference

    - Common I/O Signal Interference

The drive generates very strong interference during operation. In the case of improper routing or grounding, the drive may interfere with or be interfered with by other devices. In such a case, take the following measures.

y   Use shielded cables as the I/O signal cables and connect the shield to the PE terminal.

y Reliably connect the PE terminal of the motor to the PE terminal of the servo drive, and connect the PE terminal of the servo drive to the PE terminal of the grid.

y   Add an equipotential bonding grounding wire between the host controller and the drive.

y Add a magnetic ring for the U/V/W output cable of the drive. Wind the cable on the ring for two to four turns.

y   Increase the filter capacitance at low-speed DI terminals. A maximum of 0.1 μF capacitance is

recommended.

y   Increase the filter capacitance between AI and GND terminals. A maximum of 0.22 μF capacitance

is recommended.

y Add a magnetic buckle or magnetic ring for the signal cable. Wind the signal cable on the buckle or ring for one or two turns.

y   Use shielded cables as power cables and ground the shield securely.

    - EtherCAT Communication Interference

In such a case, take the following measures.

y Check that the communication network cables meet the specification requirements for Cat5e shielded cables.

y   Check that the communication port is secure and in good contact.

y   Separate communication cables from power cables by a distance of at least 30 cm.

y   Add equipotential grounding cables between nodes in the case of multi-node communication.

y   Check that any cable between two nodes is within 100 m in length.

y Add a magnetic buckle at each end of the communication cable. Wind the communication cable on the buckle for one or two turns.

y Add a magnetic ring for the U/V/W output cable of the drive. Wind the cable on the ring for two to four turns.

y   Use shielded cables as power cables and ground the shield securely.

Certification and Standard ▕

Certification and Standard

Chapter 16

16.1 CE Certification

| Directives | Standards |
| --- | --- |
| EMC Directive 2014/30/EU | EN 61800-3 |
| Low Voltage Directive 2014/35/EU | EN 61800-5-1 |
|  | EN 60034 |
| RoHS Directive 2011/65/EU | EN 50581 |